3GPP TSG-RAN WG2 Meeting #129	R2-2500323

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.1.5

Source:	Mediatek Inc.

Title:	Feasibility Analysis on RAN1 Identified Solution for Two-sided Model

Document for:	Discussion, Decision

Introduction

The RAN1 LS (R1-2410922)[1] requests RAN2 to evaluate the feasibility of standardized signaling for the RAN1 identified solution.

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

In this contribution, we identify potential alternatives for NW-side sharing model parameters and/or datasets to the UE/UE-side from RAN2 perspective. We discuss the necessity and possibility of using a common framework and tunnels for both one-sided and two-sided models for the purposes of data transfer and sharing model parameters/datasets. We then analyze the feasibility of these alternatives within the scope of RAN2 expertise.

Discussion

Alternatives from RAN2 Perspective

RAN1 has been discussing following directions, options, suboptions to alleviate/resolve issues related to inter-vendor training collaboration, i.e. on-device operation and UE side offline engineering:

- Direction A: Sharing parameters/reference model/dataset that enables UE-side offline engineering (Inter vendor collaboration option 3a/5a/4) - Direction B: Sharing NW side encoder parameter to UE side for UE side inference directly with on-device operation (Inter vendor collaboration option 3b) - Direction C: Fully standardized reference model(s) and parameters with specified CSI generation part and/or CSI reconstruction part (Inter vendor collaboration option 1)

According to the LS, RAN1 requests RAN2’s feedback on the feasibility of standardized signaling, via over-the-air and/or other approaches, for dataset and/or parameter exchange in Option 4-1 and Option 3a-1, which are purely for Direction A.

Among the various Options and Sub-Options for Direction A, after down-selection, RAN1 is considering the following Sub-Options for further study with potential further down-selection. - Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively - Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder - Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}

Observation 1: RAN1 requests RAN2 to provide feedback on the different options under Direction A, i.e., sharing parameters/reference models/datasets that enable UE-side offline engineering.

To enable UE-side offline engineering, Direction A requires following procedure according to RAN1 discussion.

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

First of all, it should be clarified what "UE-side" exactly means in the context of the two-sided model discussion. Similar to UE-side data collection, the UE-side can refer to a UE-side server for model training within the MNO’s network, or a UE-side OTT server outside of the MNO’s network. Whether the UE-side server is only within the MNO’s network or not can be decided later by SA WGs. The term "NW-side" refers to NG-RAN or a network entity within the CN/OAM domain.

Proposal 1: RAN2 assumes that "UE-side" refers to either a UE-side server for model training within the MNO’s network or a UE-side OTT server outside of the MNO’s network for the two-sided model discussion.

Proposal 2: RAN2 assumes that ‘NW-side’ refers to either NG-RAN or a network entity within the CN/OAM domain.

According to the description of the procedures, there are generally two alternativse for the NW-side to share model parameters and/or dataset, which may require standardization, just as illustrated in Fig 1:

- Alternative 1: NG-RAN->UE: NG-RAN node shares the model parameters and/or dataset to the UE via air interface;

- Alternative 2: NW-side->UE-side server: NG-RAN, or a network entity in CN/OAM domain shares the model parameters and/or dataset to the UE-side server via other approach.

| Alternative 1: NG-RAN->UE | Alternative 2: NW-side->UE-side server |
| --- | --- |

Figure 1 Alternatives for NW-side to share model parameters and/or dataset

In Alternative 1, it is possible for NG-RAN or CN to share the parameters and/or dataset with the UE over the air interface, for example, via RRC/NAS messages. However, considering that the use case of CSI compression is a RAN use case, it is preferred that the LCM procedures are only performed between the UE and NG-RAN.

For instance, performance monitoring to detect mismatches between NW-side data distribution and UE-side inference data distribution may trigger the gNB to share the model parameters and/or datasets with the UE over the air interface for offline training. There is no strong reason for the CN to share this information via NAS messages.

Observation 2: In Alternative 1, considering that the use case of CSI compression is a RAN use case, it is more efficient for the LCM procedures to be performed solely between the UE and NG-RAN.

Therefore, in Alternative 1, it is assumed that NG-RAN shares the model parameters and/or dataset with the UE, which then forwards the received data to the UE-side OTT server via the application layer. The transfer of model parameters and/or datasets from the UE to the UE-side OTT server is outside the scope of 3GPP.

For Alternative 2, the NW-side can be NG-RAN or a network entity within the CN/OAM domain. It is assumed that NG-RAN or the network entity can transfer the model parameters and/or dataset directly to the UE-side server for model training, as it is within the MNO’s network. The NW-side can also send the model parameter and/or dataset to the UE-side OTT server, which is out of 3GPP scope.

The feasibility study on Alternative 2 is not within RAN2 expertise scope, and should be evaluated by SA2 and SA5 depending whether the UE-side server is within the CN or OAM domain. Therefore, RAN2 should focus on the evaluation on Alternative 1.

Observation 3: Similar to UE-side data collection, the feasibility study on Alternative 2 is not within the expertise scope of RAN2. It should be evaluated by SA2 and SA5, depending on whether the UE-side server falls within the CN or OAM domain.

Proposal 3: RAN2 focus to evaluate the feasibility of Alternative 1, i.e. NG-RAN node shares the model parameters and/or dataset to the UE via air interface and leave other approaches to other SA WGs, e.g. SA2/SA5.

Feasibility from RAN2 Perspective

RAN1 provided the following dataset size information for different options:

RAN2 may assume the following for the contents for the feasibility discussion. For Option 4-1: sharing {target CSI, CSI feedback} dataset: - Data size for target CSI depends on the format. There is no agreement on the format or necessary precision of the target CSI. Some examples based on companies’ evaluations are: eType-II format (up to ~1000 bits), eType-II-like format (~ a few 1000 bits), and float32 format (up to ~ 150K bits). The data size may also vary depending on the configuration, and the captured value indicates the order of magnitude of the typical data size per sample as a guideline. - There is no agreement on the CSI feedback size. Values in the order of eType II payload size may be assumed (up to ~ 1000 bits) for RAN2 discussion. - There is no agreement on the total number of samples N1 in the dataset. The range of the number of samples N1 used in RAN1 evaluations is 40K-1200K. As a representative number, RAN2 may assume that the total number of samples is around N1=600K. - Summing up the above, and assuming eType-II-like format of a size 2000 bits, the total dataset size for Option 4-1 may be roughly in the order of 600K * (2000 bits + 1000 bits) / (8bits/Byte) = 225 MB. The size may be smaller for eType-II format. The size may be larger if we consider the dataset consisting of supporting multiple configurations. For Option 3a-1 without target CSI: sharing encoder parameter: - The number of parameters in the encoder models used in RAN1 evaluations have a wide range, ranging from 36K parameters to 13M parameters, with the median value of 5.8M. - Based on the above range, the size of encoder parameters may range from 36K*(8/8)=36KB based on the smallest model size in RAN1 evaluations and the use of float8, to 13M*(32/8)=52M based on the largest model size in RAN1 evaluations and the use of float32. Based on the median size, RAN2 may assume the size of 5.8M*(16/8) = 11.6 MB based on the medium model size and the use of float16. For Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset: - The size of the encoder parameters is the same as in the Option 3a-1 without target CSI. The size of the dataset is smaller than the Option 4-1, because the dataset only contains the target CSI but not the CSI feedback. The number of samples N2 in the dataset may be the same or smaller than N1. The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB.

Based on the information provided by RAN1, following requirements needs to be satisfied to exchange the AI model parameters and/or dataset between the the NW-side to the UE-side:

- The overall data size is not limited;

- Data content of AI model parameters and/or dataset should be visible to both UE-side and NW-side.

For inter-vendor training collaboration, it is essential that the data content of AI model parameters and datasets is visible to both the UE-side and NW-side. This visibility ensures that the data can be correctly interpreted and used by all parties involved. It allows models trained by different vendors to be effectively integrated and used together, promoting interoperability and reducing the risk of vendor lock-in.

Proposal 4: Following requirements needs to be satisfied to exchange the AI model parameters and/or dataset between the NW-side to UE-side:

- The overall data size is not limited;

- Data content should be visible to both UE-side the NW-side.

According to RAN1 LS, the dataset size can be as large as hundreds of megabytes, making it infeasible to transmit the dataset through current control plane signaling, such as RRC messages or NAS messages in SRBs. For example, considering the dataset size of option 4-1, which is 225MB, it is 25600 times larger than 9kB, the maximum RRC message size.

Transmitting the dataset in tens of thousands of RRC segments is not feasible due to the resulting complexity. Additionally, this approach would lead to significant latency in completing the dataset sharing, potentially delaying or interrupting the transmission of user data and slowing down control plane (e.g. RRC) procedures. Additionally, the overhead associated with managing such a large number of segments would place a substantial burden on both the network and the UE, increasing the risk of errors and retransmissions. This could further exacerbate latency issues and degrade overall network performance.

Observation 4: The size of the AI model parameters and dataset will be in the order of megabytes. Currently, control plane signaling, including RRC and NAS messages, is not feasible for carrying the AI model parameters and dataset, even if the number of RRC segments is increased.

Proposal 5: RAN2 reply to RAN1 that it is not feasible to use SRB to exchange the AI model parameters and/or dataset between NW-side and the UE over the air interface.

If we consider the DRB, the current DRB is also not feasible for sharing AI model parameters and/or datasets from the NW-side to the UE. This is because the DRB is terminated between the UE and the UPF, and the corresponding PDU session provides the end-to-end data path between the UE and the DN. As a result, the data content is not visible to the NW-side.

Terminating a DRB at the NG-RAN instead of the UPF would represent a significant architectural shift from the current system. Such a deviation could lead to interoperability issues, challenges with standard compliance, and potential difficulties in multi-vendor environments.

Observation 5: DRBs are terminated between the UE and UPF and the corresponding PDU sessions provide the end-to-end data path between the UE and the DN. The data content is not visible to the NW-side.

Proposal 6: RAN2 reply to RAN1 that it is not feasible to use DRB to exchange the AI model parameters and/or dataset between NW-side and the UE over the air interface.

Common Framework/Tunnel for UE-sided and Two-sided

The discussion on one-sided and two-sided models has progressed separately in both RAN1 and RAN2, with RAN2 not having advanced the two-sided model discussion for a long time. It is suggested that UE-sided and two-sided models should be considered together when discussing the system framework and tunnels to enable data collection and model transfer/delivery, as these are common issues for both cases.

It is preferred to have a common design for the framework and tunnel that can be used for both UE-sided and two-sided models to reduce complexity in both implementation and standardization. It should be noted that we are discussing potential standardized methods, and the framework for data collection and model delivery between the UE and the OTT server is not considered here.

Taking the UE-sided model in Figure 2a as an example, a tunnel going through the UE, RAN, OAM, and the UE-side server can be established between the UE and the UE-side server for both data collection for model training and model transfer. This is a bi-directional tunnel, where data collection is the upstream (in orange) and model transfer is the downstream (in purple).

Comparing the two-sided model in Figure 2b with the UE-sided model in Figure 2a, the tunnel for model parameter/dataset sharing and model transfer is quite similar. The NG-RAN can transfer the model parameter/dataset to the UE-side server e.g., via OAM for offline model training for the UE-side of the two-sided model if NW-first training is considered. Since the data used for NW-first training is collected from the UE, the mechanism for NW-side data collection over the air interface is applied. The UE-side of the two-sided model can be transferred from the UE-side server to the UE e.g., via OAM and RAN node. This is a bi-directional tunnel, where model parameter and dataset transfer is the upstream (in orange) and model transfer is the downstream (in purple).

It should be noted that Figures 2a) and 2b) use UE-side data collection option 3 (where the UE collects training data and transfers it to OAM) as an example. These figures are also applicable to option 2 (where the UE collects training data and transfers it to the Core Network) if OAM is replaced with CN.

Obervation 6: A common design for the framework and tunnel can be considered for both UE-sided and two-sided models to reduce complexity in both implementation and standardization, streamlining data collection and model transfer processes.

| Fig 2a) UE-sided model: Data collection (for model training) and model transfer (for model inference) | Fig 2b) Two-sided model: Model parameter/dataset sharing (for model traiing) and model transfer (for model inference) |
| --- | --- |

Proposal 7: A common bi-directional tunnel can be used for both the UE-sided model and the two-sided model:

- UE-sided model: Data collection for model training is the upstream (from the UE to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

- Two-sided model with NW-first training: Model parameter and dataset transfer for UE-sided model training is the upstream (from NW-side to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

In order to make the AI model parameter and/or dataset exchange over air interface feasible, one possible method is to introduce a specific radio bearer dedicated to model parameter and dataset transfer, which would be capable of handling large amount of data and be terminated between the UE and NG-RAN. For UE-sided model, the specific radio bearer can also be used for data collection from the UE to NG-RAN. It has the merit of SRB that the data content is visible to NG-RAN and the merit of DRB that the data amount is not restricted. Since it is terminated at NG-RAN, NG-RAN determines when to establish the radio bearer for model parameter and/or dataset transfer. Another benefit is that that such specific bearer can also be used for NW-side data collection.

In order to make the data content visible, it is for sure that the file format needs to be specified. Current ASN.1 can be used to define the data content. Other file formats are not excluded.

Observation 7: A specific radio bearer dedicated to model parameter and data transfer, terminated between the UE and NG-RAN, combines the advantages of SRB (data content visibility to NG-RAN) and DRB (unrestricted data amount). Additionally, it can be used for NW-side data collection.

Proposal 8: RAN2 consider to introduce a new type of bearer terminated between the UE and NG-RAN for transferring both model parameter and dataset for both one-sided and two-sided model. A file format is required to be specified.

Conclusion

Alternatives from RAN2 perspective:

Observation 1: RAN1 requests RAN2 to provide feedback on the different options under Direction A, i.e., sharing parameters/reference models/datasets that enable UE-side offline engineering.

Observation 2: In Alternative 1, considering that the use case of CSI compression is a RAN use case, it is more efficient for the LCM procedures to be performed solely between the UE and NG-RAN.

Observation 3: Similar to UE-side data collection, the feasibility study on Alternative 2 is not within the expertise scope of RAN2. It should be evaluated by SA2 and SA5, depending on whether the UE-side server falls within the CN or OAM domain.

Proposal 1: RAN2 assumes that "UE-side" refers to either a UE-side server for model training within the MNO’s network or a UE-side OTT server outside of the MNO’s network for the two-sided model discussion.

Proposal 2: RAN2 assumes that ‘NW-side’ refers to either NG-RAN or a network entity within the CN/OAM domain.

Proposal 3: RAN2 focus to evaluate the feasibility of Alternative 1, i.e. NG-RAN node shares the model parameters and/or dataset to the UE via air interface and leave other approaches to other SA WGs, e.g. SA2/SA5.

Feasibility from RAN2 perspective:

Observation 4: The size of the AI model parameters and dataset will be in the order of megabytes. Currently, control plane signaling, including RRC and NAS messages, is not feasible for carrying the AI model parameters and dataset, even if the number of RRC segments is increased.

Observation 5: DRBs are terminated between the UE and UPF and the corresponding PDU sessions provide the end-to-end data path between the UE and the DN. The data content is not visible to the NW-side.

Proposal 4: Following requirements needs to be satisfied to exchange the AI model parameters and/or dataset between the NW-side to UE-side:

- The overall data size is not limited;

- Data content should be visible to both UE-side the NW-side.

Proposal 5: RAN2 reply to RAN1 that it is not feasible to use SRB to exchange the AI model parameters and/or dataset between NW-side and the UE over the air interface.

Proposal 6: RAN2 reply to RAN1 that it is not feasible to use DRB to exchange the AI model parameters and/or dataset between NW-side and the UE over the air interface.

Common Framework/Tunnel for UE-sided and Two-sided

Obervation 6: A common design for the framework and tunnel can be considered for both UE-sided and two-sided models to reduce complexity in both implementation and standardization, streamlining data collection and model transfer processes.

Observation 7: A specific radio bearer dedicated to model parameter and data transfer, terminated between the UE and NG-RAN, combines the advantages of SRB (data content visibility to NG-RAN) and DRB (unrestricted data amount). Additionally, it can be used for NW-side data collection.

Proposal 7: A common bi-directional tunnel can be used for both the UE-sided model and the two-sided model:

- UE-sided model: Data collection for model training is the upstream (from the UE to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

- Two-sided model with NW-first training: Model parameter and dataset transfer for UE-sided model training is the upstream (from NW-side to the UE-side server), and model transfer for model inference is the downstream (from the UE-side server to the UE).

Proposal 8: RAN2 consider to introduce a new type of bearer terminated between the UE and NG-RAN for transferring both model parameter and dataset for both one-sided and two-sided model. A file format is required to be specified.

Reference

[1] R1-2410922, LS on signalling feasibility of dataset and parameter sharing, RAN1

[2] 3GPP TR 38.843 V18.0.0