3GPP RAN2-RAN#129 Meeting			R2-2500325

Athens, GREECE, February 17th – 21st, 2025

Agenda Item:	6.2

Source:	IIT Kharagpur

Title:  Data distribution and HARQ management for multihop-parallel relay topology in  5G NR

Document for:	Discussion

# 1   Introduction

The present contribution defines the methodology to establish RRC connection in Multi-hop parallel relay configuration to facilitate vehicle-to-everything (V2X) services network as specified in 3GPP TS 23.287 [3] for:

a) V2X communication among the UEs over the PC5 interface; and

b) V2X communication between the UE and the gNB over the Uu interface.

Fifth generation new radio, or 5G-NR, is a recent development in cellular technology. In order to connect everyone and everything, including machines, objects, and gadgets, 5G enable a new core and access network. 5G wireless technology aims to provide more users with faster multi-Gbps peak data rates, extremely low latency, enhanced reliability, vast network capacity, and a more consistent user experience. New user experiences are facilitated through enhanced performance and efficiency, which further connects new industries. The motto of 5G NR is that everything is connected everywhere. However, it may happen because user equipment (UE) may be in the outage. A UE may be outside of the coverage area of a gNB. In this context, relays will play a pivotal role in maintaining connectivity, however, the 5G-NR standard talks about only single-hop or multi-hop relays. Multi-hop parallel relays are not part of the standard as of now. In this work, we propose a multi-hop parallel relay network for better connectivity between a source UE and a destination UE or gNB. HARQ, which stands for Hybrid Automatic Repeat Request, is a crucial feature in 5G New Radio (NR) that enhances the reliability of data transmission in wireless communication systems. HARQ is an essential component of 5G NR, ensuring that data transmission in wireless networks can achieve high reliability and low latency, critical for applications like autonomous vehicles, industrial automation, and more. It helps maintain a balance between high data rates and robust error correction to provide a better user experience in 5G networks.

# 2   Discussion

Key Proposal:

- Our main proposal is establishing an error-free data transmission mechanism for a multihop-parallel relay network.

- A novel overall flow sequence for the relay network using a muttihop-parallel relay has been proposed in the 5G NR network to achieve seamless end-to-end connectivity at high speed.

- The multihop-parallel relay network may be established between a remote UE and a destination UE or a gNB. The relays will work in decode forward fashion.

- We propose multiple parameters like signal-to-noise ratio (SNR), load factor, and priority factor-based data distribution and HARQ management.

- A novel HARQ management scheme is proposed for error-free data transmission based on the factors mentioned in 4.

- A novel buffer management scheme in HARQ technique.

Application:

- This work may propose a change in the mechanism of BW pooling in a 5G-NR standard.

- This work will increase the TV viewing quality under the stringent channel and busy network condition through making co-located UEs act as parallel relays.

- This work may lead to additional applications/network level protocol changes for the NR sidelink V2X communications.

- The proposed scheme is useful for other mission critical services.

A schematic diagram of the proposed scheme is depicted in Fig. 1 and Fig. 2. In Fig 1 a remote UE, 101, is connected to the gNB, 110, through L hops. Lth hop and gNB are connected through the Uu link, 109. 101 is connected to 1st hop, 103, through PC-5 link, 102. A hop, 103 may contain many relay devices like 104. 1st hop, 103 is connected to 2nd hop, 106 through PC-5 links 105. Similarly, Lth hop 108 also receives PC-5 links, 107 from the previous hop.

In Fig 2 a multihop-parallel network is suggested between a remote UE, 101 and a destination UE, 202. The destination UE, 202, is connected to Lth hop, 108, through PC-5 links, 201.

Fig 1: Schematic diagram for multihop-parallel relay between a remote UE and gNB

Fig 2: Schematic diagram for multihop-parallel relay between a remote UE and a destination UE

Fig 3: Detailed diagram for establishing a relay network between a remote UE and a gNB

Fig 4: Detailed diagram for establishing a relay network between a remote UE and a destination UE

A detailed diagram of the proposed scheme is depicted in Fig. 3 and Fig. 4. In Fig2 a remote UE, 101 is connected to the gNB, 110, through L hops. The connection between L th hop UEs such as 319,320, and 321and gNB, 110 is through Uu link, 322, 323, and 324. 101 is connected to 1 st hop, UEs such as 305, 306, and 307 through PC-5 links, 302, 303, and 304. A relay device R 11 of 1 st hop, is connected to a relay device R 21 through PC-5 link 309. Similarly, at L th hop, relay devices 319, 320, 1nd 321 also receive PC-5 links, through 316, 317, and 318 from previous hop.

In Fig 4 a multihop-parallel network is suggested between a remote UE, 101 and a destination UE, 202. The destination UE, 202 is connected to L th hop relay devices 319, 320, and 321 through PC-5 links 401, 402, and 403.

Brief Description: A remote UE may find a gNB or the destination UE through L hops (typically L ≥ 1). At lth hop, we propose to limit the connection to the maximum ND number of relay devices. The ith device at the lth hop may be connected to the multiple devices at (l+1)th hop through different radio links. Similarly, multiple devices of (l-1)th hop may be connected to the ith device at lth hop through separate radio links. A relay network between a source UE and a destination UE or gNB is established at the RRC level, and every device in the relay network has a relay chain ID. A novel mechanism for data distribution and HARQ management is proposed in this document. Data transmission between one relay device at lth layer and a relay device (l+1)th layer is done through three factors: i. SNR, ii. Overload factor, and iii. Priority factor of the destination relay. A packet at RLC is subdivided if SNR between links is not greater than SNReff, the receiving relay device is overloaded, or the priority factor relay device. If all the factors, namely, SNR, overloading factor, and priority, are satisfied, then the packet is sent to the device without subdivision. At the first hop, the data is checked to see if there is any decode error. After the first hop, the data is sent to Lf forward hops according to the novel data transmission mechanisms, i.e. SNR, load factor, and priority factor. The data is checked for decode error after Lf hops, and if any error is found, the correcting the error Lb backwards hops can be backtracked. If the correct data is not found even after going Lb hops backwards, the data is retransmitted from the source. In the HARQ management scheme, a novel technique of buffer management is proposed. If the link between two relay devices and the connecting device satisfies SNR, load factor, and priority criterion, the buffer is discarded. If any one of these conditions is not satisfied, the erroneous packet is kept in the buffer and combined with the retransmitted packet to eradicate decode error.

Detailed Description: The detailed description of the steps followed are described in next figures.

Fig 5: Algorithm #1: Steps for data distribution from one relay device to another relay devices in two consecutive hops in multihop-parallel relay network

Fig. 5 shows the steps for data distribution from one relay device to another relay device in two consecutive hops in a multihop-parallel network. At step 501, gNB does CSI acquisition from nearby relay UE devices for making RRC connection. In the SDAP layer, which packet will be UE according to SNR, load factor, and priority factor at step 502 has been decided. At step 503, it has been checked whether SNR is greater than SNReff. The connecting relay device is overloaded or not is checked at step 505. The priority of the connecting relay device is checked at step 507. If SNR is not greater than SNReff or, the connecting relay device is overloaded, or the relay device does not give the highest priority to the incoming data. In that case, the packet is further subdivided at the RLC layer for that particular relay device at steps 504, 506, and 509. If all criterion of SNR, load factor and priority are met, then the packet is not divided further before being sent to the relay UE device at step 508. The packet, either divided or not divided, is sent to relay devices at step 510.

Fig 6-A: Steps for HARQ management in multihop-parallel relay network

Steps for HARQ management and data distribution is given in Fig. 6-A, Fig. 6-B, Fig. 6-C, and Fig. 6-D. There are maximum L hops in the relay network. At lth hop in relay network there can be maximum

Fig 6-B: Steps for HARQ management in multihop-parallel relay network

Dl relay devices. The data transmission takes place Lf hop in the forward direction and Lb hop can be gone backwards before retransmission from the source if an error exists at any hop. A particular relay device at any hop can be connected to maximum D devices in the next hop. At step 601, the counter for hop count is initialized. It has been checked whether the maximum number of hops was reached or not in step 602. If the maximum number of hops is reached, the process terminates in step 603. At step 605, it has been checked whether it is the first hop or not. If it is not, first hop, then go to step 620 through step 621. If it is the first hop at step 605, then the devices in the first hop are checked in step 608. At step 611, it has been checked whether there is a decode error or not for ith device at first hop. If there is no decode error at step 611, increment the device count at step 613 through step 612. At step 614, it has been checked whether the maximum device count for the first hop is reached or not. If the maximum device count is not reached, go back to step 608 through step 609. If there is any decoding error, NACK is sent to the transmitting device and the HARQ count is incremented at step 617. If the maximum HARQ count reaches the process, end at step 618. If the maximum HARQ count is not reached at step 617, the source retransmits the data at step 619 and then goes to step 608 through step 610. At step 614, if the maximum device count is reached, the hop count is incremented at step 615. At step 620, it has been checked whether the sum of the current hop and the number of forwarding hops is greater than the maximum hop count or not. If the sum exceeds the maximum hop count, the data is sent from the current hop to the last hop following algorithm 1 at step 624 through step 622. At step 623, if the sum is less than the maximum hop count, the data is transmitted to lth hop to (l+Lf) th hop following algorithm 1.

Fig 6-C: Steps for HARQ management in multihop-parallel relay network

For checking the decoding error at step 631, the counter for backwards direction is initialized at step 625. Whether the maximum numbers of allowed backward steps are reached or not is checked at step 626. If the maximum number of backward steps is reached and the decode error exists, retransmission from the source node is initiated at step 628. If a maximum number of backward hops is not reached, the device counter for a hop is initialized at step 629.

A particular device is checked for decode error at step 630. Is there any decoding error that exists or not checked at step 631. If there is no decoding error, the device count is incremented at step 632. At step 633, it has been checked whether the maximum device count is reached or not. If the maximum device count is not reached, the process returns to step 630. If maximum device count is reached at step 633,

Fig 6-D: Steps for HARQ management in multihop-parallel relay network

hop count is incremented, and the process returns to step 602 through step 604. If a device at lth hop is detected with a decode error, it is checked whether the error is from multiple devices from (l-1)th hop in step 635. The number of erroneous connections is calculated at step 636. The information of decoding error either from single or multiple devices from the preceding hop is transferred to the next step through step 637. The NACK is sent to devices of the preceding hop with erroneous data, and the HARQ count is incremented at step 638. The maximum allowable HARQ count is checked at step 639. If the maximum HARQ count is reached, the process is terminated at step 640. If the maximum HARQ count is not reached, the counter for total erroneous connections is initialized at step 641. At step 642, it has checked whether that maximum count is reached or not. If the maximum count is reached, the process returns to step 631 through step 643.

If the maximum count for multiple erroneous connections is not reached at step 645, it has been checked whether the SNR of the connecting link is greater than the SNReff. If the SNR condition of the link is satisfied, it has been checked whether the connecting device is overloaded at step 647. If the device is not overloaded, at step 650, the priority factor of the connecting device is checked. If the condition of SNR, load factor, and priority are satisfied, the erroneous message kept at the buffer is discarded. If any of the conditions at steps 645, 647, and 650 are not satisfied, the erroneous data is kept in a buffer at steps 649 through step 648. At step 652, the erroneous data is kept in the buffer or discarded buffer data is transmitted to the next step. The data is retransmitted at step 653. The counter of the erroneous connections is incremented at step 654. The process returns to step 642 after increment of connection count. The counter for backward hops is incremented, and the present hop count is decremented at step 655. The process at step 655 returns to step 626 through step 627.

Fig 7-A: Steps for HARQ management in multihop-parallel relay network when at each hop a particular relay device is connected with only one relay device in preceding and succeeding hops

Steps for HARQ management in a multihop-parallel relay network, when at each hop a particular relay device is connected with only one relay device in preceding and succeeding hops, is depicted in Fig. 7-A and Fig. 7-B. The architecture is shown in Fig. 8 and Fig. 9. At step 701, the counter for hop count is initialized. It has been checked whether the maximum number of hops was reached in step 702.

If a maximum number of hops are reached, then the process terminates in step 703. The counter of maximum relay devices at a particular hop is initialized at step 704. At step 705, the check for decode error of a particular relay device at a particular hop is performed. At step 706, it has been checked whether there is a decode error or not for ith device at first hop. If there is no decode error at step 706, increment the device count at step 707. At step 708, it has been checked whether the maximum device count for the first hop is reached or not. If the maximum device count is not reached, go back to step 705. If there is any decoding error, NACK is sent to the transmitting device and the HARQ count is incremented at step 710. If the maximum HARQ count is reached at step 711, the process ends at step

712. If the maximum HARQ count is not reached at step 711, it has been checked whether the SNR of the connecting link is greater than the SNReff or not at step 713. If the SNR condition of the link is satisfied, it has been checked whether the connecting device is overloaded or not at step 715. If the device is not overloaded, at step 717, the priority factor of the connecting device is checked. If the condition of SNR, load factor, and priority are satisfied, the erroneous message kept at the buffer is discarded at step 719. If any of the conditions at steps 713, 715, and 717 is not satisfied, the erroneous data is kept in a buffer at steps 720 through steps 714, 716, and 718. At step 721, the erroneous data kept in the buffer or discarded buffer data is transmitted to the next step. The data is retransmitted at step 722. The process returns to step 705 after retransmission through step 723.

Fig 7-B: Steps for HARQ management in multihop-parallel relay network when at each hop a particular  relay device is connected with only one relay device in preceding and succeeding hops

Fig. 8: Detailed diagram for establishing a relay network between a remote UE and a gNB when at each hop a particular relay device is connected with only one relay device in preceding and succeeding hops

Fig. 9: Detailed diagram for establishing a relay network between a remote UE and a destination UE when at each hop a particular relay device is connected with only one relay device in preceding and succeeding hops

# 3     Results

Fig. 10 shows the relationship between SER of the parallel-multihop relay combination system and transmit power of UEs, i.e P. It can be observed that if P is increased the average SER performance of the overall system improves due to improvement of received SNR at each UE. Moreover, it can be found that if the number of hops, i.e L is increased the SER performance also improves. This occurs due to the fact that if L is increased, the distances between intermediate hops reduces, therefore the pathloss reduces, hence the received SNR at each UE gets improved. This leads to the improvement of average SER performance of the given system.

Fig 10: SER vs transmit power of UE-s, P (dBm)

Fig 11: SER vs N0

Fig. 11 flashes on the relationship between SER of the parallel-multihop relay combination system and noise power, i.e., N0. It can be observed that if N0 is reduced, SER performance improves due to the SNR improvement at each UE.

Fig 12: Throughput vs Power

Fig. 12 depicts the relationship between overall throughput (Td)  of the  HARQ enabled  parallel-multihop relay communication system and transmit power of an UE, i.e., P for different number of relays per hop. It can be noted that if P goes up, the overall throughput performance rises up due to the SNR improvement at each UE. Further, if the number of parallel relays per hop (Nd) is increased from one to two, the overall system throughput improves. The reason behind this is, if Nd is increased, it becomes possible transmit more data symbols through the relay paths simultaneously.

Simulation parameters are used to obtain the results are given below:

| Simulation parameter | Value |
| --- | --- |
| ND (number of parallel relays per hop) | 2 |
| Nc (cyclic prefix length) | 7 |
| α (Pathloss exponent) | 4 |
| Ns (number of sub-carriers) | 256 |
| dgu (distance between gNB and remote UE) | 24 m |

Table1: Key simulation parameters

# 4    Conclusion

Based on the discussions and the observations, we have the following proposals:

Proposal 1:	The multihop-parallel relay network may be established between a remote UE and a destination UE or a gNB. The relays will work in decode forward fashion. Novel data distribution mechanism between two relay devices in two consecutive hops based on three parameters i.e. SNR, load factor, and priority factor.

Proposal 2:   Novel HARQ management mechanism for the proposed multihop-parallel relay network based on SNR, load factor, and priority factor.

Proposal 3:   Novel buffer management scheme in HARQ mechanism. The erroneous packet is discarded when conditions of SNR, load factor and priority factor are met.

# 5    References

[1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 3GPP RP-193253 "New SID: Study on NR sidelink relay".

[3] 3GPP TS 23.303 "Proximity-based services (ProSe);Stage 2 ".

[4] RP-202208, Revised SID: Study on NR Sidelink Relay.