3GPP TSG-RAN WG2 Meeting #R2-129		R2-2500327 Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.3.4

Source: 	Rakuten

Title:  	AI-ML based RLF/HO failure prediction

Document for:	Discussion, Decision

# Introduction

The Rel 19 SI description for “AI/ML for Mobility” includes the following objective

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2]

The above highlighted objective includes prediction of Radio Link Failure (RLF) and Handover Failure (HoF). In this contribution, we discuss some aspects of RLF/HoF. We also propose an AI/ML based way forward for this issue.

# Discussion

## Definition of RLF and HoF

The following definitions could be attributed to RLF and HoF, though a more granular definition is possible based on the different stages of a Handover process.

RLF: Radio Link Failure is an event when the radio connection between the UE and its serving gNB is broken, for example due to T310 expiry and the UE cannot communicate with the network and vice versa. RLF may also occur in the case when the HO command cannot be delivered successfully to a UE undergoing mobility.

HoF: Handover Failure is an event when a UE, after the reception of HO command from the source gNB, fails to successfully connect to the target gNB, for example due to T304 expiry and therefore causing a failed handover.

It is also important to note that the primary focus here is how AI/ML models can predict RLF/HoF and not a detailed analysis of RLF/HoF definition.

## Causes of RLF and HoF

Based on the earlier work on Self-Organising Networks (SON), there are at least 3 types of HO failures as defined in TS38.300.

- Too late HO

  - In this case, the UE has stayed for a long period of time than desirable in the serving cell, resulting in an RLF; the UE attempts RRC re-establishment of the radio link connection in a different cell.

- Too early HO

  - In this case, an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts RRC re-establishment of the radio link connection in the source cell.

- HO to wrong cell

  - In this case, an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure; the UE attempts RRC re-establishment of the radio link connection in a cell other than the source cell and the target cell.

In too late HO, the RLF happens due to measurement reports arriving too late or a delayed HO decision at the source node. If the measurements are reported on time or the HO decision is made a little earlier, the HO might be successful.

In too early HO, the failure happens due to early measurement reporting or too early HO decision. If the measurement reporting or the HO decision is made a little later, the HO might be successful.

However, if drastic adjustments are made to too-early HO or too-late HO, it may cause too-late or too-early HO respectively. Therefore, a certain level of the HO decision accuracy is required to overcompensating and causing another type of failure.

Too-early HO and too-late HO are some of the major causes for RLF and HoF.

## Metrics to predict for RLF and HoF

The HO decision criteria accuracy, which is basically an indicative of the RRM criteria to trigger HO, for example RSRP values, could be different for different pairs of source and target cells. Therefore, in a network-based AI/ML model, determination of too-early HO and too-late HO between any two cells directly enables prediction of RLF/HoF.

If the RSRP values of source and target cells, as experienced by UEs, at various points of a handover are made aware to the gNB, the gNB may be able to determine and prevent too-early HO and too-late HO between any two pair of cells.

To start with, reception of HO command from the source cell, RACH attempt by the UE to the target cell, detection of RLF/HoF at the UE, transmission of RRC Reconfiguration Acknowledgement to the network, could be considered as the various checkpoints during which the source and target cell RSRP measurements could prove useful to the gNB.

By adjusting the HO decision criteria, the HO decision criteria accuracy could be improved and RLF/HoF could also be predicted using these metrics.

HO decision criteria accuracy i.e “sweet spot for handover” could be different for different pairs of source and target cells.

In a network-based AI/ML model, determination of too-early HO and too-late HO between any two pair of source and target cells enables prediction of RLF/HoF.

RSRP values of source and target cells, as experienced by UEs, at various points of a handover are reported as assistance information to the gNB.

Network side AI/ML model uses the assistance information reported by UEs to determine too-early HO and too-late HO between any two pair of source and target cells and predict RLF/HoF.

Too-early HO and too-late HO metrics are used to predict RLF and HoF in a network side model.

# Conclusion

We have the following observations:

Observation 1	Too-early HO and too-late HO are some of the major causes for RLF and HoF.

Observation 2	HO decision criteria accuracy i.e “sweet spot for handover” could be different for different pairs of source and target cells.

Observation 3	In a network-based AI/ML model, determination of too-early HO and too-late HO between any two pair of source and target cells enables prediction of RLF/HoF.

We have the following proposals:

Proposal 1	RSRP values of source and target cells, as experienced by UEs, at various points of a handover are reported as assistance information to the gNB.

Proposal 2	Network side AI/ML model uses the assistance information reported by UEs to determine too-early HO and too-late HO between any two pair of source and target cells and predict RLF/HoF.

Proposal 3	Too-early HO and too-late HO metrics are used to predict RLF and HoF in a network side model.

# References

Study Item Description: RP-240082 : AI/ML for Mobility;