3GPP TSG-RAN WG2 Meeting #R2-129		R2-2500328 Athens, Greece, Feb 17-21, 2025

Agenda Item:	8.6.3

Source: 	Rakuten

Title:  	Discussion on L1 event triggered measurement reporting

Document for:	Discussion, Decision

# Introduction

In the WID [1], the following objective is defined for inter-CU LTM:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

The WID objective includes Event based L1 measurements for LTM.

In this contribution, we discuss the feasibility of configuring Event based L1 measurements and using them for LTM candidate cell addition/release. Further, we discuss redundancy of L3 and L1 measurements for LTM.

# Discussion

## Open issues from RAN2 #127-bis and RAN2#128

## Measurement Report for reportOnLeave

In L3 measurement reporting, the reportOnLeave is used to indicate whether to initiate a measurement report when the leaving condition of the event is satisfied. It can help the NW to know the signal quality deterioration of the concerned beam/cell in time, so that the NW can take appropriate decisions.

A similar mechanism is also beneficial for the L1 measurement report for the LTM. This information can be used to decide to, e.g., cancel the cell switch or remove the LTM candidate cell.

The reportOnLeave is configurable for event-triggered L1 measurement report and configurable by the gNB.

## Scheduling of L1 measurement report MAC CE

The below agreement was made in RAN2 #126bis.

11.  The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

While it was agreed that legacy SR procedure for resource allocation is the baseline, requesting resources using BSR to send event-triggered L1 measurements MAC CE was not agreed. We believe there is a need to include some information of MR MAC CE size, otherwise if MR MAC CE size is variable and large, there is no way for the network to assign a grant to guarantee max size.

Requesting resource allocation using the legacy BSR procedure to send the event-triggered L1 measurements MAC CE is supported.

## LTM between cells transmitting different Reference Signals

The below agreements were made in RAN2#127bis and RAN2#128 respectively.

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). We will decide which option to go next meeting.

4.   Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

RAN2 has agreed to support the both SSB and CSI-RS reference signals for L1 measurement resource configuration in LTM configuration. Then, the question is how to guarantee the same type of RS between the current beam (of serving cell) and the new beam (of the candidate cells). There are the possibilities where the UE could be configured with different type of the RS between the current serving cell beam and the candidate cell beams because the indicated TCI state is dynamically changed.

In Rel-18 LTM, the existing solution is one where the UE relies on network configuration to configure the same type of RS. This has its own limitations as a serving gNB-DU cannot be aware of the RS transmitted by a candidate/target LTM cell.

Figure 1: Reference signals in cells.

In the figure above,

- The green cells can be considered to belong to one DU or one gNB. Let’s assume they are transmitting SSB reference signals.

- The red cells can be considered to belong to another DU or another gNB. Let’s assume they are transmitting CSI-RS reference signals.

- The two sets of cells could be operating at different frequencies.

- For a UE in one of the cells transmitting SSB RS, it could become inevitable to perform an inter-frequency LTM cell switch and move to CSI-RS based cells.

- The current solution in RAN2 doesn’t address this use case.

This could be easily addressed by requesting transmission of both reference signals for a predefined duration, by the candidate cell in such scenarios.

RAN2 discusses and agrees a way forward to address the issue where LTM is configured between two cells transmitting different reference signals.

## gNB-DU initiated LTM candidate cell addition/release based on L1 measurement event

Following agreements were made in R2#126 regarding Measurement enhancements for LTM.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In Release 18 LTM, RAN2 has agreed that the candidate cell preparation is performed by the gNB-CU based on the L3 measurements, while the LTM cell switch execution is performed by the gNB-DU based on the periodic L1 measurements.

In Release 18 LTM, the L1 measurements was possible to be configured only for LTM candidate cells and was only periodic in nature. With the introduction of event triggered L1 measurements, if the cells are configured based on neighbour cell relations and not measurements, newly discovered cells are also feasible to be reported. For example, if a newly discovered cell (configured based on neighbour cell relations) meets the configured event criteria, it could be reported using L1 measurements. Hence, a new LTM candidate cell addition could be triggered based on these L1 measurements.

A maximum of 8 LTM candidate/target cells are feasible to be configured per UE and it is important that the candidate configuration slot isn’t wasted due to obvious reasons. It creates resource crunch at the network side and holding on to an unwanted candidate cell may deprive the UE of a valuable next candidate cell. Therefore, it is important to release unwanted and irrelevant LTM candidate cells.

In summary, currently since the LTM candidate cell addition and release is based only on L3 measurements, it implies that an LTM UE has to be configured for both L3 and L1 measurements, for the purpose of preparation and execution of LTM. But this could be triggered using event based L1 measurements also, which enhances benefits in terms of both signaling and latency.

LTM events agreed in RAN2 could be used for new candidate cell addition and release.

With the introduction of event based L1 measurements, newly discovered cells i.e configured based on neighbor relations can also be reported to the gNB using an L1 measurement report.

LTM candidate cell addition and release procedure could be triggered based on event based L1 measurement reports, therefore enhancing both signaling and latency.

Configuring both L3 and L1 measurements is redundant in our view and causes unnecessary overhead. On the contrary, there are other RAN2 related study items in R19 where the objective of the study is to reduce measurements by using AI-ML methods. Hence, we think that RAN2 should explore the possibility of configuring UEs with only one kind of measurements for LTM, for example, using L1 event based measurements to configure addition and release of LTM candidate cells. This does not preclude the UE from being configured with L3 measurements for other purposes.

RAN2 discusses the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# Conclusion

We have the following observations:

Observation 1	With the introduction of event based L1 measurements, newly discovered cells i.e configured based on neighbor relations can also be reported to the gNB using an L1 measurement report.

Observation 2	LTM candidate cell addition and release procedure could be triggered based on event based L1 measurement reports, therefore enhancing both signaling and latency.

We have the following proposals:

Proposal 1	The reportOnLeave is configurable for event-triggered L1 measurement report and configurable by the gNB.

Proposal 2	Requesting resource allocation using the legacy BSR procedure to send the event-triggered L1 measurements MAC CE is supported.

Proposal 4	RAN2 discusses the possibility of using event based L1 measurements for the purpose of triggering LTM candidate cell addition and release.

Proposal 5	RAN2 discusses the possibility of avoiding configuring redundant measurements for the same purpose i.e LTM candidate cell addition and release.

# References

RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”

RAN2 Chairman Notes R2_127bis_ChairNotes

RAN2 Chairman Notes R2_128_ChairNotes