3GPP TSG-RAN WG2 Meeting #129	R2-2500339

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.11.2

Source:	InterDigital, Inc.

Title:	Discussion on Random Access operation in SBFD

Document for:	Discussion

# 1. Introduction

In RAN2#127bis meeting, following agreement was made [1]:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

In RAN2#128 meeting, following agreement were made [2]:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell..

In RAN1#119 meeting, following agreement was made [3]:

Agreement It is up to RAN2 to determine the detailed specified/configured conditions/prioritizations for RO selection for initial PRACH transmission attempt in one random access procedure.

In this contribution, we discuss the random access operation on how to select additional-ROs including which metrics to use, and also how to indicate whether it’s an SBFD-aware UE during random access procedure. More details on related discussions are found in our companion contribution [5].

# 2. Discussion

Indication of prioritization for additional-ROs

In RAN2#128 meeting [2], RAN2 discussed the option 1 in CBRA that a network provides the indication on the prioritization of the additional-ROs over legacy-ROs. However, RAN2 has not reached a conclusion to support the option 1 with providing an indication of prioritization for additional-ROs.

In our view, we think that SBFD-aware UE can be configured to prioritize an RO type over the other RO type. The priorities can be configured via NW for flexibly steering UEs with different requirements and capabilities into selecting an RO type, so that none of the ROs are overloaded. The configured priorities and configurations can be indicated to UEs via cell-common indications (e.g., via SIB) or as part of RRC configurations for group-common or UE-specific indications.

For example, the SBFD-aware UEs can be configured to select additional-ROs with higher priority compared to legacy-ROs with lower priority according to the network’s decision (e.g., congestion, configured additional ROs). For instance, the UE can be configured to select from additional-ROs for PRACH preamble transmission, unless the nearest (in time) additional-RO is way farther in time compared to the nearest legacy-RO.

Observation 1. For initial RO selection from legacy-ROs or additional-ROs, the SBFD-aware UEs can be configured with the additional RO type that is considered with higher priority.

Proposal 1. Support indicating the additional-ROs with higher priority over legacy-ROs to the SBFD-aware UEs.

Other metrics than RSRP

In RAN2#128 meeting [2], if there is no such indication from the NW, it is still FFS how a SBFD-aware UE selects additional-ROs or legacy-ROs based on the other metrics than RSRP.

In our view, as another metric for selection additional-ROs or legacy-ROs, initial transmission power needs to be considered. For example, the SBFD-aware UE can be configured to select additional-ROs or legacy ROs based on the initial transmission power (e.g., threshold of transmission power).

Given that there is no such indication from the NW, it can result in overloading one type of ROs by the UEs, resulting in more collisions in initial PRACH preamble transmission. For example, the UEs with low coverage may prefer to transmit the PRACH preamble via legacy-ROs, since they can ramp up the power to higher extents without causing or being affected by the CLI. In case many low-coverage UEs use the legacy-ROs, the potential collision would degrade the performance and PRACH coverage for both SBFD-aware UEs and legacy UEs. On the other hand, in a cell with many SBFD-aware UEs, in case all SBFD-aware UEs use additional-ROs, the legacy-ROs may be left unused, resulting in waste of resources.

Observation 2. Considering RO selection in initial PRACH transmission power, the NW to control the UEs for selecting ROs from legacy-ROs or additional-ROs.

Proposal 2. Support RO selection between additional-ROs and legacy-ROs based on intital PRACH transmission power.

Early indication via MSG3

In RAN1#116 meeting, the following agreement was made [4], and it was left for FFS on how gNB identifies whether a UE is SBFD aware UE or non-SBFD aware UE:

Agreement For SBFD-aware UEs in RRC CONNECTED state, at least further study whether/how to enable Msg2, Msg3 and Msg4 related transmission/reception in SBFD symbols taking into account the following aspects: - Msg2[/Msg4 PDSCH] reception in DL subband(s) - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] frequency resource allocation and frequency hopping - Msg3 repetition - Msg3 PUSCH [/Msg4 HARQ-ACK PUCCH] power control - FFS whether/how gNB to identify whether a UE is SBFD aware UE or non-SBFD aware UE Note: Strive to make progress in accordance to the discussion in AI 9.3.1.

This agreement can be beneficial for SBFD-aware UE in RRC-IDLE/INACTIVE states, where the UE can decide on whether to connect to the selected cell based on SBFD operation or connect to the selected cell based on non-SBFD operation. For example, in case the measured CLI (based on received SSB) is low, the UE could connect to the cell based on SBFD operation. Otherwise, if the measured CLI (based on received SSB) is high, the UE could connect to the cell based on non-SBFD operation.

In another example, the UE can transmit random access preamble in ROs that coincide with SBFD symbols (e.g., via additional-ROs). As such, upon reception of the preamble [1], the gNB will recognize that the UE is SBFD-aware, and that the UE is operating based on SBFD operation. So, gNB can further transmit RAR in SBFD symbols.

The SBFD-aware UE that has decided to perform RA based on legacy-ROs based on conditions of ROs selection, could indicate its support for SBFD operation later and as part of Msg3.

Observation 3. During random access procedure, the SBFD-aware UE can indicate its capability and support for SBFD operation based on PRACH transmission or MSG3.

Proposal 3. Support early indication for SBFD-aware UE in RRC-Idle/Inactive states to indicate UE’s support of SBFD operation via RRACH transmission with additional-RO or MSG3.

# 3. Conclusion

Observation 1. For initial RO selection from legacy-ROs or additional-ROs, the SBFD-aware UEs can be configured with the additional RO type that is considered with higher priority.

Observation 2. Considering RO selection in initial PRACH transmission power, the NW to control the UEs for selecting ROs from legacy-ROs or additional-ROs.

Observation 3. During random access procedure, the SBFD-aware UE can indicate its capability and support for SBFD operation based on PRACH transmission or MSG3.

Proposal 1. Support indicating the additional-ROs with higher priority over legacy-ROs to the SBFD-aware UEs.

Proposal 2. Support RO selection between additional-ROs and legacy-ROs based on intital PRACH transmission power.

Proposal 3. Support early indication for SBFD-aware UE in RRC-Idle/Inactive states to indicate UE’s support of SBFD operation via RRACH transmission with additional-RO or MSG3.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #127bis.

[2] Chairman’s notes, 3GPP TSG RAN WG2 #128.

[3] Chairman’s notes, 3GPP TSG RAN WG1 #119.

[4] Chairman’s notes, 3GPP TSG RAN WG1 #116.

[5] R1-2409567, On SBFD random access operation, Interdigital, Inc.