3GPP TSG-RAN WG2 Meeting #129	R2-2500340

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.11.3

Source:	InterDigital, Inc.

Title:	Discussion on resource configuration in SBFD

Document for:	Discussion

# 1. Introduction

In RAN2#127 meeting, following agreement was made [1]:

Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN1#119 meeting, following agreement was made [2]:

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

In this contribution, we discuss a separate configuration with SBFD resource for requesting SBFD symbols and associated conditions.

# 2. Discussion

SBFD resource configuration

RAN2 agreed to support cell-specific SBFD time/frequency configuration with TDD system [1]. The SBFD configuration includes time slots with SBFD symbols to be used for SBFD-aware UEs and some other time slots with non-SBFD symbols to be used for all UEs (e.g., SBFD-aware UEs and legacy UEs).

The network configures the semi-static SBFD resource with SBFD slots/symbols for SBFD-aware UEs. The SBFD-aware UE may be configured with the same available SBFD resource within a cell. In a SBFD-aware UE perspective, it would be beneficial to receive UL resource in SBFD symbols at least for latency reduction benefits.

SBFD resource separation

According to [2], RAN1 agreed to separate UL power control for PUSCH/PUCCH/SRS transmission with SBFD symbols and non-SBFD symbols. The SBFD-aware UE may transmit a PUSCH with a different transmission power, upon receiving a UL grant, in SBFD symbols.

To support separate PUSCH allocation with the different power, RAN2 can consider that a network configures separate configuration for SBFD resource. For example, a SBFD-aware UE is configured with the dedicated SR ID(s) and associated PUCCH resource to use for a request of UL grant with SBFD symbols. According to the request with separate resource, the network may be able to know the request type of the UL grant (e.g., whether SBFD symbols or non-SBFD symbols), then the network can allocate a PUSCH resource with SBFD symbols for the SBFD-aware UE. Moreover, RAN2 can further discuss to separate other MAC resource (e.g., BSR, PHR) with additional benefits and specification impact on RAN2.

Proposal 1: Support to separate resource configuration (e.g., SR ID, PUCCH) to request an UL grant with SBFD symbols.

SBFD resource selection

It may be possible that a SBFD-aware UE consistently requests an UL grant with SBFD symbols to reduce latency of UL transmission when a new data is triggered, unless a relevant rule for the request is defined. However, the network may not allocate SBFD resource for all the SBFD-aware UEs within a cell, for example, when the number of UEs within the cell is increased over the amount of available configured UL resources for SBFD. Moreover, the network cannot allocate additional SBFD resources (e.g., via dynamic signaling) along with the increased SBFD-aware UEs, due to potential cross link interference across different cells or even in the same cell operating with multiple sectors.

To resolve this issue, RAN2 needs to discuss necessary conditions/rules for a SBFD-aware UE to apply when requesting an UL grant with SBFD symbols or non-SBFD symbols. RAN2 can discuss relevant conditions, for example, latency requirment of triggered new data or UL transmission power, etc. If the new triggered data is not delay-sensitive (i.e., transmission in non-SBFD symbols is satisfied), the SBFD-aware UE can request an UL grant with non-SBFD symbols.

Proposal 2: Discuss when to request an UL grant with SBFD symbols (or non-SBFD symbols) based on the conditions. The detailed conditions are FFS.

# 3. Conclusion

Proposal 1: Support to separate resource configuration (e.g., SR ID, PUCCH) to request an UL grant with SBFD symbols.

Proposal 2: Discuss when to request an UL grant with SBFD symbols (or non-SBFD symbols) based on the conditions. The detailed conditions are FFS.

# 4. References

[1] Chairman’s notes, 3GPP TSG RAN WG2 #127.

[2] Chairman’s notes, 3GPP TSG RAN WG1 #119.