3GPP TSG-RAN WG2 Meeting #129						R2-2500370

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.18

Source:	Samsung

Title:	Handling downlink BWP restriction for MT-SDT

Document for:	Discussion and Decision

# Introduction

RAN2 discussed the downlink BWP restriction for MT-SDT in RAN2#128 based on contribution R2-2410542.

The discussion summary is as follows:

R2-2410542 Quality Indication in Msg3 for SDT        Ericsson          discussion       Rel-19           TEI19

Observation 1      MT-SDT operation over initial BWP restricts DL MT-SDT transmissions to the bandwidth of CORESET 0 in RRC_INACTIVE state.

Observation 2      MT-SDT BWP restriction with no DL quality feedback results in smaller, more conservative TB allocations in the downlink at low MCS values. For larger data volumes, smaller TBS translate to a higher number of subsequent transmissions.

Proposal 1           RAN2 addresses the BWP restriction in MT-SDT by introducing a Channel Quality Indication (CQI) in Msg3 or MsgA.

Option 1: Remove the CORESET 0 restriction for DL BWP for SDT from/after first DL MT-SDT data

Option 2: Introduce Channel Quality Indication (CQI) in Message 3 or MsgA

-     ZTE has sympathy on the problem statement but not sure about the solution, we should just remove the restriction.   The CQI solution is not addressing the problem identified.

-     Qualcomm thinks that removing the BWP restriction is not a good choice.   If we don’t reports CQI in msg3 it will impact RAN4 as well.

-     LG thinks that the BWP restriction is the root cause and we should remove it.   Nokia agrees.

-     Sony explains that a similar issues was identified in MBS and we agreed to expand coreset 0, introduce CFR.  We can reuse that solution.

RAN2 will address this issue.  FFS the exact solution

In this contribution we further discuss how to handle downlink BWP restriction for MT-SDT.

# Discussion

## 2.1 RA-based MT-SDT

Current Procedure

- UE initiate random access procedure and uses the regular (i.e. not defined for MO-SDT) RA resources. A resume cause indicating MT-SDT is included in the RRCResumeRequest/RRCResumeRequest1.

- UE transmit RA preamble on Initial Uplink BWP

- UE receives RAR on Initial Downlink BWP. UE transmits Msg3 on Initial Uplink BWP based on received UL grant in RAR.

- UE monitors PDCCH addressed to TC-RNTI on Initial Downlink BWP. UE receives Msg4 on Initial Downlink BWP. Contention resolution is successful. Random access procedure is successfully completed. TC-RNTI is promoted to C-RNTI.

- UE monitors PDCCH addressed to C-RNTI on Initial Downlink BWP.

- UL and DL data transmission/reception on Initial Uplink BWP/Initial Downlink BWP

To address the issue of BWP restriction for MT-SDT, a downlink BWP for MT-SDT can be introduced. It can be signalled in SIB1. Modified procedure for RA-based MT-SDT can be as follows:

- Network configures a downlink BWP for MT-SDT

- UE initiate random access procedure. A resume cause indicating MT-SDT is included in the RRCResumeRequest/RRCResumeRequest1. UE may also need to indicate the it supports new DL BWP for MT-SDT. New resume cause indicating MT-SDT using DL BWP for MT-SDT can be defined for this purpose. Other option could be separate RA resources/partition for MT-SDT using DL BWP for MT-SDT. Resume cause approach is simpler than separate RA resources/partition for MT-SDT as it works for both CG based and RA based MT-SDT.

- UE transmit RA preamble on Initial Uplink BWP

- UE receives RAR on Initial Downlink BWP. UE transmits Msg3 on Initial Uplink BWP based on received UL grant in RAR.

- UE monitors PDCCH addressed to TC-RNTI on Initial Downlink BWP. UE receives Msg4 on Initial Downlink BWP. Contention resolution is successful. Random access procedure is successfully completed. TC-RNTI is promoted to C-RNTI.

- UE switches to DL BWP configured for MT-SDT after the completion of RA procedure. UE monitor PDCCH addressed to C-RNTI on DL BWP configured for MT-SDT. UL and DL data transmission/reception on Initial Uplink BWP/dedicated DL BWP for MT-SDT

Proposal 1: Introduce a dedicated downlink BWP for MT-SDT. This DL BWP is configured in SIB1.

Proposal 2: Introduce a new resume cause indicating MT-SDT using downlink BWP configured for MT-SDT.

Proposal 3: For RA based MT-SDT, UE switches to DL BWP configured for MT-SDT after the completion of RA procedure.

CG-based MT-SDT can follow the similar approach. UE switches to DL BWP configured for MT-SDT after receiving the acknowledgment (i.e. new DL assignment or UL grant for new transmission) for the initial uplink transmission.

Proposal 4: For CG based MT-SDT, UE switches to DL BWP configured for MT-SDT after receiving the acknowledgment (i.e. new DL assignment or UL grant for new transmission) for the initial uplink transmission.

# Conclusion

Proposal 1: Introduce a dedicated downlink BWP for MT-SDT. This DL BWP is configured in SIB1.

Proposal 2: Introduce a new resume cause indicating MT-SDT using DL BWP for MT-SDT.

Proposal 3: For RA based MT-SDT, UE switches to DL BWP configured for MT-SDT after the completion of RA procedure.

Proposal 4: For CG based MT-SDT, UE switches to DL BWP configured for MT-SDT after receiving the acknowledgment (i.e. new DL assignment or UL grant for new transmission) for the initial uplink transmission.

# References