3GPP TSG RAN WG2 Meeting #129	R2-2500390

Athens, Greece, February 17 - 21, 2025

Title:	Discussion on IoT-NTN TDD mode

Source: 	Iridium, CCL

Document for:	Discussion and Decision

Agenda Item:	8.17

# Introduction

The current NB-IoT NTN network can only operate in FDD mode. While the NB-IoT specifications include a TDD mode, this mode requires changes in direction coordinated around the timing of individual sub-frames (1 msec).   An NTN system has significant propagation time between the UE and SV that makes the TDD mode in its current form not viable for NTN systems.

It is beneficial for the NB-IoT NTN ecosystem to support the TDD mode of operation, allowing carriers with unpaired spectrum to provide NB-IoT NTN services. To enable this, the IoT-NTN TDD mode work item was approved in RAN#105 [9] and later revised in RAN#106 [1] following the conclusion of the RAN1 study phase. The revised WID outlines the following objectives for RAN2:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period. RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. Other necessary impacts on higher layers [RAN2]

The RAN1 study phase agreements can be found in the WI status report [2].

For NB-IoT frame structure type-1 to be utilized on the same frequency in both uplink and downlink, separate spectrum time is allocated for each, separated by a time gap. The period during which uplink operations are allowed is referred to here as the active uplink with duration of U subframes, and the period during which downlink operations are allowed is referred to as the active downlink with duration of D subframes in Figure 1. In RAN#106 it was agreed that the periodicity of the active uplink and downlink times are fixed for NGSO satellite system operating in L-band (1616 – 1626.5 MHz) at 90ms with N=9. There is no RF transmission or reception during inactive times, resulting in discontinuous downlink transmissions. It is also agreed that duration of uplink and downlink active time i.e. D=U=8ms are fixed for MSS spectrum spanning 1616.0 to 1626.5 MHz.

Figure 1 IoT-NTN TDD mode with HD-FDD frame structure

The start subframe of the downlink active time is under discussion in RAN1. The subframes included in the downlink active time are expected be fixed for an IoT-NTN TDD band.

The system parameters to consider in the normative phase are the following:

| Parameters | Values/Comments |
| --- | --- |
| Constellation | LEO at 600 and 1200 km |
| NTN Reference Scenario | Set-1 (3GPP TR 36.763) |
| Targeted MSS band | 1616-1626.5 MHz |
| NB-IoT mode | Standalone with anchor and non-anchor carriers |
| Cell type | Earth-fixed and Earth-moving cells |
| Baseline System for Structure and Procedures | NB-IoT NTN FDD |
| NTN-TDD pattern | Periodic pattern of N=9 radio frame constituted of U=8 non-overlapping set of usable contiguous UL subframe and D=8 set of usable contiguous DL subframe separated by fixed guard period as per Figure 1. |

In this contribution, we present the definition of IoT-NTN TDD mode as discussed in RAN1, and the necessary adaptions and associated UE procedures.

# Discussion

## EUTRA Overall Description

With the introduction of IoT-NTN TDD mode, 36.300 needs to be updated to include IoT-NTN TDD mode definitions and the associated changes as per the proposals below.

Proposal 1: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

Proposal 2: Anchor carrier definitions in 36.300 §3 needs updating:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Proposal 3: In 36.300 §23.21 should be updated to reflect that NTN is applicable to both FDD and IoT-NTN TDD system:

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

## IoT-NTN TDD Pattern and SFN/H-SFN Wraparounds

The values of SFN and H-SFN wrap around every 1024 counts, which causes a change in alignment of the active window when N=9 is used for the IoT-NTN TDD mode. In this mode, the downlink active period occurs every 90ms, causing the active time to fall into different SFNs of the cell at varying H-SFN values. As illustrated in the figure below, at HFN=0, the active time starts at SFN=0, whereas at HFN=1, SFN=0 falls within the inactive period. This pattern reset after every 9th H-SFN cycle = 9 *10240ms = 92.16 seconds.

Figure 2 Illustration of the effect of SFN wrap around on occurrence of downlink active time

There are a few options under discussion in RAN1 on how to handle SFN/H-SFN wraparounds. Depending on RAN1’s decision, these aspects can impact RAN2 specifications.

Observation 1: RAN2 should wait on RAN1 decision on the IoT-NTN TDD mode subframe offset and alignment across H-SFN cycles.

## Scheduling

In this section, we present our views on various scheduling aspects covering MIB-NB, SIB-1-NB, SI, NPDCCH, NPDSCH, NPUSCH and NPRACH with IoT-NTN TDD frame pattern.

#### MIB-NB

RAN1 agreement:

RAN1#119 Observation RAN1 makes the following observations on downlink synchronization: Case 1: For N=9, D=8: - For NPBCH: - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin): - 2.03dB for LEO-1200 (0dBi antenna gain) - -4.07dB for LEO-600 (-5.5dBi antenna gain) - For 640ms combining, 6 sources provided results, with median required SNR of -4.25dB. The link budget margin is (median margin): - 9.76dB for LEO-1200 (0dBi antenna gain) - 3.66dB for LEO-600 (-5.5dBi antenna gain) Observation Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective: - Case 1: N=9, D=8

Observation 2: When operating in IoT-NTN TDD mode, the MIB-NB can be decoded by the UE with existing MIB-NB scheduling parameters and periodicity of 640ms.

#### SIB1-NB

In NB-IoT FDD mode, SIB-1 uses a fixed schedule with a periodicity of 2560ms. It is transmitted on subframe 4 of every other frame and 8 subframes makes up one transport block. The starting frame for the first transmission of the SIB-1 is derived from the cell PCID.

The transport block size and the repetition rate of SIB1 is configurable, ranging from 208 to 680 bits, and 4, 8 or 16 block repetitions within a 2560ms period.

The IoT-NTN TDD frame pattern limits the number of SIB1 subframes a UE can see within a SIB1 periodicity window of 2560ms. Table 1 shows the maximum number of SIB1 subframes that are visible and can be soft combined within a SIB1 periodicity window of 2560ms.

| SIB-1 Repetitions | Maximum number of SIB-1 subframes |
| --- | --- |
| 4 | 3 |
| 8 | 7 |
| 16 | 15 |

Table 1 Maximum number of SIB-1 subframes visible within SIB-1 period of 2560ms in IoT-NTN TDD mode with N=9 and D=8

The link-level simulation results for SIB1-NB decoding performance are provided in our contribution paper, which will be discussed at RAN1#120. The observations and proposals are captured here.

Observation 3: When operating in IoT-NTN TDD mode, the SIB-1 repetition of 16 radio frames works even with the maximum allowed transport block size of 680 bits with adequate link budget margin of 3.91 dB and 4.51 dB for LEO-600 and LEO-1200 respectively.

Observation 4: When operating in IoT-NTN TDD mode, SIB-1 repetition pattern should be configured as 16 radio frames to allow every alternate active downlink opportunity to carry SIB-1 subframes for optimal SIB-1 decoding performance and acquisition time.

Observation 5: When operating in IoT-NTN TDD mode, with SIB-1 repetition pattern of every 16 RFs, maximum delay in receiving SIB-1 is 2520ms.

Proposal 4: When operating in IoT-NTN TDD mode, there is no need to update the SIB-1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

#### System Information (SI) Scheduling

The SI messages are transmitted within periodically occurring time domain windows called SI-windows using scheduling information provided in SIB-1. Each SI message is associated with a SI-window and the SI-windows of different SI messages do not overlap.

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IOT downlink subframes depending on TBS. Any TBS higher than 120 bits uses 8 consecutive NB-IOT downlink subframes.

Every SI window starts at subframe# 0 which gets postponed to subframe#1 as subframe#0 is reserved for NPBCH.

With IoT-NTN TDD mode frame pattern, it is possible that the SI window start time falls in the inactive downlink time. This can be resolved by postponing the SI window to the next active downlink opportunity.

Observation 6: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Observation 7: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks within a single downlink active time opportunity when SI TBS is greater than 120 bits.

Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

##### Overlap Within SI Window

SI transmission postponement to the next active downlink opportunity could result in overlap with the next repetitions within the SI window. This can be resolved by truncating SI transport block transmission if it overlaps with the next SI repetition. As illustrated in Figure 3, transmission of first repetition gets truncated at SFN 8 where start of second repetition is scheduled. Thus, next downlink active time carries subframes from the second repetitions instead of carrying the postponed last three subframes of the first repetition.

Figure 3 Truncation of SI repetition with repetition pattern every 8th radio frame

Observation 8: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 9: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time results in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

##### SI Window Overflow

The SI transmission postponement to the next downlink active time could also result in SI transmission in the subframes outside of the configured SI window. This could be resolved by proposing changes in (36.331, 18.1.0) to truncate any SI transmissions outside of the SI transmission window as shown in Figure 4

Figure 4 SI window overflow shall not be allowed

Observation 10: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in SI transmission in the subframes outside of the configured SI window.

Proposal 8: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 9: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows:

1> if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window.

#### NPDCCH Scheduling

In IoT-NTN TDD mode with existing set of configuration parameters it is possible that NPDCCH search spaces can fall in downlink inactive time. RAN1 has not yet decided the NPDCCH scheduling approach for the new mode.

Observation 11: RAN2 should wait for RAN1 decision on NPDCCH scheduling mechanism for IoT-NTN TDD mode.

## UE Procedures

#### Paging

UE paging is possible only in certain frames and subframes. The frames where paging is allowed are called Paging Frames (PF), and the subframes within these paging frames are referred to as Paging Occasions (PO). The NPDCCH with DCI format N2 is carried in the PO. Each UE is assigned to one of these paging frames based on its IMSI value, so it only needs to monitor one paging occasion per DRX cycle. The subframes that can be used for paging occasions are restricted by the number of POs per frame, and the only allowed subframes are subframe #0, subframe #4, subframe #5, and subframe #9 (36.304 §7.2).

PF is calculated as frames satisfying: SFN mod T= (T div N)*(UE_ID mod N), where:

T = DRX cycle of the UE

UE_ID = IMSI mod 4096

N = min (T, nB)

In IoT-NTN TDD mode, the available paging capacity is reduced because only 8 subframes every 90ms are available to carry paging occasions and their corresponding paging messages. In this mode, the PF and associated PO calculated for a UE based on its IMSI can fall into a downlink inactive time. When a PO falls within a downlink inactive time, it is postponed to the next downlink active time, as illustrated in Figure 5. Consequently, all the paging opportunities within the 90ms period are compressed into the 8ms downlink active time.

Figure 5 Illustration of paging opportunities postponement

When paging opportunities for UEs fall within downlink inactive time and overlap with those in downlink active time, the network scheduler can decide which UEs to page. This is because all UEs will be listening to the paging during the same downlink active time based on the rule of postponement. This behavior is like the existing network behavior where the network prioritizes paging among multiple UEs mapped to the same paging opportunities.

A paging message is transmitted in consecutive NB-IoT downlink subframes. RAN1 has yet to decide how NPDCCH and NPDSCH scheduling can work in IoT-NTN TDD mode with minimal impact on existing specifications. In RAN1#120 (Feb-25), we have proposed expanding the definition of NB-IoT downlink subframes to exclude those falling into downlink inactive time. This proposal could also help postpone any paging opportunities that fall into downlink inactive time to the next downlink active time.

In the case of paging, the network is unaware of the coverage class of a UE. Therefore, NPDCCH/NPDSCH is transmitted with many repetitions to ensure that UEs in poor coverage areas can receive the paging message. According to the link-level simulation results submitted to RAN1#120 for NPDCCH and NPDSCH, four repetitions should be sufficient to achieve a 99% success rate in receiver decoding for the LEO-600 and LEO-1200 reference scenarios.

In IoT-NTN TDD mode, paging in e-DRX can operate according to the existing NB-IoT specifications. A UE can calculate the Paging Hyper Frame (PH) and Paging Transmit Window (PTW) following the established rules. The PTW duration, specified in seconds, can accommodate multiple downlink active time opportunities. Any paging opportunity within a PTW that falls during downlink inactive time will be postponed to the next downlink active time.

Observation 12: Depending on the NPDCCH/NPDSCH scheduling mechanism to be agreed upon in RAN1, changes to the paging procedure for IoT-NTN TDD in 36.304 may not be required.

Observation 13: In IoT-NTN TDD mode, e-DRX can work without any changes. Within PTW, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

Proposal 10: In IoT-NTN TDD mode, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

#### Random Access Procedure

In FDD mode, NPRACH opportunities can be configured with periodicities of 40ms, 80ms, 160ms, 240ms, 320ms, 640ms, 1280ms, and 2560ms, none of which are multiples of 90ms. The current NPRACH configuration parameters do not allow for full alignment with IoT-NTN TDD mode uplink active time occurrences. RAN1 has yet to finalize the NPRACH scheduling mechanism for IoT-NTN TDD mode. Depending on the outcome, it may be necessary to update the NPRACH configuration parameters in ASN.1.

The RAR window size and Contention Resolution timer are specified in terms of NPDCCH periods, with maximum values of 10 and 64 NPDCCH periods, respectively. The maximum value of RAR window size and Contention Resolution timer is capped at 10.24s.  In NTN, start of the RAR window is delayed considering Kmac value as illustrated in Figure 6. The RACH procedure can operate with the existing values of these parameters. However, in IoT-NTN TDD mode, where downlink transmissions are limited to 8ms active time, high network load can delay the response to the RACH preamble. Increasing the RAR window size beyond 10 NPDCCH periods provides additional flexibility for the network scheduler to transmit RAR messages.

Figure 6 RAR window start time and size

Observation 14: Depending on the RAN1 decision regarding the NPRACH scheduling mechanism for IoT-NTN TDD mode, it may be necessary to update the RRC ASN.1 parameters.

Observation 15: In IoT-NTN TDD mode, to allow network scheduling flexibility, it may be required to increase RAR window size.

## Round Trip Delay

Figure 7 Illustration of timing relationship [10]	Figure 8 Round trip path in cross-linked satellite network

In the Figure 7:

- Kmac = Offset or delay configured to equal the roundtrip time between RP (Reference Point) and eNB. The value is defined as a 9-bit integer representing 1-512ms of delay.

- Reference Point (RP) is the uplink time synchronization point where DL and UL frames are aligned

Figure 8 depicts a round-trip communication path between a gateway and a UE in a cross-linked satellite network.

Most of Iridium network traffic is transmitted using 4 or fewer satellites before it is terminated at a gateway. This traffic is easily transmitted with the 512ms of allowable latency permitted by the 3GPP NTN standard. A small percentage of Iridium’s network traffic is transmitted over larger number of satellites before it is terminated at a gateway. While the Iridium network is designed to allow these long-range, inter-satellite routes, the latency inherent in such routing pushes our limits above the 512ms of latency permitted by the maximum value of Kmac.  Another case where this latency might need an increase (in a satellite network in general) would be for gateway diversity/redundancy of the routes to ensure reliable packet delivery; this means sometimes packets need to traverse a non-optimal route.

We believe that other satellite service providers whose constellation supports cross-linking could come across the same issue. Cross-linking is beneficial not only for reliability but also to alleviate congestion, or to save costs. For these reasons Iridium suggests that the Kmac be updated to permit latency of up to 1024ms.

We can see two ways of allowing for larger Kmac; One would be extending the Kmac range by 1 bit, from 512 to 1024 ms. Another way would be creating a new optional parameter “Kmac_extension”, that would be additional to the existing Kmac and the two together would allow for larger delay.

Observation 16: For the cross-linked satellite networks, there may be a need to increase Kmac to allow for longer round-trip delay.

Proposal 11: Increase Kmac, effectively either by increasing defined value from 9 to 10 bits, or by defining another optional parameter “Kmac_extension”, and the two together would allow for larger delay.

# Conclusion

In this contribution, we introduced the IoT-NTN TDD mode and discussed its high-level impact on associated RAN2 procedures. Since scheduling aspects are still under discussion in RAN1, it is premature to assess the impact on RAN2, including any required ASN.1 changes. Our observations and proposals are listed below:

Observation 1: RAN2 should wait on RAN1 decision on the IoT-NTN TDD mode subframe offset and alignment across H-SFN cycles.

Observation 2: When operating in IoT-NTN TDD mode, the MIB-NB can be decoded by the UE with existing MIB-NB scheduling parameters and periodicity of 640ms.

Observation 3: When operating in IoT-NTN TDD mode, the SIB-1 repetition of 16 radio frames works even with the maximum allowed transport block size of 680 bits with adequate link budget margin of 3.91 dB and 4.51 dB for LEO-600 and LEO-1200 respectively.

Observation 4: When operating in IoT-NTN TDD mode, SIB-1 repetition pattern should be configured as 16 radio frames to allow every alternate active downlink opportunity to carry SIB-1 subframes for optimal SIB-1 decoding performance and acquisition time.

Observation 5: When operating in IoT-NTN TDD mode, with SIB-1 repetition pattern of every 16 RFs, maximum delay in receiving SIB-1 is 2520ms.

Observation 6: With the IoT-NTN TDD mode frame pattern, it is possible for the SI window start time to fall within the inactive downlink period.

Observation 7: With IoT-NTN TDD mode frame pattern, it is not possible to transmit all 8 subframes of SI transport blocks within a single downlink active time opportunity when SI TBS is greater than 120 bits.

Observation 8: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time can result in overlap with the transmission of the next SI repetition when N=9 and SI repetition pattern is configured as lower than every 16th radio frame.

Observation 9: With the IoT-NTN TDD mode frame pattern, postponement of the transmission of SI transport block to the next downlink active time results in no overlap when N=9 and the SI repetition pattern is configured as every 16th radio frame.

Observation 10: With IoT-NTN TDD mode frame pattern, the postponement of transmission of SI transport block to the next downlink active time can result in SI transmission in the subframes outside of the configured SI window.

Observation 11: RAN2 should wait for RAN1 decision on NPDCCH scheduling mechanism for IoT-NTN TDD mode.

Observation 12: Depending on the NPDCCH/NPDSCH scheduling mechanism to be agreed upon in RAN1, changes to the paging procedure for IoT-NTN TDD in 36.304 may not be required.

Observation 13: In IoT-NTN TDD mode, e-DRX can work without any changes. Within PTW, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

Observation 14: Depending on the RAN1 decision regarding the NPRACH scheduling mechanism for IoT-NTN TDD mode, it may be necessary to update the RRC ASN.1 parameters.

Observation 15: In IoT-NTN TDD mode, to allow network scheduling flexibility, it may be required to increase RAR window size.

Observation 16: For the cross-linked satellite networks, there may be a need to increase Kmac to allow for longer round-trip delay.

Proposal 1: A description of IoT-NTN TDD mode shall be added in 36.300 [§5] as follows:

Downlink and uplink transmissions are organized into radio frames with 10 ms duration. Three radio frame structures are supported:

- Type 1, applicable to FDD and IoT-NTN TDD;

- Type 2, applicable to TDD;

…

For IoT-NTN TDD mode, Frame Structure Type-1 is used where uplink and downlink transmissions are separated in the time domain and constitute of set of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (GP) as per Figure 1. This pattern is repeated every N radio frames. The values of N, D, U, [TBD for GP and start subframe for D] are fixed per IoT-NTN TDD band specified in [TBD]

Proposal 2: Anchor carrier definitions in 36.300 §3 needs updating:

Anchor carrier: in NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD and IoT-NTN TDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Proposal 3: In 36.300 §23.21 should be updated to reflect that NTN is applicable to both FDD and IoT-NTN TDD system:

In this release of the specification, NTN is only applicable to FDD and IoT-NTN TDD system.

Proposal 4: When operating in IoT-NTN TDD mode, there is no need to update the SIB-1 scheduling mechanism in the existing specifications, except for disabling transmissions during downlink inactive time.

Proposal 5: In IoT-NTN TDD mode, if start of the SI window falls within the downlink inactive time, it shall be postponed to the next downlink active time.

Proposal 6: In IoT-NTN TDD mode, subframes from a single repetition of the SI transmission shall be allowed to be postponed to the next downlink active time opportunity.

Proposal 7: In IoT-NTN TDD mode, SI transmissions shall be truncated if, due to the proposed postponement, they overlap with the transmission of the next SI repetition within the SI window.

Proposal 8: In IoT-NTN TDD mode, SI transmission shall be truncated if, due to the proposed SI transmission postponement, it overflows the configured SI window length.

Proposal 9: In IoT-NTN TDD mode, by making modifications to the SI scheduling mechanism, all SI messages can be delivered to the UE. This can be achieved by updating 36.331 §5.2.3a as follows:

1> if the UE is a NB-IoT UE:

2>	receive and accumulate SI message transmissions on DL-SCH from the start of the SI-window and continue until the end of the SI-window whose absolute length in time is given by si-WindowLength, starting from the radio frames as provided in si-RepetitionPattern and in subframes as provided in downlinkBitmap, or until successful decoding of the accumulated SI message transmissions excluding the subframes used for transmission of NPSS, NSSS, MasterInformationBlock-NB/ MasterInformationBlock-TDD-NB, and SystemInformationBlockType1-NB, and in the IoT-NTN TDD mode excluding subframes outside of active downlink time. If there are not enough subframes for one SI message transmission in the radio frames as provided in si-RepetitionPattern, the UE shall continue to receive the SI message transmission in the radio frames following the radio frame indicated in si-RepetitionPattern;

In the IoT-NTN TDD mode, if postponement of SI subframes due to downlink inactive time results in overlap with the subframes of the next SI repetition, or falls outside SI window, SI transmission should be truncated to the last non-overlapping subframe or last subframe within the SI window.

Proposal 10: In IoT-NTN TDD mode, when calculated paging opportunity of a UE falls in downlink inactive time, it shall get postponed to the next downlink active time.

Proposal 11: Increase Kmac, effectively either by increasing defined value from 9 to 10 bits, or by defining another optional parameter “Kmac_extension”, and the two together would allow for larger delay.

# References

RP-243293, Revised WID on introduction of IoT-NTN TDD mode, RAN#106, 2024-12, Madrid, ES.

RP-242652, Status Report - WID on introduction of IoT-NTN TDD mode, Iridium, RAN#106, 2024-12, Madrid, ES.

3GPP TS 36.213-§16.4 NPSCH, §16.6 NPDCCH

3GPP TS 36.331 E-UTRA Radio Resource Control (RRC) Protocol specification

3GPP TS 36.306 LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities

3GPP TS 36-212 LTE; Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding

3GPP TS 36.304 Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode

3GPP TS 24.008 Mobile radio interface Layer 3 specification; Core network protocols; Stage 3

RP-242415, “New WID on introduction of IoT-NTN TDD mode”, RAN #105, Melbourne, Australia, 2024-09

3GPP TS 36.300, E-UTRA and E-UTRAN; Overall description; Stage 2