3GPP TSG-RAN WG2 Meeting #129		     R2-2500391

Athens, GR: 17th Feb – 21st Feb 2025

Source:	Qualcomm Incorporated

Title:	On Network Side Data Collection

Agenda Item:	8.1.3 NW Side Data Collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, in RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved [3].

This contribution paper discusses the data collection for the network side model training for beam management, CSI Feedback, and positioning enhancements use cases.

# Data Collection for Network Side Models

This section discusses the following:

- Data collection for network-side model training for beam management use case,

- Data collection for two-sided model training for CSI feedback use case, and

- Data collection for network-side model training for positioning enhancements use case.

## 2.1   Data Collection for Network-side Models for the Beam Management

In the RAN2#126 meeting [4], RAN2 made the following agreements for the network-side model training for the beam management use case:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. FFS how to handle case when single RRC message is not sufficient. FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting. FFS whether and what event-based reporting is supported and FFS on network request reporting

In the RAN2#127 meeting [5], RAN2 further agreed on the following for the beam management use case:

Agreements - As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is supported. - UE stores the logged training data at the AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases - When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging. - Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network . - FFS whether AS buffer event based reporting is supported. FFS if we send availability indication or full report if it is supported - FFS on event based data collection/logging - On-demand request from the network is supported. FFS details on signalling

In the RAN2#127bis meeting [6], RAN2 further agreed on the following for the beam management use case,

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

Based on the above agreements, the following issues are FFS

- Training data collection configuration contents,

- Definition of event-triggered data collection,

- Multiple configurations and dynamic activation/deactivation,

- Handling data collection configuration and report during UE mobility.

This section discusses the FFS issues mentioned above.

### 2.1.1	Training Data Collection Configuration Contents

Like any measurement configuration and reporting/logging, the training data configuration should consist of measure targets and logging configuration (e.g., periodicity, event triggers for logging). For training data collection, the UE is expected to log L1-RSRP measurements on SSB/CSI-RS. For the legacy beam management, the resources or resource sets for measurements are configured using the CSI framework. The CSI framework should be used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections. When configuring/activating the training data collection at the UE, the network should ensure that SSB/CSI-RS configured for measurements across different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring) and different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes) lies within the maximum number of SSB/CSI-RS measurements supported at the UE.

Proposal 1: The training data collection configuration consists of

- Measurement targets (i.e., CSI resources or resource sets for measurements), and

- Logging configuration (e.g., periodicity, events for logging measurements).

Proposal 2: The CSI framework is used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections.

Proposal 3: The network should continue to ensure that the maximum number of SSB/CSI-RS configured for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

### 2.1.2	Definition of Event-Trigger Data Collection

In the RAN2#128 meeting [7], RAN2 agreed to discuss the following three radio condition-based event-trigger logging in the [POST128][019][AI PHY] NW side data collection (Nokia) email discussion [8].

- L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

- Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

- L1 beam level measurement.

In the [POST128][019][AI PHY] NW side data collection (Nokia) email discussion, some companies raised concerns about whether the event-trigger data collection may impact performance. Note that, as indicated in Fig. 2.1.2-1, irrespective of periodic or event-triggered data collection methods, the training data collected may not be complete or representative if it is not performed uniformly across different locations in beams of a cell. Furthermore, the event-triggered data collection method is not a requirement from RAN1. Therefore, whether the event-triggered data collection method should be supported should be concluded in RAN2, i.e., an LS to RAN1 for checking whether event-triggered data collection is required.

Fig 2.1.2-1: UEs distribution in the network during data collection for network-side training.

Observation 1: Irrespective of periodic or event-triggered data collection methods, the training data collected may not be complete or representative if it is not performed uniformly across different locations in beams of a cell.

Proposal 4: RAN2 concludes on the support of event-triggered data collection without sending an LS to RAN1.

As discussed in the email discussion [8], the network may want UE to perform training data collection on one or more SSB / CSI-RS (when UE is connected to a given SSB / CSI-RS). The network may configure the UE with indices of SSB/CSI-RS, and the UE performs the training data collection when UE is connected to configured SSB/CSI-RS. Such a configuration can be provided in conjunction with other L1/L3 triggered data collection/logging.

Fig 2.1.2-2: The UEs can be configured with SSB/CSI-RS indices. The UE performs training data collection when connected to the configured with SSB/CSI-RS indices.

Proposal 5: The network may configure the UE with indices of SSB/CSI-RS, and the UE performs the training data collection when UE is connected to configured SSB/CSI-RS. Such a configuration can be provided in conjunction with other L1/L3 triggered data collection/logging.

Furthermore, we also want to highlight that RAN2 should not work on standardization of L1 event-triggered (beam or L1-RSRP threshold) training data collection, rather, RAN2 can wait if L1-triggered data collection/reports are defined within other RAN1/RAN2 WIs, and if they can be used for L1-triggered training data collection/logging.

Proposal 6: RAN2 waits for other WIs to define L1-RSRP triggered data collection/reporting, and such L1 triggers can be used for training data collection/logging (if L1-RSRP triggers for training data collection is agreed to be supported).

### 2.1.3	Multiple Configuration and Dynamic Activation/Deactivation

In the RAN2, the following are FFS issues associated with data collection configuration for network-side model training,

- Whether the UE can be configured with multiple RS configurations for network-side model training,

- Whether dynamic activation/deactivation is supported

Note that the performance of AI/ML functionalities/models depends on the quality/distribution of the training data. For example, with periodic data collection, the network may be able to collect the training data across different beams of the serving cells. However, if a UE is static and configured with periodic data collection, then the collected training data may be redundant. Note that when redundant data is used for training, it may create significant biases. One solution may be that redundancy is removed during the training by the NW-side training server. However, this will require a huge amount of training data to be collected at the UE, as after removing the redundancy the network side may end up with a significantly low amount of meaningful training data. Therefore, the following issues may happen with periodic data collection:

- Huge amount of training data collection at the UEs,

- Significantly high redundancy causing biases during training, if not handled by the training server properly.

Observation 2: Periodic data collection has the following issues,

- Requires a huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- The network-side training server needs to handle redundancy in the training data, otherwise, it may cause biases in the trained models.

As described in Fig. 2.1.2-1 and Fig. 2.1.2-2, for the L1-RSRP predictions of top-1/top-K beams, the training data should be independent and identically distributed (i.i.d) across beams. We need a solution that alleviates/minimizes the redundancy in the data collection and captures a good representation across beams, across beam center vs beam center, and others. This can be achieved by introducing randomness in the training data logging/reporting.

Fig. 2.1.3-1: An example of training data collection where the training data is distributed (equally) across the beams, across the beam center, beam edge, etc.

This section further discusses the FFS issues (discussed above) associated with the data collection configuration.

#### 2.1.3.1	 On Supporting Multiple RS Configuration

In the legacy CSI framework, CSI-ResourceConfig indicates the RS configuration, and CSI-ReportConfig is provided for configuring the CSI report. From TS 38.331 [9], we can see that up to 48 CSI-ReportConfigurations can be provided. Similarly, up to 112 RS Configurations can be provided. Therefore, following the legacy CSI report framework, multiple CSI RS configurations and reporting/logging configurations can be provided.

Observation 3: UE can be configured with up to 112 CSI resource configurations in the legacy CSI framework.

Proposal 7: The UE can be configured with multiple CSI resources or resource sets for training data collection.

–	CSI-ResourceConfig

The IE CSI-ResourceConfig defines a group of one or more NZP-CSI-RS-ResourceSet, CSI-IM-ResourceSet and/or CSI-SSB-ResourceSet.

CSI-ResourceConfig information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIG-START

CSI-ResourceConfig ::=      SEQUENCE {

csi-ResourceConfigId        CSI-ResourceConfigId,

csi-RS-ResourceSetList      CHOICE {

nzp-CSI-RS-SSB              SEQUENCE {

nzp-CSI-RS-ResourceSetList  SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSetsPerConfig)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL, -- Need R

csi-SSB-ResourceSetList     SEQUENCE (SIZE (1..maxNrofCSI-SSB-ResourceSetsPerConfig)) OF CSI-SSB-ResourceSetId  OPTIONAL  -- Need R

},

csi-IM-ResourceSetList      SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSetsPerConfig)) OF CSI-IM-ResourceSetId

},

/* omitted

}

-- TAG-CSI-RESOURCECONFIG-STOP

-- ASN1STOP

–	CSI-ResourceConfigId

The IE CSI-ResourceConfigId is used to identify a CSI-ResourceConfig.

CSI-ResourceConfigId information element

-- ASN1START

-- TAG-CSI-RESOURCECONFIGID-START

CSI-ResourceConfigId ::=            INTEGER (0..maxNrofCSI-ResourceConfigurations-1)

-- TAG-CSI-RESOURCECONFIGID-STOP

-- ASN1STOP

6.4	RRC multiplicity and type constraint values

/*omitted

maxNrofCSI-ResourceConfigurations       INTEGER ::= 112     -- Maximum number of resource configurations

/*omitted

maxNrofNZP-CSI-RS-ResourceSetsPerConfig INTEGER ::= 16      -- Maximum number of resource sets per resource configuration

/*omitted

maxNrofCSI-SSB-ResourceSetsPerConfig    INTEGER ::= 1       -- Maximum number of CSI SSB resource sets per resource configuration

/*omitted

#### 2.1.3.2	On Supporting Dynamic Activation/Deactivation

From TS 38.214 [10], we have the following,

- The reporting configuration for CSI can be aperiodic (using PUSCH), periodic (using PUCCH), or semi-persistent (using PUCCH, and DCI activated PUSCH).

- The CSI-RS Resources can be periodic, semi-persistent, or aperiodic.

Furthermore, Table 5.2.1.4-1 in TS 38.214 illustrates supported combinations of CSI reporting configuration and CSI-RS Resource configuration (also copied below).

| CSI-RS Configuration | Periodic CSI Reporting | Semi-Persistent CSI Reporting | Aperiodic CSI Reporting |
| --- | --- | --- | --- |
| Periodic CSI-RS | No dynamic triggering/activation | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Semi-Persistent CSI-RS | Not Supported | For reporting on PUCCH, the UE receives an activation command, as described in clause 6.1.3.16 of [10, TS 38.321]; for reporting on PUSCH, the UE receives triggering on DCI | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |
| Aperiodic CSI-RS | Not Supported | Not Supported | Triggered by DCI; additionally, subselection indication as described in clause 6.1.3.13 of [10, TS 38.321] possible as defined in Clause 5.2.1.5.1. |

Observation 4: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is periodic, the Report Configuration can be set to any type (periodic, semi-persistent, or aperiodic).

Observation 5: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is semi-periodic, the Report Configuration can be Semi-persistent or Aperiodic.

Observation 6: When the Resource Configuration (CSI-RS Configuration = NZP CSI RS) is aperiodic, the Report Configuration can only be aperiodic.

As discussed previously, to achieve adequate AI/ML functionality/model performance, the redundancy in the training data should be minimized. This can be achieved by introducing randomness in the training data logging/reporting. Following the above observation 2 – observation 4 from the legacy CSI framework, Fig. 2.2.1.2-1 illustrates a potential solution for redundancy in the training data collection.

Fig. 2.1.3.2-1: Training data collection: UE can be configured with periodic/semi-persistent/aperiodic RS configuration, but the CSI report configuration is semi-persistent/aperiodic.

Proposal 8: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

### 2.1.4	Handling Data Collection Configuration and Reporting during UE Mobility

In the RAN2#126 meeting [4], RAN2 agreed that “For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.” This section discusses several issues on handling data collection configuration and reporting of collected training data during UE mobility.

Observation 7: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

#### 2.1.4.1 	UE Behavior when the UE has Unretrieved Training Data During Mobility

This section discusses the UE behavior when the UE has unretrieved training data during the mobility. In the legacy immediate MDT, the UE reports the measurements to the gNB that configures the measurements, i.e., logging and reporting of the measurements later is not supported. The serving gNB determines measurement configuration (for example, based on the configuration received from the OAM) and receives the measurements from the UE which are then forwarded to the TCE. During the handover/reestablishment/resume/reconnection, the immediate MDT configuration is forwarded from the source gNB/AMF to the target gNB if they belong to the MDT PLMN List configured by the OAM. While the reporting of measurements collected is immediate, UE is not required to check the PLMN before reporting. The UE is not aware of the MDT PLMN List. Therefore, no undesired leakage of information/measurements.

In SON and logged MDT reports, the UE stores PLMN identity list (equivalent PLMNs) where the UE can report SON/logged MDT reports. This ensures no undesired leakage of information/measurements.

For the training data collection, UE may need to allocate sufficient resources such as memory and power. Therefore, discarding the unretrieved training data during UE mobility is not desired. We prefer RAN2 to clarify the UE behavior when UE has unretrieved logged training data during handover, RRC reestablishment, handover failures, radio link failures, or RRC state transitions. RAN2 needs to clarify further if there are concerns associated with the leakage of the training data when UE moves from

- One PLMN to another PLMN, i.e., whether training data collected in one PLMN can reported to any PLMN, and

- One infra-vendor to another infra-vendor, i.e., whether training data collected at the NG-RAN from one infra-vendor can reported to NG-RAN from another infra-vendor.

Observation 8: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by configuring gNB1.

Proposal 9: Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

Proposal 10: RAN2 should clarify if there are concerns associated with the leakage of the training data when UE moves from

- One PLMN to another PLMN, i.e., whether training data collected in one PLMN can reported to any PLMN, and

- One infra-vendor to another infra-vendor, i.e., whether training data collected at the NG-RAN from one infra-vendor can reported to NG-RAN from another infra-vendor.

##### 2.1.4.1.1  On the leakage of information from one PLMN to another PLMN

Let us consider the following scenario (see Fig. 2.1.4.1.1-1) where a cell gNB1 (belonging to vendor 1 and PLMN 1) configures the UE with the training data collection (measurement and reporting configuration), but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before the handover, or

- UE has RLF before the training data can be retrieved by gNB1.

2.1.4.1.1-1: OAM-centric data collection, whether the logged training data can be reported to other PLMN or other-infra-vendor cells if they belong to the same/different PLMN.

##### 2.1.4.1.2  On the leakage of information from one infra-vendor to another infra-vendor

In the legacy L3 measurement collection and reporting, data logging is not supported. This implies that the serving gNB configures the UE with measurement/reporting configuration, and receives the measurements. During the handover, the measurement/reporting configuration is handled by the following methods

- UE maintains the configuration if configured with need code M

- UE releases the configuration if configured with need code R

- Serving/target cell can explicitly release the configuration by sending a reconfiguration message

Consider the following scenario (see Fig. 2.1.4.1.2-1) where a single PLMN deploys the cells/gNBs from two infra-vendors. Let us assume that gNB1 from vendor 1 configures the data collection at the UE, but cannot retrieve the logged training data because of the following reasons

- The gNB1 was overloaded and couldn’t retrieve the training data before the handover, or

- UE has RLF before the training data can be retrieved by gNB1.

Fig. 2.1.4.1.2-1: gNB-centric data collection, whether the logged training data can be reported to other infra-vendor cells if they belong to the same/different PLMN.

The UE allocates significant resources, such as memory and power, for NW-side training data collection, therefore reporting unretrieved training data should be supported. Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent on NW-side data collection. To allow reporting of unretrieved data, the UE should be provided with area configuration (e.g., list of cells/gNBs/PLMNs) where UE can report the logged training data.

Proposal 11: The UE is provided with an area configuration (e.g., list of cells/gNBs/PLMNs) for the reporting of the unretrieved logged training data.

#### 2.1.4.2 Handing of Training Data Collection Configuration during UE Mobility

##### 2.1.4.2.1 UE Behaviour Related to the Training Data Collection Configuration

In the legacy CSI framework, the CSI-MeasConfig is used to configure CSI-RS (reference signal) belonging to the serving cell. Following the legacy CSI framework, the training data collection configuration for the beam management may be released upon handover/RLF/RRC state transitions from CONNECTED to IDLE/INACTIVE states. Therefore, UE is not configured with area configuration for the measurements.

Proposal 12: UE is not configured with area configuration for the measurement of resources or resource sets for training data collection, as these configurations are cell-specific.

##### 2.1.4.2.2 Handing of Training Data Collection Configuration at the Network

Legacy immediate MDT can be signaling-based MDT or management-based MDT. Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM. There is no need to collect training from specific UEs for the network-side model training. Therefore, there is no need to support signaling-based immediate MDT for network-side model training.

Observation 9: Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM.

Observation 10: There is no need to collect training from specific UEs for the network-side model training.

Proposal 13: Signaling-based immediate MDT is not supported for NW-side training data collection.

On the other hand, the management-based immediate MDT is used for collecting data in a specific area, and UE selection for immediate MDT is performed at the gNB. For the management-based MDT, the Management Based MDT PLMN List IE is sent HANDOVER REQUEST from the source to target NG-RAN, and from AMF to the target NG-RAN. Therefore, if the serving cell belongs to the Management Based MDT PLMN List IE, then they should, if supported, select UEs for configuring training data collection configuration. Furthermore, note that Management Based MDT PLMN List IE is stored in the UE context for management-based MDT data collection.

Observation 11: For management-based MDT, the data collection is performed at cells/gNBs if they belong to the Management-Based MDT PLMN List IE and support MDT data collection.

For the management-based MDT, the OAM may indicate cell(s)/frequency(ies) on which the training data collection should be performed. However, as the CSI resource configuration is serving cell-specific, therefore during handover/Reestablishment/RRC state transition, the new NG-RAN should provide data training collection configuration based on area configuration indicated by OAM.

Proposal 14: For OAM-centric data collection (i.e., for management-based MDT), the OAM may indicate cell(s)/frequency(ies) to the NG-RANs on which the training data collection should be performed.

#### 2.1.4.4 Override Protection of unretrieved Training Data

The UE may allocate sufficient resources and power for NW-side training data collection, the override protection of unretrieved training data should be supported. RAN2 is requested to discuss whether/how to override protection of unretrieved training data collection should be supported.

Observation 12: UEs allocate sufficient resources and power for NW-side training data collection, thus override protection of unretrieved training data should be supported.

Proposal 15: RAN2 should further discuss whether/how to override protection of unretrieved training data collection should be supported.

## 2.2   Data Collection for Network-side Model for Positioning Enhancements

### 2.2.1	UE-assisted positioning with LMF-side model (Case 2b)

For (2nd priority) Case 2b, the UE reports DL-PRS-based measurements to an LMF which may be any of the existing measurement types (e.g., RSTD, UE Rx-Tx Time Difference, etc.) or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.). For Case 2b data collection, SA2 agreed the general procedure in Fig. 2.2.1-1 [11].

Fig. 2.2.1-1: Data collection by LMF to train the AI/ML based positioning model using UE measurements (Case 2b) (according to [11]).

At Step 1 in Fig. 2.2.1-1, an LMF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover AMF(s) in an area of interest. At Step 2, the LMF subscribes to an AMF to retrieve the list of SUPIs located in an area of interest using the Namf_EventExposure_Subscribe_Request operation. The AMF then responds with a Namf_EventExposure_Subscribe_Response operation including a list of SUPIs in the area of interest at Step 3, possibly together with the UE positioning capabilities if available and requested at Step 2. After the LMF verified the user consent for data collection for model training for each SUPI at Step 4, the LMF requests necessary data from the UE to train the LMF-side models at Step 5.

Observation 13: A list of available UEs for data collection together with their UE positioning capabilities and SUPIs can be available at an LMF.

To obtain the input data for model training from the UE at Step 5, any of the existing positioning procedures can be used [11],[12]. For example, Fig. 2.2.1-2 shows an MT-LR procedure as specified in clause 6.1.2 of TS 23.273 [11] for data collection.

Fig. 2.2.1-2: MT-LR procedure for data collection by an LMF (Case 2b).

Steps 1-2 in Fig. 2.2.1-2 are the same as Steps 1-3 in Fig. 2.2.1-1. At Step 3, the user consent for data collection for each UE is checked as in Step 4 of Fig. 2.2.1-1. However, given that the UE "ground-truth" location may need to be obtained for data collection, the UE LCS Privacy Settings [11] should also be checked as indicated by Step 3 in Fig. 2.2.1-2. If the UE's LCS Privacy Settings indicate that the UE must either be notified or notified with privacy verification and if the UE supports LCS notification (according to the UE capability information), a notification invoke message is sent to the target UE at Step 4 in Fig. 2.2.1-2 as defined for an MT-LR in TS 23.273 [11].

Proposal 16: Since the data collection for Case 2b may involve determining a UE "ground-truth location", the UE LCS Privacy Settings [11] should also be checked by the NW. If the LCS Privacy Settings indicate that privacy verification/notification is required, the NW should instigate the LCS Location Notification signalling according to TS 23.273. The details are up to SA2.

For obtaining a "ground-truth" location, for both, labeling the training data and model performance metric calculation, the LMF may use any UE and NW supported positioning procedure at Step 5 in Fig. 2.2.1-2. This may be any of the UE-based, UE-assisted, or NW-based location method specified in TS 38.305 [12]. The UE location determination must be independent on the data collected for model training. For example, the LMF may use a RAT-independent location method (e.g., GNSS) or a NW-based method (e.g., UL-TDOA) to obtain the label.

Proposal 17: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), existing UE positioning procedures [12] are used to obtain the UE location estimate by an LMF.

The actual data collected for Case 2b are "DL-PRS measurements". These may be existing measurement types (e.g., RSTD, UE Rx-Tx Time Difference, etc.) or any new measurement types defined by RAN1 (e.g., channel impulse response measurements, etc.). Requesting and Reporting of the "training data" may have LPP impacts. However, these impacts may be the same (or similar) to those required for the positioning operation (inference) anyhow. Similar, for performing the measurements, the UE would need assistance data (e.g., DL-PRS assistance data, etc.). These assistance data are likely the same to those needed for positioning.

Observation 14: Potential LPP changes to support data collection for Case 2b may be the same (or similar) to those required for the positioning operation (inference) and depend on further RAN1 progress.

A pre-requisite for any LPP signalling (e.g., at Step 5 and 6 in Fig. 2.2.1-2) is that a LCS Correlation Identifier and the AMF Identity has been passed to the LMF by the serving AMF (see e.g., clause 6.11 of TS 23.273 [11]). However, since the LMF initiates the data collection procedure without a location request from an AMF, no LCS Correlation ID would be available at the LMF. With the current SA2 procedure shown in Fig. 2.2.1-1, the only way to obtain a LCS Correlation ID for each UE seems be at Steps 2/3 in Fig. 2.2.1-1. E.g., the AMF may assign a LCS Correlation ID for each UE when providing the available UE information to the LMF, or the SUPI functions as a LCS Correlation ID.  Note, for PRUs the LMF would have a LCS Correlation ID obtained during the PRU registration procedure (see clause 6.17 of TS 23.273 [11]). Since the AMF does not know if and when a LMF initiates the data collection for a particular UE, the LCS Correlation ID would essentially always be a "Deferred LCS Correlation Identifier" (e.g., for certain UEs included in the list provided by the AMF, the LMF may never instigate the data collection procedure, since, e.g., some UEs are not suitable or not sufficient capable for that purpose).

Observation 15: A pre-requisite for any LPP and LCS signalling used for data collection by an LMF is that a LCS Correlation Identifier and the AMF Identity has been passed to the LMF by the serving AMF. The details are up to SA2.

For the LPP transport at Step 6 in Fig. 2.2.1-2, both, control plane NAS signalling and user plane signalling are already specified [11], [12]. The LMF may trigger the user plane connection establishment procedure (when desired and supported) as described in clause 6.18 of TS 23.273 [11] for the LPP session.

Observation 16: LPP transport (e.g., for positioning and data collection) can already support both, NAS CP and secure UP TCP/TLS transport.

If periodic or event triggered measurement reporting is desired, the LMF may instigate a deferred MT-LR procedure as specified in clause 6.3 of TS 23.273 [11], using a desired event type (e.g., periodic location, motion event, area event, etc.). Additional trigger criteria can be added to existing messages/procedures, if needed. An example is illustrated in Fig. 2.2.1-3.

Fig. 2.2.1-3: Deferred MT-LR procedure for periodic and triggered data collection by an LMF (Case 2b).

Steps 1-4 in Fig. 2.2.1-3 are the same as in Fig. 2.2.1-2. At Step 5, the LMF may perform a LPP positioning procedure to obtain the UE positioning capabilities or transfer any assistance data to the UE. The LMF may also obtain an initial location estimate for the UE at this step. At step 6a, the LMF sends a supplementary services LCS Periodic-Triggered Invoke Request to the UE via the serving AMF by invoking the Namf_Communication_N1N2MessageTransfer service operation as described in TS 23.273 [11]. The LCS Periodic-Triggered Location Invoke carries the data collection request information incl. the periodic or event triggered reporting criteria and whether user plane reporting of Event Reports at Step 9 is requested. The LCS Periodic-Triggered Location Invoke message could be the same as currently specified [11] or may explicitly indicate that this is a request for data collection (which, however, seems not strictly needed). The LCS Periodic-Triggered Location Invoke also includes a Deferred Routing Identifier, which can be the identification of the LMF when the LMF will act as a serving LMF or a default LMF identification otherwise [11]. The Deferred Routing Identifier may be global (e.g. an IP address or UUID) or may be local. The Deferred Routing Identifier is used for routing in Step 9a.

Note, as part of NAS transport of the LCS Periodic-Triggered Location Invoke from the serving AMF to the UE, the serving AMF includes an immediate routing identifier in the NAS transport message containing the LCS Correlation Identifier. The same as in Observation 15 applies also here: SA2 has to specify how this LCS Correlation Identifier can be obtained for the data collection use case (i.e., without location request from an AMF).

If the request in Step 6a can be supported, the UE returns a supplementary services acknowledgment via the serving AMF to the LMF at Step 6b.

At Step 7, the UE then monitors for the occurrence of the trigger or periodic event requested in step 6a. If the UE has all available assistance data (e.g., from Step 5), the UE may also collect the training data together with the ground-truth label as requested at Step 6a.

At Step 9a, the UE sends a supplementary services Event Report message to the LMF. If CP transport is used, the Event Report is sent via the serving AMF (which may be different to the original serving AMF from Step 6 (due to mobility)). Otherwise, if UP transport is used, the Event Report is sent via the established UP connection indicated at Step 6a via UPF as described in TS 23.273 [11]. The Event Report may include any data collected at Step 7 (e.g., training data and UE ground-truth location (as requested at Step 6a) in an embedded LPP PDU). If CP is used, the UE also includes the Deferred Routing Identifier received in step 6a in the NAS Transport message used to transfer the Event Report from the UE to the AMF. The AMF then forwards the Event Report to the indicated LMF.

If no training data are available when the event is triggered (e.g., due to lack of required assistance data), the LMF may initiate any required LPP procedure to obtain the required data at Step 10 (e.g., the same as in Step 5/6 in Fig. 2.2.1-2).

At Step 11, the UE continues to monitor for further periodic or trigger events as in Step 7 and instigates steps 8-10 each time a trigger event is detected.

Proposal 18: For LMF-side data collection in Positioning Enhancements Case 2b, existing LCS and UE positioning procedures (e.g., NI-LR, MT-LR and/or deferred MT-LR) are used to obtain the desired measurements from a UE/PRU. The details are up to SA2.

The procedure for cancellation of reporting of data collection by a UE is already specified in TS 23.273 [11], clause 6.3.4 and is repeated in Fig. 2.2.1-4 below.

Fig. 2.2.1-4: UE Cancellation of a data collection according to [11].

Steps 2 and 3 in Fig. 2.2.1-4 are the same as specified for the Deferred MT-LR in Clause 6.3.2 of TS 23.273 [11]. The UE sends a Cancel Location request message at Step 2 to the LMF which is transferred via the serving AMF and is delivered to the LMF using an Namf_Communication_N1MessageNotify service operation. The UE includes the Deferred Routing Identifier originally received in step 6a in Fig. 2.2.1-3 in the NAS Transport message used to transfer the cancel location request from the UE to the AMF. The AMF then forwards the cancel location request to the indicated LMF.

Observation 17: The procedure for UE cancellation of data collection is already specified by SA2 in TS 23.273 [11], which implies that a (Deferred) MT-LR can be used for data collection.

### 2.2.2	Network-based positioning with gNB-side model (Case 3a) and LMF-side model (Case 3b)

For both, Case 3a and Case 3b the UE only transmits positioning SRS, and therefore, no additional RAN2 impacts are foreseen (unless RAN1 defines new/additional SRS parameter). RAN2 already agreed at RAN2#126:

- RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b.

Case 3a and Case 3b are currently discussed in RAN3, which already made progress on these two cases.

Observation 18: Procedures to support AI/ML positioning Case 3a and Case 3b are discussed in RAN3 and there is no additional RAN2 specification work for Case 3a/3b foreseen.

# Conclusion

Data collection and reporting for beam management use case

Proposal 1: The training data collection configuration consists of

- Measurement targets (i.e., CSI resources or resource sets for measurements), and

- Logging configuration (e.g., periodicity, events for logging measurements).

Proposal 2: The CSI framework is used for configuring measurement targets (i.e., CSI resource or resource sets) for training data collections.

Proposal 3: The network should continue to ensure that the maximum number of SSB/CSI-RS configured for measurements lies within the maximum number of SSB/CSI-RS measurements supported at the UE per slot and per FR. That includes SSB/CSI-RS used for

- Different non-AI/ML-related tasks (i.e., legacy CSI reporting for different purposes), and

- Different AI/ML-related tasks (e.g., training data collection configuration, inference, and monitoring), and

- Different AI/ML features (e.g., beam management, CSI prediction, CSI Feedback).

Observation 1: Irrespective of periodic or event-triggered data collection methods, the training data collected may not be complete or representative if it is not performed uniformly across different locations in beams of a cell.

Proposal 4: RAN2 concludes on the support of event-triggered data collection without sending an LS to RAN1.

Proposal 5: The network may configure the UE with indices of SSB/CSI-RS, and the UE performs the training data collection when UE is connected to configured SSB/CSI-RS. Such a configuration can be provided in conjunction with other L1/L3 triggered data collection/logging.

Proposal 6: RAN2 waits for other WIs to define L1-RSRP triggered data collection/reporting, and such L1 triggers can be used for training data collection/logging (if L1-RSRP triggers for training data collection is agreed to be supported).

Observation 2: Periodic data collection has the following issues,

- Requires a huge amount of training data collection at the UEs, as there may exist significantly high redundancy,

- The network-side training server needs to handle redundancy in the training data, otherwise, it may cause biases in the trained models.

Observation 3: UE can be configured with up to 112 CSI resource configurations in the legacy CSI framework.

Proposal 7: The UE can be configured with multiple CSI resources or resource sets for training data collection.

Observation 4: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is periodic, the Report Configuration can be set to any type (periodic, semi-persistent, or aperiodic).

Observation 5: When the Resource Configuration(CSI-RS Configuration = NZP CSI RS) is semi-periodic, the Report Configuration can be Semi-persistent or Aperiodic.

Observation 6: When the Resource Configuration (CSI-RS Configuration = NZP CSI RS) is aperiodic, the Report Configuration can only be aperiodic.

Proposal 8: Dynamic activation/deactivation of training data collection and logging should be used to randomize the training data collection and logging, following semi-persistent and aperiodic CSI reporting as the reference.

Observation 7: For the beam management use case, RAN2 agreed that logging of L1-RSRP measurement for gNB-centric / OAM-centric data collection for NW-side model training is an optional feature for the UE.

Observation 8: In the following scenarios, the configuring gNB may not be able to retrieve the logged training data

- The configuring gNB was overloaded and could not retrieve the training data before handover, or

- UE has RLF before the training data can be retrieved by configuring gNB1.

Proposal 9: Discarding the unretrieved training data upon handover / RLF / RRC state transition is not desired, as it is a waste of UE power and memory spent towards NW-side data collection.

Proposal 10: RAN2 should clarify if there are concerns associated with the leakage of the training data when UE moves from

- One PLMN to another PLMN, i.e., whether training data collected in one PLMN can reported to any PLMN, and

- One infra-vendor to another infra-vendor, i.e., whether training data collected at the NG-RAN from one infra-vendor can reported to NG-RAN from another infra-vendor.

Proposal 11: The UE is provided with an area configuration (e.g., list of cells/gNBs/PLMNs) for the reporting of the unretrieved logged training data.

Proposal 12: UE is not configured with area configuration for the measurement of resources or resource sets for training data collection, as these configurations are cell-specific.

Observation 9: Signaling-based immediate MDT is used for collecting data from specific UEs, and the UE selection for immediate MDT is performed at the OAM.

Observation 10: There is no need to collect training from specific UEs for the network-side model training.

Proposal 13: Signaling-based immediate MDT is not supported for NW-side training data collection.

Observation 11: For management-based MDT, the data collection is performed at cells/gNBs if they belong to the Management-Based MDT PLMN List IE and support MDT data collection.

Proposal 14: For OAM-centric data collection (i.e., for management-based MDT), the OAM may indicate cell(s)/frequency(ies) to the NG-RANs on which the training data collection should be performed.

Observation 12: UEs allocate sufficient resources and power for NW-side training data collection, thus override protection of unretrieved training data should be supported.

Proposal 15: RAN2 should further discuss whether/how to override protection of unretrieved training data collection should be supported.

Data collection and reporting for positioning enhancements use case:

Observation 13: A list of available UEs for data collection together with their UE positioning capabilities and SUPIs can be available at an LMF.

Proposal 16: Since the data collection for Case 2b may involve determining a UE "ground-truth location", the UE LCS Privacy Settings [11] should also be checked by the NW. If the LCS Privacy Settings indicate that privacy verification/notification is required, the NW should instigate the LCS Location Notification signalling according to TS 23.273. The details are up to SA2.

Proposal 17: For Positioning Enhancements Case 2b ground-truth label generation (for training and label-based monitoring), existing UE positioning procedures [12] are used to obtain the UE location estimate by an LMF.

Observation 14: Potential LPP changes to support data collection for Case 2b may be the same (or similar) to those required for the positioning operation (inference) and depend on further RAN1 progress.

Observation 15: A pre-requisite for any LPP and LCS signalling used for data collection by an LMF is that a LCS Correlation Identifier and the AMF Identity has been passed to the LMF by the serving AMF. The details are up to SA2.

Observation 16: LPP transport (e.g., for positioning and data collection) can already support both, NAS CP and secure UP TCP/TLS transport.

Proposal 18: For LMF-side data collection in Positioning Enhancements Case 2b, existing LCS and UE positioning procedures (e.g., NI-LR, MT-LR and/or deferred MT-LR) are used to obtain the desired measurements from a UE/PRU. The details are up to SA2.

Observation 17: The procedure for UE cancellation of data collection is already specified by SA2 in TS 23.273 [11], which implies that a (Deferred) MT-LR can be used for data collection.

Observation 18: Procedures to support AI/ML positioning Case 3a and Case 3b are discussed in RAN3 and there is no additional RAN2 specification work for Case 3a/3b foreseen.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- Meeting Report, “RAN2#126 meeting report”

- Meeting Report, “RAN2#127 meeting report”

- Meeting Report, “RAN2#127bis meeting report”

- Meeting Report, “RAN2#128 meeting report”

- [POST128][019][AI PHY] NW side data collection (Nokia) email discussion report

- TS 38.331, RRC Specification

- TS 38.214

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".