3GPP TSG-RAN WG2 Meeting #129		     R2-2500392

Athens, GR: 17th Feb – 21st Feb 2025

Source:	Qualcomm Incorporated

Title:	On LCM for UE-sided Models for Beam Management

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, RAN #102, a new work item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This paper discusses the “LCM of UE-sided models for beam management use case” considering recent RAN1/RAN2 agreements.

# Discussion on LCM of UE-sided Models for Beam Management

## 2.1 Applicability Signaling Aspects

In RAN1 LS Reply [4], RAN1 agreed on the following for the applicability signaling

Agreement - In Step 3, the following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig Any specification impact is a separate discussion

From the RAN1 LS reply [4], we can make the following observation

- In step 3, for determining the applicability, the NW can configure the UE with

  - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

  - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

- In step 5, the network can configure the UE with

  - CSI-ReportConfig for inference configuration, if the B) is used in step 3

  - Optionally, update CSI-ReportConfig for inference configuration if the A) is used in step 3

Observation 1: From the RAN1 LS reply [4], we can make the following observation

- In step 3, for determining the applicability, the NW can configure the UE with

  - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

  - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

- In step 5, the network can configure the UE with

  - CSI-ReportConfig for inference configuration, if the B) is used in step 3

  - Optionally, update CSI-ReportConfig for inference configuration, if the A) is used in step 3

In this section, we discuss the following

- Using CSI-ReportConfig to determine the applicability over using inference related parameter sets,

- How the applicability information should be reported after step 5, and

- Whether implicit or explicit signaling should be used for signaling applicability.

### 2.1.1 Inference Related Parameter Set vs. CSI Report Configuration for Applicability Determination

The UE side trains the AI/ML models for one or more CSI resource/resource sets and associated IDs signaled (e.g., using CSI-ReportConfig) by the gNB during the data collection (see Fig. 2.1.1-1). To determine the applicability, the UE needs to check whether the UE has trained models for RS configurations (i.e., [set A, set B] RS sets) and associated IDs. Let us consider the two scenarios where

- In step 3, for determining the applicability, the NW can configure the UE with A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied).

- In step 3, for determining the applicability, the NW can configure the UE with B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

Observation 2: To determine the applicability, the UE needs to check whether the UE has trained models for RS configurations (i.e., [set A, set B] RS sets) and associated IDs.

Fig. 2.1.1-1: Measurement configuration for training data collection and UE models

#### 2.1.1.1 A) One or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) is configured in step 3 for determining the applicability

If the network configures A) One or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied), it will be easier for the UE to determine the applicability because it will contain RS configurations (i.e., [set A, set B] RS sets) and associated IDs. This will avoid ambiguity at the UE, and the UE can accurately determine the applicability of the CSI-ReportConfiguration for inference configuration.

Observation 3: If the network configures A) One or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in the CSI framework as the working assumption applied), the UE can accurately determine the applicability because the CSI-ReportCOnfig for inference contains RS configurations (i.e., [set A, set B] RS sets) and associated IDs.

#### 2.1.1.2 B) One set or multiple sets of inference related parameters for applicability report only (not for inference) is configured in step 3 for determining the applicability

In RAN1 LS Reply [4], RAN1 replied that the inference related parameter set may contain IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

- The associated ID

  - Note: this doesn’t imply the associated ID is mandatory

- Set A related information

- Set B related information

- Report content related information

- For BM-Case 2,

  - Time instances related information for measurements

  - Time instances related information for prediction

Note that the RAN1 LS reply [4] did not mention the details on set A related information, set B related information, and others. We considered two scenarios here for discussion as follows:

- The inference related parameter set contains much richer IEs in/or the IEs referred by CSI-ReportConfig, such as RS configurations (i.e., [set A, set B] RS set) and associated ID. This helps the UE to accurately determine inference related parameter sets applicability, as the UE can check whether the UE has a trained model available for the configured RS configurations (i.e., [set A, set B] RS set) and associated ID. This also implies that a single CSI-ReportConfig for inference can be derived from the inference related parameter set.

- The inference related parameter sets contain IEs in/or the IEs referred by CSI-ReportConfig and associated IDs, however, do not contain RS configurations (i.e., [set A, set B] RS set). One or more CSI-ReportConfigs can be derived/configured by the network to the UE in step 5 based on applicable inference related parameter sets in step 4.

Observation 4: Considering inference related parameters for applicability report only (not for inference) is provided in step 3 and inference configuration (i.e., CSI-ReportConfig) is provided in step 5, there may exist following cases

- The inference related parameter set contains much richer IEs in/or the IEs referred by CSI-ReportConfig, such as RS configurations (i.e., [set A, set B] RS set) and associated ID (see Fig. 2.1.1.2-1). This helps the UE to accurately determine inference related parameter sets applicability, as the UE can check whether the UE has a trained model available for the configured RS configurations (i.e., [set A, set B] RS set) and associated ID. This also implies that a single CSI-ReportConfig for inference can be derived from the inference related parameter set.

- The inference related parameter sets contain IEs in/or the IEs referred by CSI-ReportConfig and associated IDs, however, do not contain RS configurations (i.e., [set A, set B] RS set) (see Fig. 2.1.1.2-2). One or more CSI-ReportConfigs can be derived/configured by the network to the UE in step 5 based on applicable inference related parameter sets in step 4.

Fig. 2.1.1.2-1: Case 1: The inference related parameter set contains much richer IEs in/or the IEs referred by CSI-ReportConfig, such as RS configurations (i.e., [set A, set B] RS set) and associated ID.

Fig. 2.1.1.2-2: Case 2: The inference related parameter sets contain IEs in/or the IEs referred by CSI-ReportConfig and associated IDs, however, do not contain RS configurations (i.e., [set A, set B] RS sets).

Observation 5: For case 1 in observation 4 where the inference related information contains RS configuration (i.e., [set A, set B] RS set) and associated ID to help UE accurately determine inference related parameter sets applicability

- There may not be much saving in the signaling overhead, as inference related parameter sets need to contain much richer IEs in/or the IEs referred by CSI-ReportConfig.

Observation 6: For case 2 in observation 4 where the inference related information does not contain RS configurations (i.e., [set A, set B] RS set)

- The UE needs to know all relevant RS configurations (i.e., [set A, set B] RS set) for an inference related parameter set, otherwise UE will inaccurately declare the applicability of inference related parameter sets. FFS on how UE knows the relevant RS configurations (i.e., [set A, set B] RS set) for an inference related parameter set.

- This will result in the network configuring UE with CSI-ReportConfiguration in step 5 which may not be applicable.

=> would require vendor collaboration for a clean mapping.

### 2.1.2 Applicability signaling after step 5

Based on RAN2 agreements [5] – [8], the UE reports the applicability information in the following scenarios

- In response to the initial configuration,

- When an applicable function (i.e., inference related parameter set or CSI-ReportConfig) becomes non-applicable,

- When a functionality .e., inference related parameter set or CSI-ReportConfig) configured by the network to be reported via UAI, becomes from non-applicable to applicable.

Let us consider how the applicability needs to be evaluated at the UE in the following two scenarios after step 5

- The UE is configured with CSI-ReportConfig in step 3,

- The UE is configured with inference related parameter sets in step 3, and CSI-ReportConfig for inference in step 5.

Observation 7: After step 5, the UE keeps evaluating CSI-ReportConfig

- for applicability if A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) is configured at the UE in step 3 for determining applicability at the UE.

Observation 8: If B) One set or multiple sets of inference related parameters for applicability report only (not for inference) is configured in step 3 for applicability and CSI-ReportConfig for inference is provided in step 5, then UE needs to keep evaluating

- One set or multiple sets of inference related parameters for applicability report only (not for inference) configured in step 3, and

- CSI-ReportConfigs provided in step 5 for inference.

Proposal 1: Considering the UE complexities associated with determining applicability when UE is configured with inference related parameter sets in step 3

- RAN2 agrees that A) one or more CSI-reportConfig for inference configuration and associated IDs (wherein the associated ID may be configured in CSI framework as working assumption applied) are configured in step 3 for determining applicability.

### 2.1.3 Implicit vs Explicit Signaling

In RAN2#128 meeting [8], RAN2 agreed on the following

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability. verify this aligns with RAN1 configuration design.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

RAN2 agreed that the source cell UAI can be sent from the source cell to the target cell using existing signaling. This implies that the serving cell needs to store the applicability information reported in the UAI. As the most recent applicability information is available at the serving cell, the UE does not need to report the complete applicability information. To reduce the signaling overhead, the UE may report the change in applicability information.

Observation 9: The serving cell stores the applicability information reported in the UAI.

Proposal 2: The UE reports explicitly “applicable” and “non-applicable” functionality (e.g., CSI-ReportConfig ID or inference related parameter set IS) when there is a change of applicability.

## 2.2 Training Data Collection Configuration

In the RAN2#128 meeting [8], RAN2 agreed on the following

- For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains.

Note that the UE may have sufficient training data collection for one or more associated IDs and/or one or more RS configurations (i.e., [set A, set B]), while it may want to perform training data collection for other sets of one or more associated IDs and/or one or more RS configuration (i.e., [set A, set B]). Therefore, the training data collection configuration should include

- One or more associated IDs for which UE wants to request RS configuration, and/or

- Preferred list of RS configurations for one or more associated IDs.

Observation 10: In the RAN2#128 meeting, RAN2 agreed the following

- For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains.

Observation 11: The UE may have sufficient training data collection for one or more associated IDs and/or one or more RS configurations (i.e., [set A, set B]), while it may want to perform training data collection for other sets of one or more associated IDs and/or one or more RS configuration (i.e., [set A, set B]).

Proposal 3: The UE request for training data collection may include

- One or more associated IDs for which UE wants to request RS configuration, and

- Preferred list of RS configurations for one or more associated IDs.

In RAN2#128 [8], RAN2 further agreed on the following

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

A few companies in RAN2 argued that the network may be transmitting UE-specific RS for training data collection, therefore, after such indication the network may stop transmitting the RS. However, there is no such requirement from RAN1 that RS configuration can be UE-specific.

Observation 12: RAN1 has not agreed that the RS configuration can be UE-specific.

Proposal 4: UE does not need to indicate if the UE cannot perform training data collection (e.g., due to memory or power issues) if the RS configuration is provided without UE request.

Note that measurement configuration (RS configuration and associated ID) provided by the serving cell may not remain suitable due to UE mobility, e.g., UE has moved or changed orientation and UE may not obtain measurements on the RS configuration. In such a scenario, the UE should be allowed to send another training data collection configuration request or an indication that the training data collection configuration is not suitable.

Observation 13: Measurement configuration (RS configuration and associated ID) provided by the serving cell may not remain suitable due to UE mobility, e.g., UE has moved or changed orientation and UE may not obtain measurements on the RS configuration.

Proposal 5: The UE is allowed to send

- Another training data collection request, and/or

- An indication to indicate that training data collection configuration is not suitable.

## 2.3 Uniqueness of Associated IDs within A PLMN

In the RAN1#117 meeting [9], RAN1 had the following working assumption

gr

Working Assumption Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study).

Observation 14: RAN1 assumed that NW-side additional conditions with the same associated ID are consistent at least within a cell.

While consistency within a cell is useful for UE-side for model development and runtime LCM (e.g., model selection/switching/activation/deactivation), it is not scalable. If the associated ID at different cells represents different network-side additional conditions, then it will be hard for the UE side to generalize the model across multiple cells. This poses challenges not only on the UE-side model development but also on the runtime LCM. Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions. Therefore, the uniqueness of associated IDs extended to the PLMN. How the uniqueness of the associated IDs within a PLMN is achieved can be left up to the OAM implementation.

Observation 15: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 16: The associated ID's lack of uniqueness poses challenges not only to the UE-side model development but also to the runtime LCM.

Observation 17: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 6: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 7: The OAM implementation can decide how to achieve the uniqueness of the associated IDs within a PLMN.

# Conclusion

Observation 1: From the RAN1 LS reply [4], we can make the following observation

- In step 3, for determining the applicability, the NW can configure the UE with

  - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

  - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

- In step 5, the network can configure the UE with

  - CSI-ReportConfig for inference configuration, if the B) is used in step 3

  - Optionally, update CSI-ReportConfig for inference configuration, if the A) is used in step 3

Observation 2: To determine the applicability, the UE needs to check whether the UE has trained models for RS configurations (i.e., [set A, set B] RS sets) and associated IDs.

Observation 3: If the network configures A) One or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in the CSI framework as the working assumption applied), the UE can accurately determine the applicability because the CSI-ReportCOnfig for inference contains RS configurations (i.e., [set A, set B] RS sets) and associated IDs.

Observation 4: Considering inference related parameters for applicability report only (not for inference) is provided in step 3 and inference configuration (i.e., CSI-ReportConfig) is provided in step 5, there may exist following cases

- The inference related parameter set contains much richer IEs in/or the IEs referred by CSI-ReportConfig, such as RS configurations (i.e., [set A, set B] RS set) and associated ID (see Fig. 2.1.1.2-1). This helps the UE to accurately determine inference related parameter sets applicability, as the UE can check whether the UE has a trained model available for the configured RS configurations (i.e., [set A, set B] RS set) and associated ID. This also implies that a single CSI-ReportConfig for inference can be derived from the inference related parameter set.

- The inference related parameter sets contain IEs in/or the IEs referred by CSI-ReportConfig and associated IDs, however, do not contain RS configurations (i.e., [set A, set B] RS set) (see Fig. 2.1.1.2-2). One or more CSI-ReportConfigs can be derived/configured by the network to the UE in step 5 based on applicable inference related parameter sets in step 4.

Observation 5: For case 1 in observation 4 where the inference related information contains RS configuration (i.e., [set A, set B] RS set) and associated ID to help UE accurately determine inference related parameter sets applicability

- There may not be much saving in the signaling overhead, as inference related parameter sets need to contain much richer IEs in/or the IEs referred by CSI-ReportConfig.

Observation 6: For case 2 in observation 4 where the inference related information does not contain RS configurations (i.e., [set A, set B] RS set)

- The UE needs to know all relevant RS configurations (i.e., [set A, set B] RS set) for an inference related parameter set, otherwise UE will inaccurately declare the applicability of inference related parameter sets. FFS on how UE knows the relevant RS configurations (i.e., [set A, set B] RS set) for an inference related parameter set.

- This will result in the network configuring UE with CSI-ReportConfiguration in step 5 which may not be applicable.

=> would require vendor collaboration for a clean mapping.

Observation 7: After step 5, the UE keeps evaluating CSI-ReportConfig

- for applicability if A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) is configured at the UE in step 3 for determining applicability at the UE.

Observation 8: If B) One set or multiple sets of inference related parameters for applicability report only (not for inference) is configured in step 3 for applicability and CSI-ReportConfig for inference is provided in step 5, then UE needs to keep evaluating

- One set or multiple sets of inference related parameters for applicability report only (not for inference) configured in step 3, and

- CSI-ReportConfigs provided in step 5 for inference.

Proposal 1: Considering the UE complexities associated with determining applicability when UE is configured with inference related parameter sets in step 3

- RAN2 agrees that A) one or more CSI-reportConfig for inference configuration and associated IDs (wherein the associated ID may be configured in CSI framework as working assumption applied) are configured in step 3 for determining applicability.

Observation 9: The serving cell stores the applicability information reported in the UAI.

Proposal 2: The UE reports explicitly “applicable” and “non-applicable” functionality (e.g., CSI-ReportConfig ID or inference related parameter set IS) when there is a change of applicability.

Observation 10: In the RAN2#128 meeting, RAN2 agreed the following

- For data collection configuration UE-side model training, the UE can send a request for data collection. FFS what the request contains.

Observation 11: The UE may have sufficient training data collection for one or more associated IDs and/or one or more RS configurations (i.e., [set A, set B]), while it may want to perform training data collection for other sets of one or more associated IDs and/or one or more RS configuration (i.e., [set A, set B]).

Proposal 3: The UE request for training data collection may include

- One or more associated IDs for which UE wants to request RS configuration, and

- Preferred list of RS configurations for one or more associated IDs.

Observation 12: RAN1 has not agreed that the RS configuration can be UE-specific.

Proposal 4: UE does not need to indicate if the UE cannot perform training data collection (e.g., due to memory or power issues) if the RS configuration is provided without UE request.

Observation 13: Measurement configuration (RS configuration and associated ID) provided by the serving cell may not remain suitable due to UE mobility, e.g., UE has moved or changed orientation and UE may not obtain measurements on the RS configuration.

Proposal 5: The UE is allowed to send

- Another training data collection request, and/or

- An indication to indicate that training data collection configuration is not suitable.

Observation 15: The uniqueness of associated ID within a single cell is not scalable, as it will create challenges for the UE side to generalize models across multiple cells.

Observation 16: The associated ID's lack of uniqueness poses challenges not only to the UE-side model development but also to the runtime LCM.

Observation 17: Without sufficient generalization, the UE side has to download/get the model frequently causing unnecessary interruptions.

Proposal 6: NW-side additional conditions with the same associated ID are consistent within a PLMN.

Proposal 7: The OAM implementation can decide how to achieve the uniqueness of the associated IDs within a PLMN.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- R1-2410898, “Reply LS on applicable functionality reporting for beam management UE-sided model,” Nov 2024

- RAN2#127 meeting report

- RAN1#118 meeting report

- RAN2#127bis meeting report

- RAN2#128 meeting report

- RAN1#118bis meeting report