3GPP TSG-RAN WG2 Meeting #129		        R2-2500394

Athens, GR: 17th Feb – 21st Feb 2025

Source:	Qualcomm Incorporated

Title:	Discussion on Dataset and Parameter Sharing from the Network to the UE for Two-Sided Model Training

Agenda Item:	8.1.5 Model transfer/delivery

Document for:	Discussion and Decision

# Introduction & Background

This paper discusses the following

- Dataset (and/or parameters) transfer from the network to the UE for two-sided model training,

- Model delivery/transfer for inference, considering discussions during the SI phase and recent agreements in RAN1 as the baseline.

# Discussion

## 2.1   Dataset and Parameter Sharing from the Network to the UE for Two-Sided Model Training

In the RAN1LS R1-2410922 [1], RAN1 indicated that RAN2 that among the various Options and Sub-Options for Direction A, after down-selection, RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}

RAN1 further indicated RAN2 that the dataset and/or parameter sharing is per training collaboration instance, originating from a NW-side training entity and consumed at a UE-side training entity. RAN1 concluded that standardized signaling, if feasible and specified, can be used for parameter and/or dataset exchange to alleviate/resolve the inter-vendor training collaboration complexity. The standardized signaling solutions concluded in RAN1 were

- Over-the-air signaling, or

- Other approaches.

Observation 1: Based on RAN1 LS [1], the dataset and parameters can be shared from the network to the UE side using

- Over-the-air Signaling, or

- Other standard-based solutions for offline delivery/transfer.

During the study item [2], RAN2 made the following agreements for training of the two-sided CSI feedback use case models

o For the two-sided CSI compression use case, training data can be generated by either the UE or the gNB, depending on specific requirements, while the termination point for training data may include the gNB, OAM, Over-The-Top (OTT) server, or UE.  Note: RAN2 identified the case in which Core Network may be used for model training. However, no study was conducted since this is beyond the scope of this Working Group.

Note that the training of the UE-part model of two-sided models will be performed at the UE-side training entity, rather than at the UE. Therefore, the dataset and parameters need to be sent to the UE-side training entity, irrespective of whether the over-the-signaling or other approaches are used. The UE-side training entity may reside within a server in the MNO network or OTT server.

Proposal 1: Two-sided model training is performed between an infra-vendor and one or more UE vendors. Therefore, the network should ensure the required dataset and parameters are successfully shared with the participating UE vendors (the UE side entity) to train the UE part of the two-sided models.

Observation 2: Based on the RAN2 agreement on training the two-sided CSI compression use case [2], the network part of two-sided model training is performed at the gNB or OAM.

Observation 3: The UE-part model of two-sided models will be trained at the UE-side training entity rather than at the UE. Therefore, the dataset and parameters should be sent to the UE-side training entity, regardless of whether over-the-signaling or other approaches are used.

As the dataset and parameters are required to be shared with the UE-side entity (i.e., dataset and parameters need to be sent to an entity that is not UE), therefore, “over-the-air” signalling for dataset and parameter sharing involves

- The procedure to share the dataset and parameters from the network side to UE, and thereafter

- From the UE to the UE-side entity (data collection or training).

Proposal 2: The over-the-air signalling for dataset and parameter sharing from the network to the UE-side entity involves

- A procedure to share the dataset and parameters from the network side (e.g., serving cell/gNB or OAM) to UE, and thereafter

- A procedure to share the dataset and parameters from the UE to the UE-side entity (data collection or training).

We would also like to emphasize that training at the gNB or OAM does not imply that the dataset and parameters need to be shared from the gNB/OAM to the UE side entity. The TS 23.288 [3] outlines procedures for sharing

- The data from OAM to NWDAF (seen in Fig. 2.1-1), and

- Analytics from NWDAF to AF (see Fig. 2.1-2).

Fig. 2.1-1: Data collection from OAM to NWDAF (section 6.2.3.2 [3])

Fig. 2.1-2: Analytics sharing from NWDAF to AF (section 6.1.1.2 [3])

Observation 4: The network part of two-sided model training at the gNB or OAM does not imply that the dataset and parameters need to be shared from the gNB/OAM to the UE side entity. The dataset and parameter can be shared from the network side to the UE-side entity by 5GC or OAM. The TS 23.288 [3] outlines procedures for sharing

- The data from OAM to NWDAF (seen in Fig. 2.1-1), and

- Analytics from NWDAF to AF (see Fig. 2.1-2).

Proposal 3: The “other approaches” for dataset and parameter sharing from the network to the UE-side entity involves

- A procedure to share the dataset and parameters from the NW-side (e.g., 5GC or OAM) to the UE-side entity (data collection or training), without UE’s involvement.

### 2.1.1 Dataset and Parameters Sharing using Ove-the-Air Signaling

Fig. 2.1.1-1 illustrates the dataset and parameters sharing from the network to the UE over the air. In Fig. 2.1.1-1, the gNB sends the dataset and parameters to the UE, thereafter, the UE sends the dataset and parameters to the UE side entity (data collection or training entity/server), as the training cannot be performed at the UE due to the sheer size of the dataset and the required computation for training.

Fig. 2.1.1-1: Dataset and parameter sharing using over-the-air signaling

For further discussion on over-the-air signaling for the dataset and parameters sharing, we consider the following scenario

- The gNB shares the dataset and parameters with all UEs in the coverage,

- The OAM or 5GC selects UEs for sharing the dataset and parameters (for a UE vendor), or

- The gNB shares the dataset and parameters with the predetermined UEs (among UE vendors and infra-vendors/operators)

Let us discuss issues associated with the above methods for sharing the dataset and parameters one by one. However, before that let us make some baseline assumptions for the discussion as follows:

- The gNB has the temporary UE identity available, i.e., it cannot map the UE vendor identity from the temporary UE identity (at least not without communicating with the 5GC),

- The gNB may need to know the UE capability on whether it can forward the received dataset and parameters to the UE-side entity,

- Supporting and non-supporting gNBs: in a geographical area, gNBs may or may not support this feature (sharing the dataset and parameters over the air).

- In a geographical area, gNB/cells from different vendors can coexist. The UE needs to know infra-vendor identity (or alternatively, information such as pairing ID) to ensure that the dataset and parameters are from the same infra-vendor.

Observation 5: RAN2 should consider the following baseline for the discussion of whether the over-the-air signaling can be used for the dataset and parameter sharing

- User consent may be needed, as the shared dataset and parameters are significantly large,

- The gNB has the temporary UE identity available, i.e., it cannot map the UE vendor identity from the temporary UE identity (at least not without communicating with the 5GC),

- The gNB may need to know the UE capability on whether it can forward the received dataset and parameters to the UE-side entity,

- Supporting and non-supporting gNBs: in a geographical area, gNBs may or may not support this feature (sharing the dataset and parameters over the air),

- In a geographical area, gNB/cells from different vendors can coexist. The UE needs to know infra-vendor identity (or alternatively, information such as pairing ID) to ensure that the dataset and parameters are from the same infra-vendor.

Fig. 2.1.1-1: Dataset and parameters sharing in different mobility and deployment scenarios

#### 2.1.1.1	The gNB shares the dataset and parameters with all UEs in the coverage

As discussed earlier, consider a scenario where the gNB can share a small part of the data with each UE in the geographical area. Note that the gNB cannot derive the UE vendor identity based on the temporary UE identity (C-RNTI). Therefore, it cannot ensure the successful delivery of the dataset and parameters to any of the UE vendors. Furthermore, Xn/NG-AP enhancement is required for the continuity of sharing of the dataset and parameters if the UE performs a handover or RRC state transition (from RRC CONNECTED to IDLE/INACTIVE) or RLF happens. Otherwise, the gNB has to select a new UE, however, the gNB cannot ensure a UE from the same vendor is selected. The aforementioned issue arises also if UE moves to non-supporting gNB from supporting gNB (as for UE 1 in Fig. 2.1.1-1).

Observation 6: By sending a small amount of dataset and parameters to all UEs, the gNB cannot ensure the successful delivery of the dataset and parameter to the UE-side entity as

- The gNB has the temporary UE identity available, i.e., it cannot map the UE vendor identity from the temporary UE identity,

- The gNB can never ensure that the dataset and parameter are shared with a UE vendor, in the absence of a method to determine UE vendor identity,

- If a UE moves away from the sharing gNB, then gNB cannot ensure a UE from the same vendor is selected to share the remaining data,

- Without Xn/NG-AP enhancements, continuity of sharing of the dataset and parameters cannot be achieved.

#### 2.1.1.2 The OAM or 5GC selects UEs for sharing the dataset and parameters (for a UE vendor)

The OAM or 5GC may perform UE selection for the UE vendor based on the user's consent. However, this will cause undue load on selected UEs that would be selected to perform the dataset and parameters transfer from the UE to the UE-side entity. Other issues such as Xn/NG-AP enhancements, and selecting a new UE if the previously selected UE leaves are still needed. NG-RAN can assume the successful sharing of the dataset and parameter to the UE-side entity only if all portions/segments are successfully shared with the UEs of a vendor. This can be hard to achieve due to UE mobility and shorter RRC CONNECTED state duration.

Observation 7: When the OAM or 5GC selects UEs for sharing the dataset and parameters (for a UE vendor), the following challenges exist with over-the-air sharing of the dataset and parameter to the UE-side entity

- Undue load on one or more unlucky UEs that would be selected to perform dataset and parameter transfer from the UE to the UE-side entity

- NG-RAN can assume the successful sharing of the dataset and parameter to the UE-side entity only if all portions/segments are successfully shared with the UEs of a vendor. This can be hard to achieve due to UE mobility and shorter RRC CONNECTED state duration.

- If a UE moves away from the sharing gNB, then OAM or 5GC may need to select new UEs from the same vendor to share the remaining data. Alternatively, the other selected UEs may be forced to deliver the remaining data.

- Without Xn/NG-AP enhancements, continuity of sharing of the dataset and parameters cannot be achieved.

#### 2.1.1.3 The gNB shares the dataset and parameters with the predetermined UEs (among UE vendors and infra-vendors/operators)

Inter-vendor collaboration will be required to select the predetermined UEs (among UE vendors and infra-vendors/operators). Furthermore, the above-discussed issues will still apply to this scenario.

Observation 8: Sharing the dataset and parameters with the predetermined UEs (among UE vendors and infra-vendors/operators) of a UE vendor has the following challenges

- Inter-vendor collaboration will be required to select the predetermined UEs (among UE vendors and infra-vendors/operators),

- Undue load on one or more unlucky UEs that would be selected to perform dataset and parameter transfer from the UE to the UE-side entity

- NG-RAN can assume the successful sharing of the dataset and parameter to the UE-side entity only if all portions/segments are successfully shared with the UEs of a vendor. This can be hard to achieve due to UE mobility and shorter RRC CONNECTED state duration.

- If a UE moves away from the sharing gNB, then OAM or 5GC may need to select new UEs from the same vendor to share the remaining data. Alternatively, the other selected UEs may be forced to deliver the remaining data.

- Without Xn/NG-AP enhancements, continuity of sharing of the dataset and parameters cannot be achieved.

Apart from individual challenges associated with different scenarios of over-the-air signaling for the dataset and parameter sharing, the over-the-air signaling creates the Uu overhead twice: First for sharing the dataset and parameters from the RAN to the UE (in downlink), and the second for delivering/transferring the dataset and parameter from the UE to the UE side entity (in uplink).

Observation 9: The over-the-air signaling creates the Uu overhead twice: First for sharing the dataset and parameters from the RAN to the UE (in downlink), and the second for delivering/transferring the dataset and parameter from the UE to the UE side entity (in uplink).

Proposal 4: Over-the-air signaling for sharing the dataset and parameters to support UE-side model training is not a suitable and scalable solution.

### 2.1.2 Other Standard-based Solutions for Dataset and Parameter Sharing

In the SA2 WG, we have precedents where different types of data are shared with application functions (within or outside the 3GPP network) and/or with the edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Observation 10: Within the SA2 WG, we have precedents where different types of data are shared with application functions (within/outside the 3GPP network) and/or with the edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Note that for the UE-side data collection (i.e., data collection for the UE-side training), RAN2 and SA WGs are discussing the standard-based solution that involves UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server.

Observation 11: For the UE-side data collection (i.e., data collection for the UE-side training), RAN2 and SA WGs are discussing the standard-based solution that involves UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server.

Proposal 5: For sharing the dataset and parameters from the NW side (e.g., 5GC or OAM) to the UE side entity, 3GPP should be able to develop a solution similar to

- Other existing solutions for data exposure to AF from 5GC, or

- The solutions being discussed for the UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server, or

- A new solution (separate from being discussed for UE side data collection) for sharing dataset and parameters from 5GC or OAM to UE side entity.

SA WGs can further study this and provide feedback to RAN WGs.

Proposal 6: RAN2 checks with SA2/SA5 to provide feedback.

Proposal 7: Send LS Reply attached in ANNEX A to RAN1, SA2, and SA5.

## 2.2 Model Transfer/Delivery

### 2.2.1	Two-Sided Models

Observation 12: In RAN1 LS R1-2410922 [1], RAN1 mentioned that among the various Options and Sub-Options for Direction A, after down-selection, RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder,

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}.

In RAN1 LS R1-2410922 [1], RAN1 did not ask RAN2 to study the feasibility/solution for direction B. Therefore, RAN2 should wait for RAN1 to conclude that the model transfer/delivery solution from NW to UE is needed for two-sided models. In direction A, the model is trained at the UE-side training entity and can be transferred/delivered to the UE for inference. Therefore, a standardized solution for one-sided model transfer/delivery (from 5GC or server in MNO) can be reused for the two-sided model (if introduced).

Proposal 8: RAN2 should wait for RAN1 to conclude if direction B is supported.

# Conclusion

Dataset and parameter sharing from the network to the UE-side training entity

Observation 1: Based on RAN1 LS [1], the dataset and parameters can be shared from the network to the UE side using

- Over-the-air Signaling, or

- Other standard-based solutions for offline delivery/transfer.

Proposal 1: Two-sided model training is performed between an infra-vendor and one or more UE vendors. Therefore, the network should ensure the required dataset and parameters are successfully shared with the participating UE vendors (the UE side entity) to train the UE part of the two-sided models.

Observation 2: Based on the RAN2 agreement on training the two-sided CSI compression use case [2], the network part of two-sided model training is performed at the gNB or OAM.

Observation 3: The UE-part model of two-sided models will be trained at the UE-side training entity rather than at the UE. Therefore, the dataset and parameters should be sent to the UE-side training entity, regardless of whether over-the-signaling or other approaches are used.

Proposal 2: The over-the-air signalling for dataset and parameter sharing from the network to the UE-side entity involves

- A procedure to share the dataset and parameters from the network side (e.g., serving cell/gNB or OAM) to UE, and thereafter

- A procedure to share the dataset and parameters from the UE to the UE-side entity (data collection or training).

Observation 4: The network part of two-sided model training at the gNB or OAM does not imply that the dataset and parameters need to be shared from the gNB/OAM to the UE side entity. The dataset and parameter can be shared from the network side to the UE-side entity by 5GC or OAM. The TS 23.288 [3] outlines procedures for sharing

- The data from OAM to NWDAF (seen in Fig. 2.1-1), and

- Analytics from NWDAF to AF (see Fig. 2.1-2).

Proposal 3: The “other approaches” for dataset and parameter sharing from the network to the UE-side entity involves

- A procedure to share the dataset and parameters from the NW-side (e.g., 5GC or OAM) to the UE-side entity (data collection or training), without UE’s involvement.

Observation 5: RAN2 should consider the following baseline for the discussion of whether the over-the-air signaling can be used for the dataset and parameter sharing

- User consent may be needed, as the shared dataset and parameters are significantly large,

- The gNB has the temporary UE identity available, i.e., it cannot map the UE vendor identity from the temporary UE identity (at least not without communicating with the 5GC),

- The gNB may need to know the UE capability on whether it can forward the received dataset and parameters to the UE-side entity,

- Supporting and non-supporting gNBs: in a geographical area, gNBs may or may not support this feature (sharing the dataset and parameters over the air),

- In a geographical area, gNB/cells from different vendors can coexist. The UE needs to know infra-vendor identity (or alternatively, information such as pairing ID) to ensure that the dataset and parameters are from the same infra-vendor.

Observation 6: By sending a small amount of dataset and parameters to all UEs, the gNB cannot ensure the successful delivery of the dataset and parameter to the UE-side entity as

- The gNB has the temporary UE identity available, i.e., it cannot map the UE vendor identity from the temporary UE identity,

- The gNB can never ensure that the dataset and parameter are shared with a UE vendor, in the absence of a method to determine UE vendor identity,

- If a UE moves away from the sharing gNB, then gNB cannot ensure a UE from the same vendor is selected to share the remaining data,

- Without Xn/NG-AP enhancements, continuity of sharing of the dataset and parameters cannot be achieved.

Observation 7: When the OAM or 5GC selects UEs for sharing the dataset and parameters (for a UE vendor), the following challenges exist with over-the-air sharing of the dataset and parameter to the UE-side entity

- Undue load on one or more unlucky UEs that would be selected to perform dataset and parameter transfer from the UE to the UE-side entity

- NG-RAN can assume the successful sharing of the dataset and parameter to the UE-side entity only if all portions/segments are successfully shared with the UEs of a vendor. This can be hard to achieve due to UE mobility and shorter RRC CONNECTED state duration.

- If a UE moves away from the sharing gNB, then OAM or 5GC may need to select new UEs from the same vendor to share the remaining data. Alternatively, the other selected UEs may be forced to deliver the remaining data.

- Without Xn/NG-AP enhancements, continuity of sharing of the dataset and parameters cannot be achieved.

Observation 8: Sharing the dataset and parameters with the predetermined UEs (among UE vendors and infra-vendors/operators) of a UE vendor has the following challenges

- Inter-vendor collaboration will be required to select the predetermined UEs (among UE vendors and infra-vendors/operators),

- Undue load on one or more unlucky UEs that would be selected to perform dataset and parameter transfer from the UE to the UE-side entity

- NG-RAN can assume the successful sharing of the dataset and parameter to the UE-side entity only if all portions/segments are successfully shared with the UEs of a vendor. This can be hard to achieve due to UE mobility and shorter RRC CONNECTED state duration.

- If a UE moves away from the sharing gNB, then OAM or 5GC may need to select new UEs from the same vendor to share the remaining data. Alternatively, the other selected UEs may be forced to deliver the remaining data.

- Without Xn/NG-AP enhancements, continuity of sharing of the dataset and parameters cannot be achieved.

Observation 9: The over-the-air signaling creates the Uu overhead twice: First for sharing the dataset and parameters from the RAN to the UE (in downlink), and the second for delivering/transferring the dataset and parameter from the UE to the UE side entity (in uplink).

Proposal 4: Over-the-air signaling for sharing the dataset and parameters to support UE-side model training is not a suitable and scalable solution.

Observation 10: Within the SA2 WG, we have precedents where different types of data are shared with application functions (within/outside the 3GPP network) and/or with the edge cloud, for example,

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

Observation 11: For the UE-side data collection (i.e., data collection for the UE-side training), RAN2 and SA WGs are discussing the standard-based solution that involves UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server.

Proposal 5: For sharing the dataset and parameters from the NW side (e.g., 5GC or OAM) to the UE side entity, 3GPP should be able to develop a solution similar to

- Other existing solutions for data exposure to AF from 5GC, or

- The solutions being discussed for the UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server, or

- A new solution (separate from being discussed for UE side data collection) for sharing dataset and parameters from 5GC or OAM to UE side entity.

SA WGs can further study this and provide feedback to RAN WGs.

Proposal 6: RAN2 checks with SA2/SA5 to provide feedback.

Proposal 7: Send LS Reply attached in ANNEX A to RAN1, SA2, and SA5.

Model transfer/delivery

Observation 12: In RAN1 LS R1-2410922 [1], RAN1 mentioned that among the various Options and Sub-Options for Direction A, after down-selection, RAN1 is considering the following Sub-Options for further study with potential further down-selection.

- Option 4-1: sharing dataset containing N1 samples of {target CSI, CSI feedback} corresponding to the input and output of a nominal encoder, respectively

- Option 3a-1 without target CSI: sharing parameters corresponding to a nominal encoder,

- Option 3a-1 with target CSI: sharing parameters corresponding to a nominal encoder, along with dataset containing N2 samples of {target CSI}.

Proposal 8: RAN2 should wait for RAN1 to conclude if direction B is supported.

# Reference

- R1-2410922, “LS on signalling feasibility of dataset and parameter sharing,” Nov 2024

- TS 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) 
for NR air interface”

- TS 23.288, “Architecture enhancements for 5G System (5GS) to support network data analytics services”

- TS 28.105, “Artificial Intelligence / Machine Learning (AI/ML) management,” Dec 2024

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

- RAN1#116 meeting report

- RAN1#116bis meeting report

# ANNEX A

3GPP TSG-RAN WG2 Meeting #129		     R2-25xxxxx

Athens, GR: 17th Feb – 21st Feb 2025

# Title:	[Draft] Reply LS on Signalling feasibility of data set and parameter sharing

# Response to:	LS on signalling feasibility of dataset and parameter sharing (R1-2410922/R2-2500015)

# Release:	Release 19

# Work Item:	FS_NR_AIML_air

Source:	RAN2

To:	RAN1, SA2, SA5

Cc:	SA3

Contact Person:

Name:	Rajeev Kumar

E-mail Address:	rkum@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

# Overall Description

RAN2 thanks RAN1 for their LS regarding the Rel-19 two-sided AI/ML CSI compression study for the AI/ML Air Interface in R2-2500015.

RAN2 understands that the over-the-air signalling for dataset and parameter sharing from the network to the UE-side entity involves

- A procedure to share the dataset and parameters from the network side (e.g., serving cell/gNB or OAM) to UE, and thereafter

- A procedure to share the dataset and parameters from the UE to the UE-side entity (data collection or training).

RAN2 further understands that the “other approaches” for dataset and parameter sharing from the network to the UE-side entity involves

- A procedure to share the dataset and parameters from the NW-side (e.g., 5GC or OAM) to the UE-side entity (data collection or training), without UE’s involvement.

RAN2 has concluded that over-the-air signaling for sharing the dataset and parameters from the network side to the UE-side entity is not a suitable or scalable solution.

RAN2 believes that within the SA2 WG, we have precedents where different types of data are shared with application functions (within/outside the 3GPP network) and/or with the edge cloud, a few examples of such data sharing are as follows:

- Target UE location to AF [TS 23.273],

- Analytics exposure from NWDAF to AF, e.g., network status reporting [TS 23.288],

- Traffic volume, UL/DL data rates, PDU session status, and others [TS 23.501].

RAN2 would further like to bring to RAN1's attention that for the UE-side data collection (i.e., data collection for the UE-side training), RAN2 and SA WGs are discussing the standard-based solution that involves UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server.

RAN2 believe that for sharing the dataset and parameters from the NW side (e.g., 5GC or OAM) to the UE side entity, 3GPP should be able to develop a solution similar to

- Other existing solutions for data exposure to AF from 5GC, or

- The solutions being discussed for the UE data delivery from the 5GC or OAM to the server for data collection for UE-side model training / OTT server, or

- A new solution (separate from being discussed for UE side data collection) for sharing dataset and parameters from 5GC or OAM to UE side entity.

SA WGs can further study this and provide feedback to RAN WGs.

# Actions

To: RAN1

ACTION: RAN2 confirms the feasibility of a solutions to share the dataset and parameters from the network (e.g., from 5GC or OAM) to the UE side entity.

To: SA2, SA5

ACTION: RAN2 requests SA2 and SA5 to provide feedback.

# Dates of next TSG-WG2

RAN1#130bis	from 2025-04-07	to 2025-04-11		China

RAN1#131	from 2025-05-19	to 2025-05-2		Malta