3GPP TSG-RAN WG2 #129	R2-2500396

Athens, Greece, 17 – 21 February 2025

Agenda item:		8.5.3	On-demand SIB1

Title:	Discussion on On-demand SIB1 WUS provisioning and UE behaviour

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

During RAN2 #127bis, the following agreements were made:

OD-SIB1 Validity: 7. A cell for which SIB1 request configuration is available, can periodically broadcast SIB1. 8. If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

During RAN2 #128, the following agreement was made:

NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

During RAN #106, it was agreed that:

RAN confirms that Case 2 also includes the case where a particular Cell A can switch to become a NES Cell.

In this contribution, we discuss further xxx

# 2. Discussion

2.1	UE behaviour before UL WUS request transmission

During RAN2 #127bis, it was agreed that UE should check if SIB1 is being broadcast before it transmits WUS request.
There are two scenarios when this could happen:

- Scenario 1: the NES cell is broadcasting SIB1 periodically

  - SIB1 may be regularly broadcast by NES cell with the legacy periodicity (e.g. 160ms)

- Scenario 2: the NES cell is transmitting SIB1 only temporarily

  - It may be triggered by the on demand SIB1 request sent to the NES cell from other UE(s).

  - It may be triggered by a regular SIB1 update within the NES cell.

In Scenario 1, the UE will be aware of the broadcast SIB1 from the MIB, so the default behaviour should be to ignore the UL WUS configuration and listen to the broadcast SIB1, as agreed in RAN2 #127-bis.

Observation 1: If SIB1 is being broadcast periodically and no SIB1 indication in the MIB is not set, UEs will not request OD-SIB1.

In Scenario 2, if SIB1 is transmitted only temporarily, it is not clear what condition there is or how long a UE should wait until sending its own WUS request.

We see two options:

- Option 1: The network can temporarily indicate SIB1 transmission using MIB.

- Option 2: UE follows the CORESET0 to monitor SIB1 transmission for a specific duration before requesting OD-SIB1.

In Option 1, the situation is similar to Scenario 1. A new indication might be required to avoid misinterpreting the announced SIB1 broadcast as being periodic, although previous agreements indicate that a legacy UE having a valid SIB1 shall not bar the NES Cell. Then another option is that the broadcast SIB1 itself will then bar legacy UEs.

While this has RAN2 impacts, this is ultimately up to RAN1 to define such signalling, if deemed necessary.

Proposal 1: RAN2 to discuss whether an OD-SIB1 NES Cell can indicate that it will broadcast (OD-)SIB1 temporarily, depending on RAN1 progress.

In Option 2, we assume that the UE tries to receive the SIB1 for a specific duration. RAN2 should discuss the need to define a duration (e.g. configurable) for which the UE monitors SIB1 before attempting UL WUS for OD-SIB1. If the duration is too short, many UEs may unnecessarily trigger UL WUS request. If the duration is too long, there could be a problem where every UE waits for another UE to trigger UL WUS request.

Proposal 2: RAN2 to discuss whether SIB1 monitoring duration before a UE may attempt OD-SIB1 request needs to be specified.

  - Special “Case 2” where Cell A becomes an NES cell

During RAN #106, it was agreed to include the special scenario where a Cell A can provide UL-WUS configuration for itself before becoming a NES cell.

There are two scenarios:

- Scenario 1: No other cell A, only future NES cell is broadcasting its UL WUS config

  - Then the NW needs to configure at least one new Cell A

- Scenario 2: There was already another Cell A2

  - NES cell may indicate to the other Cell A that is switching to NES mode, however

  - May not be necessary since other Cell A can broadcast WUS config anyway and UEs check if SIB1 is being broadcast before doing OD-SIB1

In any scenario, configuring a Cell A is up to Xn/RAN3 and will be possible even in a more typical “Case 2” scenario. On top of that, a neighbouring Cell A may provide UL WUS configuration even if the NES cell is not in NES mode at the time. Indeed, according to RAN2 #128 agreement, UEs will check if SIB1 if being broadcast before requesting OD-SIB1, or potentially even UL WUS configuration.

Observation 2: A UE provided with an UL WUS configuration will still check whether SIB1 is being periodically broadcast.

Proposal 3: From RAN2 perspective, the case where the Cell A signals its own UL WUS configuration has no specification impact.

2.3	UE request of UL WUS configuration at non Cell A neighbour cell

It was agreed during RAN2 #126 that “more than one Cell A may provide configuration for the same NES cell”, as exemplified in the figure below:

Figure 1- Two CellAs

Last meeting, it was further agreed that an NES cell can include neighbouring NES cell’s WUS configuration. This means that potentially any neighbour cell could be a Cell A, and that this is not limited to a limited number of coverage cells (potentially just one) for which the NES cell acts as e.g. a capacity cell.

Figure 2- Many CellAs including NES cells

Then, it stands to reason that not all neighbouring cells will act as a Cell A, especially the other NES cells within a coverage cell area.

Observation 3: Not all coverage cells overlapped with the NES cell are required to be Cell As.

Furthermore, during RAN2 #126, it was also agreed that “the same Cell A may assist more than one NES Cells”. By default, we can assume that a coverage cell would provide UL WUS configuration for all OD-SIB1 NES cells under its coverage. However, there could be a few scenarios where this may not be the case:

- Scenario 1: There may be multiple layers of coverage cells and the middle layer may be the one providing the UL WUS configuration (see figure 3) or it would be the role of the upper layer cell (see figure 4).

- Scenario 2: The OD-SIB1 NES Cell may be partially outside of the coverage area of a coverage cell but within coverage distance of UEs camping on the coverage cell, what we may call “extended coverage area”, then, the UE may require the coverage cell to provide UL WUS configuration (see figure 5).

- Scenario 3: Assuming that there is at least one Cell A associated with the OD-SIB1 NES Cell, a coverage cell may only provide UL WUS configuration based on past NW activity, i.e. whether UEs often request configuration for this OD-SIB1 NES Cell. If demand is not high enough, the NW may choose not to use resources for this purpose.

| Figure 3- UE gets configuration from the macro cell | Figure 4- UE gets configuration from the middle coverage cell |
| --- | --- |

Figure 5- OD-SIB1 cell is outside of Cell A coverage

Observation 4: An OD-SIB1 NES Cell may not be fully under its cell A’s coverage.

Observation 5: A coverage cell may not provide UL WUS configuration for all the OD-SIB1 NE Cells under its coverage.

In Scenario 1, the NW may achieve signalling load balancing by not having all coverage cells provide UL WUS configuration. However, if for instance only the upper layer coverage cell provides the UL WUS configuration of the NES Cell, then a UE connected to the middle layer coverage cell may still request UL WUS configuration without needing to go to the upper layer coverage cell.

In Scenario 2, the coverage cell A2 may cover a part of this OD-SIB1 NES Cell, for which Cell A1 already provides UL WUS configuration.

In Scenario 3, typically in a greenfield deployment, UE request would allow the NW to learn its UE traffic patterns and requirements instead of automatically using a lot of signalling resources to provide UL WUS configuration.

The above scenarios are not exhaustive. Rather, they aim to show possibilities to achieve the intended NES. Other scenarios may be possible, and it will be up to NW implementation to decide which one to implement.

In summary, the flexibility to choose how many or which neighbouring cell can provide UL WUS configuration can provide many NES opportunities. Involving UEs may be further beneficial.

Proposal 4: RAN2 to discuss various scenarios for the configuration of Cell As for a given OD-SIB1 NES Cell. FFS if enhancements are required to support these scenarios.

# 3. Conclusion

In this contribution we discussed support of on-demand SIB1 for UEs in Idle/Inactive and made the following observations and proposals:

Observation 1: If SIB1 is being broadcast periodically and no SIB1 indication in the MIB is not set, UEs will not request OD-SIB1.

Proposal 1: RAN2 to discuss whether an OD-SIB1 NES Cell can indicate that it will broadcast (OD-)SIB1 temporarily, depending on RAN1 progress.

Proposal 2: RAN2 to discuss whether SIB1 monitoring duration before a UE may attempt OD-SIB1 request needs to be specified.

Observation 2: A UE provided with an UL WUS configuration will still check whether SIB1 is being periodically broadcast.

Proposal 3: From RAN2 perspective, the case where the Cell A signals its own UL WUS configuration has no specification impact.

Observation 4: An OD-SIB1 NES Cell may not be fully under its cell A’s coverage.

Observation 5: A coverage cell may not provide UL WUS configuration for all the OD-SIB1 NE Cells under its coverage.

Proposal 4: RAN2 to discuss various scenarios for the configuration of Cell As for a given OD-SIB1 NES Cell. FFS if enhancements are required to support these scenarios.