3GPP TSG-RAN WG2 #129	R2-2500397

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.5.4 Adaptation of common signal channel transmissions

Source:	NEC

Title:	PRACH and paging adaptation for NES

Document for:	Discussion, Decision

# Introduction

This paper intends to discuss RAN2 impact of adaptation of common signal/channel transmissions, which is an objective within Rel-19 NES WID led by RAN1.

While PRACH adaptation has not been discussed yet, some agreements have been reached in RAN2 #127-bis for paging adaptation, including:

3. Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

In this contribution, we discuss the latest agreements in RAN1 #118bis and how they impact RAN2 specification work going forward.

# Discussion

## Agreements during previous RAN1 meetings on PRACH adaptation in time domain

At RAN1#116 meeting, the following agreements were reached for this subtopic regarding PRACH adaptation and paging adaptation:

Agreement For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study - Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any) - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs - For the additional PRACH resources, - Adaptation of PRACH resource periodicity/PRACH occasion - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle - Adaptation based on extending cell DRX operation for PRACH - Concentrating ROs in time domain Other options are not precluded

Agreement For the adaptation mechanisms of PRACH in time-domain - Support at least PRACH adaptation provided by gNB without UE trigger - FFS: PRACH adaptation with UE trigger - Note: UE trigger means UE requests adaptation of PRACH - Study at least the following, - Dynamic signaling and/or semi-static signaling of PRACH adaptation - Adaptation of PRACH transmission according to certain condition - Applicability to idle/inactive and/or connected mode UEs Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

At RAN1 #117 meeting, the following was agreed:

For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed Above does not preclude discussion for the case where the configuration for additional PRACH resources

- contains legacy PRACH resources

Observation 1: From RAN1 point of view, only time AND frequency overlap, i.e. sharing, of PRACH resources for NES-capable and legacy UEs has been discarded.

Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Observation 2: From RAN1 point of view, there should be no SSB-RO mapping ambiguity for additional PRACH resources in the case where there is no overlap, either in time or frequency, which is expected legacy behaviour.

## PRACH adaptation from RAN2 point of view

For the adaptation of PRACH, the time-domain based approach was discussed by RAN1. Specifically, the adaptation can be based on the configuration of additional or different PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs.

Traditionally, the UE knows the PRACH time-domain resources (periodicity, sub-frame, slot and OFDM symbol it used) through the PRACH configuration index (i.e., prach-ConfigurationIndex, or by msgA-PRACH-ConfigurationIndex) that is transmitted by SIB1/RRC messages to the UE.

Hence, the first issue for RAN2 is to clarify how the gNB configure such additional or different PRACH resources for NES-capable UEs. To discuss this issue, an assumption should be made if the UE can read the SIB1 message from the serving cell. In the case where SIB1 is subject to on-demand request as discussed in Rel-19 NES WID, the UE may be not able to read this additional or different PRACH resources.

Proposal 1: RAN2 to discuss the configuration of additional or different PRACH resources for NES-capable UEs.

In legacy NR systems, the UE will use RA-RNTI to identify itself during RACH procedure. RA-RNTI is used by UE to receive Random Access Response (RAR) once PRACH is transmitted. Different RACH occasions within a system frame are associated with different RA-RNTI values to ensure that RAR for a given PRACH transmission can be properly addressed to only the UE which has used that given RACH occasion. If RA-RNTI values are same for 2 RACH occasions, then there is a chance of confusion where 2 UEs which have used different RACH occasions act on the same RAR message. To ensure this uniqueness, RA-RNTI is computed as:

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

When additional RACH resources are configured to UE, there may be a chance that RA-RNTI confusion may occur as one legacy RACH resource and an additional RACH resource have get the same RA-RNTI value.

Indeed, RAN1 has not ruled out the possibility of sharing PRACH resources in both time and frequency “FFS: Additional case whether full/partial overlap in both time and frequency is allowed.”

Proposal 2: RAN2 to discuss the possibility of the occurrence of RA-RNTI confusion with additional RACH resources.

Another topic is whether Rel-19 NES-capable UEs should be restricted to their new resources or could also access legacy resources.

For paging adaptation, RAN2 has agreed that PF/POs could be shared between legacy and new UEs.

In the context of PRACH adaptation, the scenario is to have a base long periodicity PRACH for new UEs joining, while dynamically adapting PRACH occasions following, e.g., a high paging load.

Observation 3: PRACH adaptation may be linked to dynamic incoming paging, rather than more stable initial access.

Then, depending on the amount of Rel19+ NES-capable UEs, the additional PRACH resources allocated for an upcoming “signalling storm”, which may be indicated as part of paging indication, should not impact legacy PRACH directed to new joining UEs.

Proposal 3: RAN2 to discuss sharing or separation of additional PRACH resources.

## Adaptation of PRACH for mobility

It is not clear whether additional PRACH would be used sparsely to fill the gaps in more abundant legacy resources or, on the contrary, legacy PRACH resources will be very sparse and additional PRACH resources will be used to fill the gaps whenever needed, which could be often.

In the context of NES, we believe that the main scenario is that few UEs are camping on the cell and that additional PRACH resources would be available dynamically depending on potential traffic requirements.

For an NES cell with little to no traffic, this is not the main scenario. However, in the context of mobility, especially when a neighbour cell will stop serving and wishes to offload its UEs, then allocating additional PRACH resources would be beneficial.

Proposal 4: RAN2 to discuss adaptation of PRACH resources in the context of mobility.

## Remaining paging adaptation issues

Some progress was made during RAN2 #127-bis regarding paging enhancements. Namely, it was agreed to extend Ns value to 8 and 16 (i.e. no need to introduce large Ns vale like 32 or 64) while N value can be extended with 32, 64, 128, and 256.

Further UE behaviour may be impacted due to Cell DTX/DRX. It must be noted that this may not always happen as these are two optional features that are not required to be used in conjunction. In this section, we discuss the scenario where both Rel-18 Cell DTX/DRX and Rel-19 paging adaptation are used together.

During cell DRX inactive, less paging occasions may be configured by prolonging the default DRX cycle, e.g. using smaller N or Ns values.

As shown in Figure 1, paging occasion falls within cell DTX inactive period may be skipped. Rel-19 NES capable UE may only monitor paging occasions in cell DTX active time if cell DTX is activated and common channel adaptation for DL is enabled. This may require UE to be provisioned with cell DTX pattern during idle/inactive mode.

Figure 1 - Paging occasion skipped during cell DTX inactive

Proposal 5: Paging occasions within cell DTX inactive period can be skipped.

Release 17 specifies that the UE in Idle and Inactive mode may receive a Paging Early Indication (PEI). PEI which can indicate to the UE whether to decode paging in its PO. For paging occasions skipping during cell DTX inactive duration, PEI may also be enhanced to indicate if a PO will be skipped during the next cell DTX non-active period and if paging information will be received during the next PO within cell DTX active period. Idle or inactive mode UEs will not need to wake up for PEI/PO monitoring when paging is not sent to them during cell DTX inactive duration.

As shown in Figure 2, PEI may be used to indicate the paging reception for the next cell DTX active duration. If the PEI indicates that the UE is to be paged, the UE will wake up at the next paging occasion within the upcoming cell DTX active period. If the PEI indicates ‘No Paging’, UE would assume that there is no paging information (and no associated SSB/TRS) for the next paging occasion within the cell DTX active period.

Figure 2 - PEI indicating ‘paged’ for next cell DTX active

PEI indication may also indicate that the next PO within cell DTX non-active period will ‘NOT’ be skipped e.g. to be within a paging latency tolerance; or due to low SINR. If not present, the next PO within cell DTX non-active period will be skipped.

If a PEI occasion coincides with cell DTX inactive duration, and there is no paging information, PEI may not need to be transmitted. If PEI is not received within cell DTX inactive duration, UE assumes that there is no PO (and no associated SSB/TRS) for this cell DTX inactive duration. If PEI is not received for a cell DTX active duration, UE may continue to monitor every PO within the cell DTX active duration.

Proposal 7: PEI indication may indicate if a PO can be skipped for the next cell DTX non-active period and if paging information will be received at the next PO within cell DTX active period.

# Conclusion and Proposal

We have the following observations and proposals:

Observation 1: From RAN1 point of view, only time AND frequency overlap, i.e. sharing, of PRACH resources for NES-capable and legacy UEs has been discarded.

Observation 2: From RAN1 point of view, there should be no SSB-RO mapping ambiguity for additional PRACH resources in the case where there is no overlap, either in time or frequency, which is expected legacy behaviour.

Proposal 1: RAN2 to discuss the configuration of additional or different PRACH resources for NES-capable UEs.

Proposal 2: RAN2 to discuss the possibility of the occurrence of RA-RNTI confusion with additional RACH resources.

Observation 3: PRACH adaptation may be linked to dynamic incoming paging, rather than more stable initial access.

Proposal 3: RAN2 to discuss sharing or separation of additional PRACH resources.

Proposal 4: RAN2 to discuss adaptation of PRACH resources in the context of mobility.

Proposal 5: Paging occasions within cell DTX inactive period can be skipped.

# Reference

- RAN1#116 meeting minutes

- TS38.321

- TS38.304