3GPP TSG-RAN WG2 Meeting #129	R2-2500407

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.2.2.

Source:	InterDigital Inc.

Title:	Generalization of AIML models for RRM measurement prediction

Document for:	Discussion

# 1. Introduction

In RAN2-128 [1], several agreements were made regarding model generalization, some of which are listed below:

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases,

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

- Generalization issues on RRM measurement prediction are prioritized.

- Start the study with generalization issue with RRM measurement prediction in temporal domain.

- Study generalization over UE speeds

- The simulation assumption of FR1 temporal domain case B is reused for generalization study with 3 UE speeds i.e. 30Km/h, 60Km/h and 90Km/h.

- The simulation assumption of FR2 temporal domain case A is reused for generalization study with 3 UE speeds i.e. 60Km/h, 90Km/h and 120Km/h.

An email discussion was carried out after RAN2-128 [2] that covered other parameters for model generalization, apart from speed and frequency which was agreed during the meeting.

In this contribution, we will provide initial results on model generalization for different UE speeds, for the RRM measurement prediction use case.

# 2. Discussion

### 2.1 Intra-cell intra-frequency FR1 to FR1 temporal domain case B

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 1: Non-sliding L1/L3 Filtering

We consider 50% measurement reduction rate as shown in Figure 2. For predicting cell-level RSRPs at time instance T, we use input from 3 previous time instances, which are T-5, T-3, and T-1. It is necessary to use input from multiple time instances to enable the AI/ML model to learn information about UE trajectory. The RSRPs at time instances T-5, T-3, and T-1 are all measured RSRPs.

In Case 2, we consider predicting L3 cell-level RSRPs directly according to Sub-case 2 where:

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs from 3 time instances (T-5, T-3, and T-1) are input to AI/ML model to predict L3 cell-level RSRP at time instance T directly.

Figure 2: Temporal-domain Case B Observation Windows (OWs in yellow) and Prediction Windows (PW in green) Pattern with 50% Measurement Reduction Rate

To study the capability of the AI/ML model to generalize to different test cases (i.e., different UE speeds = 30, 60, 90 Km/h), we consider the following generalization cases:

- Baseline: The AI/ML model is trained using the dataset for UE Speed #A and tested using the dataset for UE Speed #A.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset for UE Speed #B and tested using the dataset for UE Speed #A.

- Generalization Case #2 (GC#2): The AI/ML model is trained using a mixed dataset with examples for multiple UE speeds and tested using the dataset for UE Speed #A.

  - GC#2-A: The AI/ML model is trained using a mixed dataset with the same size as the dataset for UE Speed #A for fair comparison.

  - GC#2-B: The AI/ML model is trained using a mixed dataset with larger size than the dataset for UE Speed #A (by incorporating all examples from the dataset for UE Speed #A and the dataset for UE Speed #B) as a fair advantage for the mixed AI/ML model. In our study, the size of the mixed dataset for GC#2-B is 3 times the size of the dataset for UE Speed #A.

Simulation parameters of Case 2 are listed in Table A.1 [3]-[5]. Evaluation and AI/ML model parameters are shown in Table 2. Generalization results are shown in Table 3.

Table 2: Evaluation and AI/ML model parameters of intra-cell intra-frequency FR1 to FR1 temporal domain case B Generalization

| Parameters | Value |
| --- | --- |
| UE trajectory option [2] | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 |
| UE speed | 30, 60, 90 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A |
| Measurement reduction rate | 50% |
| Observation window | 1000 ms |
| Prediction window | 200 ms |
| Any other parameters | N/A |
| Number of training examples | ~14.7 k |
| Number of validation examples | ~2.1 k |
| Number of testing examples | ~4.2 k |
| AI/ML model input | L3 cell-level RSRPs from 3 time instances T-5, T-3, T-1 |
| AI/ML model output | L3 cell-level RSRPs at 1 time instance T |
| AI/ML model type | CNN |
| AI/ML model number of parameters | 0.104 M |
| AI/ML model number of FLOPs | 0.128 M |

Table 3: Evaluation results of intra-cell intra-frequency FR1 to FR1 temporal domain case B generalization

| Inference using the dataset for UE Speed = 30 Km/h |  |  |  |  |
| --- | --- | --- | --- | --- |
| Generalization Case | Trained with examples from dataset for UE Speed = 30 Km/h | Trained with examples from dataset for UE Speed = 60 Km/h | Trained with examples from dataset for UE Speed = 90 Km/h | AIML-based Average L3 cell-level RSRP Difference (dB) |
| Baseline | ü |  |  | 0.0176 |
| GC#1 |  | ü |  | 0.0192 |
| GC#1 |  |  | ü | 0.0203 |
| GC#2-A | ü | ü | ü | 0.0188 |
| GC#2-B | ü | ü | ü | 0.0180 |
| Inference using the dataset for UE Speed = 60 Km/h |  |  |  |  |
| Generalization Case | Trained with examples from dataset for UE Speed = 30 Km/h | Trained with examples from dataset for UE Speed = 60 Km/h | Trained with examples from dataset for UE Speed = 90 Km/h | AIML-based Average L3 cell-level RSRP Difference (dB) |
| Baseline |  | ü |  | 0.0543 |
| GC#1 | ü |  |  | 0.0566 |
| GC#1 |  |  | ü | 0.0535 |
| GC#2-A | ü | ü | ü | 0.0538 |
| GC#2-B | ü | ü | ü | 0.0533 |
| Inference using the dataset for UE Speed = 90 Km/h |  |  |  |  |
| Generalization Case | Trained with examples from dataset for UE Speed = 30 Km/h | Trained with examples from dataset for UE Speed = 60 Km/h | Trained with examples from dataset for UE Speed = 90 Km/h | AIML-based average L3 cell-level RSRP Difference (dB) |
| Baseline |  |  | ü | 0.0928 |
| GC#1 | ü |  |  | 0.1062 |
| GC#1 |  | ü |  | 0.0967 |
| GC#2-A | ü | ü | ü | 0.0946 |
| GC#2-B | ü | ü | ü | 0.0945 |

For a fixed UE inference speed, the AI/ML models trained with the same UE speed have slightly better accuracy than the AI/ML models trained with different UE speeds (i.e., baseline vs. GC#1 cases). Moreover, the accuracy decreases as the difference between the UE speed used during training and the speed used for inference increases. Nevertheless, the worsening in accuracy is minor.

The slight worsening in the accuracy of GC#1 cases can be negated by training at multiple UE speeds. For example, all GC#2-A cases lead to slightly better accuracy compared to the respective GC#1 cases. They also achieve comparable accuracy compared to the respective baseline case. Using additional training data, i.e., as in GC#2-B, offers negligible improvements in the accuracy of the AI/ML models.

Observation 1: Regarding GC#1, there is a minor deterioration in the accuracy of the AI/ML model compared to the baseline case.

Observation 2: Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case. Further increasing the amount of data within the mixed dataset (GC#2) offers minor improvement.

Proposal 1: Companies should report the size of the mixed dataset (GC#2) with respect to the size of the baseline dataset

### 2.2 Intra-cell intra-frequency FR2 to FR2 temporal domain case A

To generate data, we consider a sampling period of 20 ms for L1 beam-level RSRPs. To measure RSRPs for 4 Rx beams, a measurement period of 80 ms (i.e., 4 sampling periods) is required. Then, sliding L1 filtering (with window length that is equal to 5) is applied to obtain L1 filtered beam-level RSRPs., the measurement period of sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 80 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 3: Sliding L1/L3 Filtering

In Case 4, we consider predicting L3 cell-level RSRPs directly according to Sub-case 2, where:

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs from OW time instances are input to AI/ML model to predict L3 cell-level RSRP of PW time instances directly.

Figure 4: Temporal-domain Case A Observation Window (OW) and Prediction Window (PW) Pattern

Simulation parameters of Case 4 are listed in Table A.2 [3]-[5]. Evaluation and AI/ML model parameters are shown in Table 4. Generalization results are shown in Table 5.

Table 4: Evaluation and AI/ML model parameters of intra-cell intra-frequency FR2 to FR2 temporal domain case A Generalization

| Parameters | Value |
| --- | --- |
| UE trajectory option [2] | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 |
| UE speed | 60, 90, 120 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A |
| Measurement reduction rate | N/A |
| Observation window | 400 ms |
| Prediction window | 400 ms |
| Any other parameters | N/A |
| Number of training examples | ~14 k |
| Number of validation examples | ~2 k |
| Number of testing examples | ~4 k |
| AI/ML model input | L3 Cell-level RSRPs of OW time instances |
| AI/ML model output | L3 Cell-level RSRPs of PW time instances |
| AI/ML model type | DNN |
| AI/ML model number of parameters | 0.271 M |
| AI/ML model number of FLOPs | 0.270 M |

Table 5: Evaluation results of intra-cell intra-frequency FR1 to FR1 temporal domain case B generalization

| Inference using the dataset for UE Speed = 60 Km/h |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Generalization Case | Trained with examples from dataset for UE Speed = 60 Km/h |  | Trained with examples from dataset for UE Speed = 90 Km/h | Trained with examples from dataset for UE Speed = 120 Km/h |  | AIML-based Average L3 cell-level RSRP Difference for all PW Instances (dB) |
| Baseline | ü |  |  |  |  | [0.0573, 0.1651, 0.3073, 0.4715, 0.6490] |
| GC#1 |  |  | ü |  |  | [0.0603, 0.1766, 0.3283, 0.5020, 0.6885] |
| GC#1 |  |  |  | ü |  | [0.0693, 0.1914, 0.3552, 0.5456, 0.7501] |
| GC#2-A | ü |  | ü | ü |  | [0.0633, 0.1738, 0.3242, 0.4974, 0.6836] |
| GC#2-B | ü |  | ü | ü |  | [0.0561, 0.1642, 0.3081, 0.4728, 0.6510] |
| Inference using the dataset for UE Speed = 90 Km/h |  |  |  |  |  |  |
| Generalization Case | Trained with examples from dataset for UE Speed = 60 Km/h |  | Trained with examples from dataset for UE Speed = 90 Km/h | Trained with examples from dataset for UE Speed = 120 Km/h |  | AIML-based Average L3 cell-level RSRP Difference for all PW Instances (dB) |
| Baseline |  |  | ü |  |  | [0.1078, 0.2992, 0.5317, 0.7822, 1.0364] |
| GC#1 | ü |  |  |  |  | [0.1104, 0.3070, 0.5583, 0.8363, 1.1234] |
| GC#1 |  |  |  | ü |  | [0.1123, 0.3053, 0.5474, 0.8048, 1.0589] |
| GC#2-A | ü |  | ü | ü |  | [0.1079, 0.2929, 0.5215, 0.7719, 1.0279] |
| GC#2-B | ü |  | ü | ü |  | [0.1010, 0.2801, 0.5000, 0.7389, 0.9806] |
| Inference using the dataset for UE Speed = 120 Km/h |  |  |  |  |  |  |
| Generalization Case | Trained with examples from dataset for UE Speed = 60 Km/h | Trained with examples from dataset for UE Speed = 90 Km/h |  | Trained with examples from dataset for UE Speed = 120 Km/h | AIML-based Average L3 cell-level RSRP Difference for all PW Instances (dB) |  |
| Baseline |  |  |  | ü | [0.1611, 0.4112, 0.6996, 1.0100, 1.3368] |  |
| GC#1 | ü |  |  |  | [0.1798, 0.4757, 0.8436, 1.2430, 1.6453] |  |
| GC#1 |  | ü |  |  | [0.1672, 0.4383, 0.7548, 1.0843, 1.4257] |  |
| GC#2-A | ü | ü |  | ü | [0.1646, 0.4231, 0.7244, 1.0458, 1.3808] |  |
| GC#2-B | ü | ü |  | ü | [0.1557, 0.4054, 0.7015, 1.0134, 1.3389] |  |

The generalization accuracy of the AI/ML models in the baseline case is higher than the accuracy of the respective GC#1 case. However, when mixed datasets are used, the accuracy reaches and, in many cases, surpasses the accuracy of the baseline case, as multiple UE speeds are utilized during training. For example, for all 3 inference UE speeds, the accuracy of the GC#2-A cases is comparable to the one of the respective baseline cases. However, a model trained with such a mixed dataset can be used for inference in all 3 UE speeds, whereas 3 different baseline models would have to be trained, one for each UE speed, for equivalent accuracy. This increases complexity and triples the amount of the required training data.

Tripling the amount of training data in the mixed dataset (case GC#2-B) improves the accuracy compared to the GC#2-A case, while offering slightly better accuracy compared to the respective baseline AI/ML model (that uses one third of the training data). For the same amount of training data, a single mixed AI/ML model would offer comparable, and at times better, accuracy than the combination of 3 baseline models, while offering increased flexibility.

Also, in GC#1, for inference at UE speed = 90 Km/h, we notice that an AI/ML model trained at a higher UE speed (e.g., UE speed = 120 Km/h) generalizes better (i.e., GC#1 has a closer performance to the baseline) than an AI/ML model trained at a lower UE speed (e.g., UE speed = 60 Km/h).

In addition, in GC#1, for inference at UE speed = 60 Km/h, an AI/ML trained at a closer UE speed (i.e., UE speed = 90 Km/h) achieves a closer performance to the baseline than an AI/ML trained at a further UE speed (i.e., UE speed = 120 Km/h). A similar observation can be made about the inference at UE speed = 120 Km/h.

Furthermore, in GC#1, Using an AI/ML trained at UE speed = 120 Km/h for inference at UE speed = 60 Km/h achieves a closer performance to baseline than the opposite case, i.e., using an AI/ML trained at UE speed = 60 Km/h for inference at UE speed = 120 Km/h.

Observation 3: An AI/ML model trained with a mixed dataset (multiple UE speeds) of the same size as an AI/ML model for a dedicated UE speed (GC#2 vs. baseline case) offers comparable accuracy.

Observation 4: An AI/ML model trained with a mixed dataset with multiple UE speeds (GC#2) outperforms using a dedicated AI/ML model for inference on a different UE speed than the one it was trained for (GC#1).

Observation 5: Increasing the size of the mixed dataset (GC#2 case) improves the accuracy of the AI/ML model.

Observation 6: An AI/ML model trained with the mixed dataset in GC#2 has an accuracy equivalent to having multiple baseline models dedicated for specific UE speeds, while offering increased flexibility.

Observation 7: For GC#1, using an AI/ML model trained at a higher UE speed for inference at a lower UE speed achieves a closer performance to the baseline than using an AI/ML model trained at a lower UE speed for inference at a higher UE speed (e.g., training at 120 to infer on 90 is better than training at 60 to infer on 90, training at 120 to infer on 60 is better than training at 60 to infer on 120, etc.)

Observation 8: For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline.

# 3. Conclusion

In this contribution, initial simulation results for the RRM measurement event prediction use case were discussed and the following observations and proposals were made:

Observation 1: Regarding GC#1, there is a minor deterioration in the accuracy of the AI/ML model compared to the baseline case.

Observation 2: Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case. Further increasing the amount of data within the mixed dataset (GC#2) offers minor improvement.

Observation 3: An AI/ML model trained with a mixed dataset (multiple UE speeds) of the same size as an AI/ML model for a dedicated UE speed (GC#2 vs. baseline case) offers comparable accuracy.

Observation 4: An AI/ML model trained with a mixed dataset with multiple UE speeds (GC#2) outperforms using a dedicated AI/ML model for inference on a different UE speed than the one it was trained for (GC#1).

Observation 5: Increasing the size of the mixed dataset (GC#2 case) improves the accuracy of the AI/ML model.

Observation 6: An AI/ML model trained with the mixed dataset in GC#2 has an accuracy equivalent to having multiple baseline models dedicated for specific UE speeds, while offering increased flexibility.

Observation 7: For GC#1, using an AI/ML model trained at a higher UE speed for inference at a lower UE speed achieves a closer performance to the baseline than using an AI/ML model trained at a lower UE speed for inference at a higher UE speed (e.g., training at 120 to infer on 90 is better than training at 60 to infer on 90, training at 120 to infer on 60 is better than training at 60 to infer on 120, etc.)

Observation 8: For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline.

Proposal 1: Companies should report the size of the mixed dataset (GC#2) with respect to the size of the baseline dataset

# 4. References

[1]	RAN2-128 Chairman Notes, Orlando, USA, November 2024

[2]	R2-2500559: Email discussion summary for [Post128][018][AI Mob] generalization (Apple)

[3]	R2-2404955: Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)

[4]	R2-2406095: Summary of [POST126][031][AIMob] Simulations

[5]	R2-2497849: Summary of [POST127][030][AIMob] RRM Simulation Assumptions

[6]	TR 38.901: Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)

# Appendix

### A.1 Simulation assumptions of Intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [6] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h, 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [6] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |

### A.2 Simulation assumptions of Intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 – 30 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 200 m |
| Channel model | According to TR 38.901 [6] |
| System BW | 80 MHz |
| Sub-carrier Spacing | 120 kHz |
| UE Speed | 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (4,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | 2 panels (left, right) |
| BS Tx Power | 40 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [6] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 16 Tx Beams (8 Azimuth angles x 2 Elevation angles) 4 Rx Beams (4 Azimuth angles x 1 Elevation angle) |
| L1-Filtering: Method | Sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 3 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |