3GPP TSG-RAN WG2 Meeting #129	R2-2500408

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.3

Source:	InterDigital Inc.

Title:	Measurement event predictions

Document for:	Discussion

# 1. Introduction

In RAN2-128, several agreements were made regarding the simulation assumption for the RRM measurement event predictions [1]:

- Agree to definitions of indirect measurement event prediction for temporal domain case A and for temporal domain case B

- The input of model for RRM measurement prediction can be reused as baseline for corresponding direct measurement event prediction. Additional input(s) is also allowed.

- For intra-frequency temporal domain case B indirect measurement event prediction is taken as baseline. Direct prediction is optional.

- To agree on definitions for true event prediction, false event detection and missed event detection for indirect measurement event prediction

In this contribution, we provide initial results for indirect A3 measurement event prediction temporal domain case A and for temporal domain case B

# 2. Discussion

### 2.1 Intra-cell intra-frequency FR1 to FR1 temporal domain case B

To generate data, we consider a sampling period of 40 ms for L1 beam-level RSRPs. Then, non-sliding L1 filtering is applied to obtain L1 filtered beam-level RSRPs. With L1 filtering window length that is equal to 5, the sampling period of non-sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 200 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 1: Non-sliding L1/L3 Filtering

We study 50% measurement reduction rate. For predicting cell-level RSRPs at time instance T, we use input from 3 previous time instances, which are T-5, T-3, and T-1. It is necessary to use input from multiple time instances to enable the AI/ML model to learn information about UE trajectory. The RSRPs at time instances T-5, T-3, and T-1 are all measured RSRPs.

Figure 2: Temporal-domain Case B Observation Windows (OWs in yellow) and Prediction Windows (PW in green) Pattern with 50% Measurement Reduction Rate

In Case 2, we consider predicting L3 cell-level RSRPs of multiple cells directly according to Sub-case 2 where:

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs of all 21 cells from 3 time instances (T-5, T-3, and T-1) are input to AI/ML model to predict L3 cell-level RSRPs of all 21 cells at time instance T directly.

After predicting L3 cell-level RSRPs of all cells, we indirectly derive if A3 event occurred in predicted output and compare it with the ground-truth.

Simulation parameters of Case 2 are listed in Table A.1 [2]-[4]. Evaluation results are shown in Table 1.

Table 1: Evaluation results of indirect A3 event prediction for FR1 case B for Sub-case 2

| Parameters | Sub-case 2 (Cell-to-cell Direct) |
| --- | --- |
| UE trajectory option [2] | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 |
| UE speed | 90 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A |
| Measurement reduction rate | 50% |
| Observation window | 1000 ms |
| Prediction window | 200 ms (non-sliding) |
| A3 event offset (dB) | 2 |
| TTT (ms) | 200 |
| Max ETD (ms) | 80 |
| Number of training examples | 14630 |
| Number of validation examples | 2090 |
| Number of testing examples | 4180 |
| AI/ML model input | L3 cell-level RSRPs of all 21 cells from 3 time instances T-5, T-3, T-1 |
| AI/ML model output | L3 cell-level RSRPs of all 21 cells at 1 time instance T |
| AI/ML model type | CNN |
| AI/ML model number of parameters | 0.098 M |
| AI/ML model number of FLOPs | 0.269 M |
| AIML-based average L3 cell-level RSRP difference (dB) | 0.0367 dB |
| Precision | 0.9857 |
| Recall | 0.9173 |
| F1-Score | 0.9503 |
| Percentage of A3 events in test set | ~ 9% |

We notice that indirect A3 event prediction for temporal-domain case A achieves a precision, recall, and F1-score greater than 91%. Since recall is lower than precision, this indicates that there are more detection errors due to missed events than due to false alarms.

Observation 1: Indirect A3 event prediction for temporal-domain case A achieves a precision, recall, and F1-scores all greater than 91%.

### 2.2 Intra-cell intra-frequency FR2 to FR2 temporal domain case A

To generate data, we consider a sampling period of 20 ms for L1 beam-level RSRPs. To measure RSRPs for 4 Rx beams, a measurement period of 80 ms (i.e., 4 sampling periods) is required. Then, sliding L1 filtering (with window length that is equal to 5) is applied to obtain L1 filtered beam-level RSRPs., the measurement period of sliding L1 filtered beam-level RSRPs, and consequently L3 cell-level RSRPs is 80 ms. It is worth noting that L1 filtering is a simple averaging process of the input L1 beam-level RSRPs.

Figure 3: Sliding L1/L3 Filtering

In Case 4, we consider predicting L3 cell-level RSRPs directly according to Sub-case 2, where:

- Sub-case 2 (Cell-to-cell Direct Approach): L3 cell-level RSRPs from OW time instances are input to AI/ML model to predict L3 cell-level RSRP of PW time instances directly.

Figure 4: Temporal-domain Case A Observation Window (OW) and Prediction Window (PW) Pattern

After predicting L3 cell-level RSRPs of all cells, we indirectly derive if A3 event occurred in predicted output and compare it with the ground-truth.

Simulation parameters of Case 4 are listed in Table A.2 [2]-[4]. Evaluation results are shown in Table 2.

Table 2: Evaluation results of indirect A3 event prediction for FR2 case A for Sub-case 2

| Parameters | Sub-case 2 (Cell-to-cell Direct) |
| --- | --- |
| UE trajectory option [2] | Option 1 |
| UE trajectory boundary processing option [2] | Option 3 |
| UE speed | 90 Km/h |
| Inter-frequency correlation assumption (yes or no) | N/A |
| Measurement reduction rate | N/A |
| Observation window | 400 ms |
| Prediction window | 400 ms |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| Max ETD (ms) | 80 |
| Number of training examples | 14344 |
| Number of validation examples | 2049 |
| Number of testing examples | 4098 |
| AI/ML model input | L3 cell-level RSRPs of all 21 cells from OW time instances |
| AI/ML model output | L3 cell-level RSRPs of all 21 cells from PW time instances |
| AI/ML model type | CNN |
| AI/ML model number of parameters | 0.070 M |
| AI/ML model number of FLOPs | 1.457 M |
| AIML-based average L3 cell-level RSRP difference (dB) | [0.022, 0.065, 0.126, 0.202, 0.292] dB |
| Precision | 0.9555 |
| Recall | 0.9388 |
| F1-Score | 0.9471 |
| Percentage of A3 events in test set | ~ 8% |

We notice that indirect A3 event prediction for temporal-domain case B achieves a precision, recall, and F1-score greater than 93%. Since recall is lower than precision, this indicates that there are more detection errors due to missed events than due to false alarms.

Observation 2: Indirect A3 event prediction for temporal-domain case B achieves a precision, recall, and F1-scores all greater than 93%.

We also notice that the representation of A3 event in datasets may impact the training and the results of the AI/ML model. Hence, it is proposed that the percentage of A3 events in the test set should be reported.

Proposal 1: Companies are encouraged to report the percentage of A3 events in the test set as (n3+n2)/number of test set examples.

# 3. Conclusion

In this contribution, initial simulation results for the RRM measurement event prediction use case were discussed and the following observations and proposals were made:

Observation 1: Indirect A3 event prediction for temporal-domain case A achieves a precision, recall, and F1-scores all greater than 91%.

Observation 2: Indirect A3 event prediction for temporal-domain case B achieves a precision, recall, and F -scores all greater than 93%.

Proposal 1: Companies are encouraged to report the percentage of A3 events in the test set as (n3+n2)/number of test set examples.

# 4. References

[1]	RAN2-128 Chairman Notes, Orlando, USA, November 2024

[2]	R2-2404955: Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology (OPPO)

[3]	R2-2406095: Summary of [POST126][031][AIMob] Simulations

[4]	R2-2497849: Summary of [POST127][030][AIMob] RRM Simulation Assumptions

[5]	TR 38.901: Study on channel model for frequencies from 0.5 to 100 GHz (3GPP TR 38.901 version 16.1.0 Release 16)

# Appendix

### A.1 Simulation assumptions of Intra-cell intra-frequency FR1 to FR1 temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 – 4 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 500 m |
| Channel model | According to TR 38.901 [5] |
| System BW | 20 MHz |
| Sub-carrier Spacing | 30 kHz |
| UE Speed | 30 Km/h, 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44 dBm |
| UE receiver Noise Figure | 9 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [5] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 4 Tx Beams (4 Azimuth angles x 1 Elevation angle) 1 Rx Beam (1 Azimuth angle x 1 Elevation angle) |
| L1-Filtering: Method | Non-sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 1 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |

### A.2 Simulation assumptions of Intra-cell intra-frequency FR2 to FR2 temporal domain case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 – 30 GHz |
| Deployment | UMa, 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Inter site distance | 200 m |
| Channel model | According to TR 38.901 [5] |
| System BW | 80 MHz |
| Sub-carrier Spacing | 120 kHz |
| UE Speed | 60 Km/h |
| UE distribution | 100% outdoor (in vehicle) |
| BS Antenna Configuration | (8,4,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern |
| UE Antenna Configuration | (4,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| UE Antenna radiation pattern | 2 panels (left, right) |
| BS Tx Power | 40 dBm |
| UE receiver Noise Figure | 10 dB |
| BS Antenna height | 25 m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR 38.901 [5] |
| UE trajectory model | Option 1 (Linear trajectory model with random direction change) |
| LOS Update | LOS Soft |
| Number of Beams | 16 Tx Beams (8 Azimuth angles x 2 Elevation angles) 4 Rx Beams (4 Azimuth angles x 1 Elevation angle) |
| L1-Filtering: Method | Sliding Averaging |
| L1-Filtering: Filter Window | 5 |
| Beam Consolidation: nrofSS-BlocksToAverage | 3 |
| Beam Consolidation: absThreshSS-BlocksConsolidation | -110 dBm |
| L3-Filtering: Filter Coefficient K | 4 |