# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500425
Athens, Greece, February 17-22, 2025

Agenda item:	8.2.2

Source:	Apple

Title:	Discussion on Ambient IoT Paging

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In the WID [1] for Rel-19 Ambient IoT work, RAN2 objectives include the following:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this paper, we discuss the following issues regarding the normative work for Ambient IoT paging:

      - Paging message format

      - Concurrent paging transactions and transaction ID design

      - Paging message including a command

It is worth noting that the Rel-19 Ambient-IoT WI has a reduced scope and a shortened timeline, so we prefer the design choices which has less specification work for the consideration of timely completion of the WI.

# 2 Discussion

### 2.1 Paging message format(s)

For A-IoT paging message design, the WID has two following requirements:

A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

First, for the paging message format, we think it shall be folded into the generic MAC PDU format in a way that to be consistent with the generic MAC PDU format, which has been discussed in another Apple contribution [1]. To be more specific, one “message type” for R2D MAC PDU should be reserved for “paging”, as the paging message will have some unique formats, different from other messages, such as Msg 2, or “data-only” MAC PDU.

Proposal 1: 	One dedicated “Message Type” is used to indicate “Paging” in DL MAC PDU format.

Second, regarding “Support the options that a paging message contains one identifier, and that a paging message contains no identifier” part, we think this “one identifier” format actually contains two cases:

- Case A: ID of a single A-IoT device

- Case B: (Group) ID of a group of A-IoT device.

Which corresponding to the highlighted green parts of the following agreement in RAN2:

RAN2 will study the following cases for AIoT paging message: · a message containing an ID of a single A-IoT device. · a message containing a group ID that maps to multiple A-IoT devices. · a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. · a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

However, so far, SA2 only has a detailed design of “permanent device ID” structure for a single device, assigned either by NW operator or third-party. As all paging identifiers are supposed to be from upper layer, we believe the exact format of this group identifier should also be provided by SA2.

Proposal 2: 	RAN2 confirms that “one identifier” requirement in the paging message includes both the case of “one device identifier” and “one group identifier”, while the exact format of latter is supposed to be designed by SA2.

Then, regarding the “multiple ID” part, as indicated by WID, the case for paging message “containing multiple IDs of A-IoT devices” is a case aimed for forward-compatibility. This implies that the “multiple ID” paging message could be a Rel-20 feature, so that Rel-20 A-IoT device shall be able to decode the paging message containing multiple IDs, which is sent by a Rel-20 reader. However, the exact forward-compatibility requirement is unclear. There are several different interpretations of this:

Option 1: There is only one paging message format. Rel-19 reader can send paging message with multiple IDs, while the Rel-19 device will only decode the very first single ID and ignore the other IDs, if present. Thus, there is no paging message format change needed in Rel-20. This means the Rel-19 reader is already forward-compatible with Rel-20 device.

Option 2: There are two different paging message formats, one for Rel-19 and one for Rel-20. While the two paging message formats shares a certain similarity, Rel-19 device cannot be paged by a Rel-20 paging message. Instead, it may recognize the existence of a Rel-20 paging message and ignores/drops it.

For the above two different understandings, the message format design will be dramatically different. For Option 1, Rel-19 paging message format would need to have an explicit or implicit indication of the number of ordered IDs, so that the Rel-19 device can process “a single ID” successfully. For Option 2, we only need to add a version field or reserve a spare message type for Rel-20 paging message.

Proposal 3: 	RAN2 to clarify the meaning of “forward-compatibility”: whether there will be a single common Paging message for both Rel-19 and Rel-20 or two different paging message formats in the respective release.

According to RAN2 agreement in SI: ”An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.“, it is reasonable to include a transaction ID in the paging message. Note that the transaction ID is used to avoid duplicate response(s), which will only happen for the group paging case. This is because the NW will initiate another paging round to collect the response of device(s) which has missed the prior round, but will not want to receive the response for the device(s) which have already responded in the prior round.

Logically, if the paging is for a single device, there is no need for such an ID because the NW/reader will not initiate another round of paging if the device’s inventory response has been received successfully. On the other hand, even if NW decides to page the same device again, the paging message for a single device will be associated with a dedicated D2R resource for device to respond. As a result, even if the device skipping a “duplicate” response, the resource will be wasted anyway, and it does not benefit any other device(s). Theoretically, this targeted device can skip the “transaction ID” check if the paging ID exactly matches its own permanent ID.

Observation 1  Logically, “transaction ID” is not needed for the paging of “single A-IOT device” case.

However, in regards of message format design for extremely low-cost, low-complexity communication device, we recognize the advantage to have a simple and fixed MAC message format instead of allowing the absence/optional fields. Although transaction ID is unnecessary for the case when the Paging ID is only for a single device, there is no harm to always have a transaction ID present so that the receiving A-IoT device can derive the corresponding message fields in a fixed manner.

Proposal 4: 	A “transaction ID” is always included in the paging message no matter how many devices are paged/triggered.

Based on the same design principle, RAN2 should avoid the “absence” of “paging ID” field even in the case that “no ID” is provided by upper layer. Similar to the Ethernet MAC address design of using an “all one” broadcast address, we think it is reasonable to reserve an “all-one” address for such a case, so that the device can decode this “Paging ID” field’s meaning as “addressed for all devices” by assuming there is always a “Paging ID” field in the Paging message. If RAN2 agrees this approach, then we need inform this decision to SA2, so that the same “all-one” address shall be avoided to represent a single permanent device ID.

Proposal 5: 	In paging message, an “all-one” paging identifier is used by AS layer to represent the case of addressed for all devices.

Proposal 6: 	If Proposal 5 is agreed, RAN2 inform the “all-one” broadcast address decision to SA2.

### 2.2 Paging Transactions and Concurrency

One consequence of the allowing multiple paging rounds is that the paging transaction associated with a CN request would take a much longer time to complete, compared to a single-shot transmission of paging message in PRDCH channel. Therefore, whether the A-IoT reader can be involved in multiple overlapping CN-triggered paging procedures need to be discussed. If a single reader conducts multiple paging transactions at the same time, then an A-IoT device may be potentially paged by the same reader multiple times with different “paging records” (e.g. with different paging identifiers), either in the same R2D transmission or in multiple subsequent transmissions. Then, a couple of complicated questions needs to be answered. For example, whether the device need to respond to each of those separate paging transactions with different transaction IDs? And if not, how to select/prioritize the paging message to respond. It is worth noting that solving those issues would add complexity to the device side. So, RAN2 should have some restrictions on CN/NW side to not cause such a scenario. For instance, the reader may not initiate a new paging procedure associated to a new CN request until the previous inventory procedure is completed.

Proposal 7	An A-IoT reader should not initiate paging for a new CN request which overlaps with the paging procedure triggered by a prior CN request.

With this restriction on NW behavior, the device will not need to remember multiple transaction IDs at the same time, as a reader will only initiate a single paging transaction at one time. However, regarding the exact size of transaction ID, we need to consider some practical device behaviors. First, once a device has remembered a “transaction ID”, it will no longer respond to the paging message with the same “transaction ID” unless it forgets this ID. Based on RAN1 conclusion, non-volatile memory is not suitable for dynamic “write” operation. So, we think the A-IoT device will only store “transaction ID” in its volatile memory.

Observation 2  Received “transaction ID” is stored in the “volatile” memory of the A-IoT device.

It is also unreasonable to assume the Type 1 A-IoT device could run a timer to conduct a timer-based erasure of memory, so the only two possible reasons for a transaction ID to be forgotten are:

- Case 1: The device has lost power completely (and all of its volatile memory)

- Case 2: The device’s transaction ID storage has been overwritten by a new different “transaction ID”.

If neither of the above case happens, then the device has a risk to ignore a “new” paging transaction as “outdated” and skip responding. In A-IoT system, there is a chance that neither of the above condition occurs. For example, the device responds to a paging message, records its transaction ID, moves out-of-coverage (thereby missing the ensuing transactions from the same reader), then moves back in the prior reader coverage and detects the appearance of  the same “transaction ID” while this is a completely new transaction. In this case, it is wrong for device to not to respond.  Thus, if the transaction ID is just 1-bit or 2-bit, such a risk of error is really high. So, we suggest to have a reasonably long transaction ID, such as 6-bit or 8-bit to mitigate this risk.

Proposal 8	At least 6-bit “Transaction ID” is used for Paging message.

It is worth noting that the paging concurrency is not completely eliminated by restricting the A-IoT reader to only one transaction at a time. This is because when CN generates a paging request for inventory, it may send the same CN request to multiple readers because it does not know the exact location(s) of the A-IoT devices. Then, it is quite probable that the device may be paged by multiple different readers, simultaneously (e.g., w/o TDM coordination among the readers). Thus, some sort of concurrent paging for a device located in the overlapped area of multiple readers would occur. For this case, we need to have SA2/RAN3 decision to whether categorize them as the same transaction or different transactions. If the latter, then some division of “transaction ID” space among neighboring readers need to be sort out by RAN3 and/or SA2. Whether the device should respond to every request from different readers can be further discussed based on this decision. Then, RAN2 can revisit this issue to adjust transaction ID size, if needed.

Proposal 9	For the A-IoT paging messages initiated by different readers, RAN2 send an LS to SA2/RAN3 to ask whether this shall be regarded as the same transaction or different transactions.

### 2.3 Paging Message Containing Command

In SI phase, RAN2 has made an agreement for the so-called “command only” case for allowing a paging (initial triggering) message to contain a command:

RAN2#126 agreements: Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

Then, SA2 has studied this issue with the following “tentative” conclusion with an EN:

Editor’s Note: If the “command-only” case applies, pending SA3, if a command and paging can be performed in a single operation, then an AIoT specific NAS message may be included in the request.

Basically, SA2 has no firm conclusion on whether this is to be supported and rely on SA3 input on this issue. According to SA3 discussion on the procedure involving both inventory and command,

For operator managed AIoT devices, the following principles are taken as the conclusion for KI#5: Inventory-only Procedure: - Authentication (i.e., at least one-way authentication; the 5G network authenticates the AIoT device) shall be supported. NOTE 1: Details of the authentication procedure (e.g., one-way , parameter(s) used, etc.) are to be resolved during normative phase. Inventory and Command Procedure: - Mutual authentication between the AIoT device and the 5G network shall be supported. - The authentication procedure is based on a challenge-response mechanism. NOTE 2: Details of the mutual authentication procedure and details of the challenge-response mechanism(e.g., parameter(s) used for the 5G network to authenticate the AIoT device and for the AIoT device to authenticate the network) are to be resolved during normative phase.

As SA3 has indicated that for anything involving a “command”, “mutual authentication between AIOT device and 5G network” shall be supported with a method of “challenge-response”.

Given that, we think the device will be required to follow-up with some mutual message exchange with the NW & reader to authenticate each other. Then, there is not too much advantage to support the “command-only” use case.

Figure 1: Comparison of “mutual authentication” signaling flow

(a) Command + Inventory vs.(b) Command only

As shown in Figure 1, for the “command only” case (as illustrated in (b) of Figure 1), although A-IoT Function and reader does not need to send the command in a separate message, it does not save too much in the overall signalling overhead in the above diagram. Only one message is reduced when compared with (a) and (b). Anyway, the A-IoT device is not supposed to execute the command unless it has verified the NW.

Observation 3	 A-IoT device will not execute “command” unless it authenticates the NW.

Also, as there are at least one more DL R2D messages involved in this signalling diagram besides the paging message itself, the “Command” can be delivered to a discovered device later without causing any extra signalling overhead in paging. So, there is no obvious benefit to deliver the command earlier to the device as device will not execute this command only after it authenticates the NW.

Nonetheless, given that paging is a system-wide broadcast initiated by multiple readers, the paging including a command may actually waste radio resources by transmitting “command” information in areas whether there is no device to respond. Moreover, supporting this case will add complexity in device side as it requires the device to support another variant form of R2D Paging message. This is against the design principle for the low-cost Ambient-IoT device, especially for device type 1. The device needs to understand a new variant of message type. Also, it needs to store the received “command” in a buffer for a later processing.

Observation 4	 Adding the support of “paging messaging containing a command” adds complexity in device side.

Given the above considerations, we think RAN2 should not support this in Rel-19 work.

Proposal 10: 	Not Support “command only (paging containing a command)” in Rel-19\

# 3 Conclusion

In this contribution, we discuss the paging issue for Ambient IoT, and have the following observations:

Observation 1  Logically, “transaction ID” is not needed for the paging of “single A-IOT device” case.

Observation 2  Received “transaction ID” is stored in the “volatile” memory of the A-IoT device.

Observation 3	 A-IoT device will not execute “command” unless it authenticates the NW.

Observation 4  Adding the support of “paging messaging containing a command” adds complexity in device side.

Then, we have the following proposals:

Proposal 1 	One dedicated “Message Type” is used to indicate “Paging” in DL MAC PDU format.

Proposal 2 	RAN2 confirms that “one identifier” requirement in the paging message includes both the case of “one device identifier” and “one group identifier”, while the exact format of latter is supposed to be designed by SA2.

Proposal 3 	RAN2 to clarify the meaning of “forward-compatibility”: whether there will be a single common Paging message for both Rel-19 and Rel-20 or two different paging message formats in the respective release.

Proposal 4	A “transaction ID” is always included in the paging message no matter how many devices are paged/triggered.

Proposal 5	In paging message, an “all-one” paging identifier is used by AS layer to represent the case of addressed for all devices.

Proposal 6 	If Proposal 5 is agreed, RAN2 inform the “all-one” broadcast address decision to SA2.

Proposal 7	An A-IoT reader should not initiate paging for a new CN request which overlaps with the paging procedure triggered by a prior CN request.

Proposal 8	At least 6-bit “Transaction ID” is used for Paging message.

Proposal 9	For the A-IoT paging messages initiated by different readers, RAN2 send an LS to SA2/RAN3 to ask whether this shall be regarded as the same transaction or different transactions.

Proposal 10 	Not Support “command only (paging containing a command)” in Rel-19.

# 4 References

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR