# GPP TSG RAN WG2 Meeting #129      	                           R2-2500426
Athens, Greece, February 17-22, 2025

Agenda item:	8.2.3

Source:	Apple

Title:	Discussion on Random Access for Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In the WID for Rel-19 Ambient IoT work[1], RAN2 objectives include the following:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this paper, we discuss the following issues regarding the normative work for Ambient IoT paging:

      - need of unified RA Scheme

      - CBRA and CFRA procedures

      - Re-access

It is worth noting that the Rel-19 Ambient-IoT WI has a reduced scope and a shortened timeline, so we prefer a “tight-the-belt” approach to remove less important design choices even if they are included in SI TR, and aims to support less signaling message formats, less information exchanges, and less specification work for the consideration of timely completion of the WI.

# 2 Discussion

### 2.1 Unified RA scheme

In Rel-19 Ambit IoT WID, it has been required that RAN2#129 needs to make decision about Solution 1 (3-Step only) vs Solution 3 (unified solution). In our view, the proposed unified Solution contains too much flexibility and overhead to be supported by the “low-cost” device type 1. For example:

      - A device need to support two different Msg1 format, one contains a permanent device ID and the other does not contain this ID.

      - A device has to support two different resources for Msg1 transmission, one has a smaller TBS than the other.

      - The device has to examine the Paging message to know whether the paging is supposed to trigger 2-step RACH or 3-step RACH or both; or alternatively

      - The reader/NW need to know which device to support which RACH option w/o any OTA capability signaling.

Given the above considerations:

Proposal 1	RAN2 concludes that only 3-step RACH is supported in Rel-19.

### 2.2 Signalling messages used for RACH procedures

According to the instruction of WID and SI conclusion, only MAC layer signalling will be considered. Thus, all RACH signalling will be layer 2 MAC messages in A-IoT systems. According to RAN2 agreements in SI phase, the following DL and UL message types needs to be considered for the sake of RACH signalign in A-IoT MAC layer:

DL (R2D) Message Type:

      - Paging (Msg0): include (at least) Paging ID

      - Msg2: echo of Msg1 Random ID

      - Msg3Feedback: success/failure of Msg3

UL(D2R) Message Type:

      - Msg1: 16-bit Random ID

      - Msg3: Device ID + data(?)

First, for CFRA, we think only three messages are involved: Paging, Msg3, Msg3Feedback. It is very obvious that there is no need for a Msg1/2 exchange for any random ID because there is no need of collision resolution in contention-free access. It is also clear that the dedicated UL grant for transmission of Msg3 is provided in Paging message (Msg0). After receiving Msg3, reader can send “Msg3Feedback” to confirm the reception in case there is no follow-up R2D data to continue with.

Proposal 2	CFRA procedure involves 3 Message types: Paging, Msg3, Msg3Feedback.

Then, the only addition for CBRA is to add the Msg1/2 exchange in between, so CBRA will involve 5 different messages sequentially.

Proposal 3	CFRA procedure involves 5 Message types: Paging, Msg1, Msg2, Msg3, Msg3Feedback.

Note that Msg3Feedback message is not needed if Msg3 is not received, because in such a case, the reader will just retransmit the last message to trigger a device retransmission of Msg3. Thus, we propose that Msg3Feedback is only needed for the positive case, and negative feedback is not needed. Also, the transmission of Msg3Feedback is not needed if the reader has some data to transmit to the discovered device

Proposal 4	Msg3Feedback (i.e., positive feedback) is only needed if Msg3 is received successfully and the reader has no follow-up “R2D data” to send.

Based on the above discussions, we provide the signalling exchange diagrams for CFRA and CBRA respectively in Figure 1 and Figure 2 below. We believe this would be the exact set of messages to be exchanges for RA procedures with least complexity and specification impact.

Figure 1: Message flow for CFRA Procedures (a) Success case (b) Failures case

Figure 2: Message flow for CBRA Procedures (a) Success case (b) Failures case

Finally, regarding the “AS ID” discussion in SI phase, we think the easiest and simplest fail-safe approach is to let NW/reader assign an AS ID to an A-IOT device, if needed. It should be up to NW to decide whether to use a shorter AS ID or not, given the trade-off between reduced signalling size vs. increased soft-state maintenance requirements. The device will only need to use AS ID if such an ID is assigned by NW. Whether this ID is same as Random ID (included in Msg1) or not is also up to NW to decide.

The benefit of this approach is to provide the maximum flexibility in NW side, with minimum requirements for device side to track/remember the random numbers. It also eliminates any risks of AS ID collisions.

Proposal 5	AS ID is assigned by reader. It is up to NW to decide whether/how to assign AS ID.

Then, there could be potentially a new message type for “AS ID assignment” in MAC layer in R2D direction. We think there is no need to have a dedicated message type for this because this can be merged with Msg3Feedback. Basically, the AS ID assignment signalling can be used as implicit positive feedback of Msg3. RAN2 can formally define this message as Msg4 to replace Msg3Feedback.

With this, to conclude the A-IoT RA procedure, the reader can decide whether to send “Msg4” (to assign a AS ID or as mere positive feedback if no AS ID contained) or “Data” (if it does not want to use AS ID).

Proposal 6	Introduce Msg4 as the “AS ID assignment signalling” in MAC, and also serves as Msg3feedback.

### 2.3. Re-access

In regards of re-access, RAN2 has agreed the following in SI phase:

- It is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

- Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.

- For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

- For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

- R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

- The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose

- Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

In lieu of all the above agreements, the “re-access” scheme seems very complex. However, from the A-IoT device behavior perspective, there is not too much difference between the “initial access” and the so-called “re-access”. This is because there is no automated access/re-access supported for A-IoT device type 1. Every access attempt shall be triggered by R2D trigger message (i.e., paging message). Whenever the paging message contains a matching paging identifier, the device is supposed to “access”. Whether this purpose of access is re-access or not does not matter to the device, unless RAN2 wants to intentionally complicated the design by segregating the access and re-access D2R resources. However, it is very hard to justify adding such a complication:

- There are not any obvious AS layer benefits for such a resource segregation scheme (e.g., designing a separate resource occasion set for the sake of re-access). The overall chance of access success does not improve no matter how many partitions of resource are added. Instead, there is increased chance of resource waster.

- There is no credible justification or analysis to support prioritize/deprioritize “re-access” or “initial access”;

- On the contrary, such a segregation scheme will add singling impact (e.g., need more MAC IE to indicate different resource set) and device complexity.

Thus, for the sake of simplicity and less spec impact, initial access and re-access shall be treated similarly from the perspective of resource allocation.

Proposal 7	D2R resources for initial access and re-access resource are not distinguished.

Accordingly, we think it is also evident that there is no need for introducing other R2D triggers for access purpose except the Paging message. Therefore, the option of “devices can be triggered by R2D message to re-access before subsequent paging retransmission” is not worth pursuing in the normative phase.

Proposal 8	RAN2 not pursue the option “The devices can be triggered by R2D message to re-access before subsequent paging retransmissions” in the normative phase.

# 3 Conclusion

In this contribution, we discuss the Random-Access design for Ambient IoT, and have the following proposals:

Proposal 1	RAN2 concludes that only 3-step RACH is supported in Rel-19.

Proposal 2	CFRA procedure involves 3 Message types: Paging, Msg3, Msg3Feedback. Proposal 8	RAN2 not pursue the option “The devices can be triggered by R2D message to re-access before subsequent paging retransmissions” in the normative phase.

Proposal 3	CFRA procedure involves 5 Message types: Paging, Msg1, Msg2, Msg3, Msg3Feedback.

Proposal 4	Msg3Feedback (i.e., positive feedback) is only needed if Msg3 is received successfully and the reader has no follow-up “R2D data” to send.

Proposal 5	AS ID is assigned by reader. It is up to NW to decide whether/how to assign AS ID.

Proposal 6	Introduce Msg4 as the “AS ID assignment signalling” in MAC, and also serves as Msg3feedback.

Proposal 7	D2R resources for initial access and re-access resource are not distinguished.

Proposal 8	RAN2 not pursue the option “The devices can be triggered by R2D message to re-access before subsequent paging retransmissions” in the normative phase.

# 4 References

[1] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR