# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500427
Athens, Greece, Feburary 17-21, 2025

Agenda item:	6.6

Source:	Apple, Ericsson, Qualcomm

Title:	Discussion on procedure descriptions of SL-DRX

WID/SID:	NR_SL_enh – Release 17

Document for:	Discussion and Decision

# 1 Introduction

In the RAN2#128 meeting, Apple CRs regarding SL-DRX procedure [1][2] have been discussed and the following is captured in Chairman’s notes [3]:

R2-2409961 Correction on SL DRX procedure Apple CR Rel-17 38.321 17.10.0 1982 - F NR_SL_enh-Core R2-2409962 Correction on SL DRX procedure Apple CR Rel-18 38.321 18.3.0 1983 - A NR_SL_enh-Core Noted. Companies would like to have more time to think of. [OPPO]: Change 1 and 4 are ok. [Huawei]: For change 1, we can add the correct RRC configured IE names instead of removing it. [Apple]: Considering sl-drx-HARQ-RTT-Timer is not real RRC configured IE name and it is commonly applied to all cases (both UC and GC/BC), it should be ok without explicitly adding names. [Samsung]: All should be RRC configured IE names because of the sentence “by configuring the following parameters” in the above bullet. [OPPO]: It is not an urgent issue, would like to have more time to think of. [OPPO]: For change 2 and 3, do not understand what is real difference between the current text and the proposed text. [Qualcomm]: The association between QoS and destination id is specified in the upper layer spec. We do not need to specify it in exact. [CATT]: Understand the intention is to make the procedure being performed for each destination Layer-2 ID associated to groupcast or broadcast. [ZTE]: For change 2, isn’t it better to remove “and the associated cast type is groupcast or broadcast” in the sub-bullets since it is already covered in the upper bullet? [OPPO]: It is hard to understand what changes correspond to issue 2 and issue 3. Cover page is also note clear.

As there are some confusion and questions about the changes presented in the CR [1] and the related issues listed in “reason for change” part of the CR cover page, we clarify the problems and issues with more details and illustrate the suggested corresponding TP changes in this paper. Note that the TP may not as same as suggested in CR[1] because better rewording solutions may be considered and recommended.

# 2 Discussion

## 2.1	DRX-related parameters in 5.28.1

In 5.28.1 section of TS 38.321[4], the SL-DRX parameters controlled by RRC are listed.

### 5.28.1 General <text omitted> RRC controls Sidelink DRX operation by configuring the following parameters: - sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer: the duration at the beginning of an SL DRX cycle; - sl-drx-SlotOffset: the delay before starting the sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer; - sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer (except for the SL broadcast communication): the duration after the first slot of SCI (i.e., 1st stage SCI and 2nd stage SCI) reception in which an SCI indicates a new SL transmission for the MAC entity; - sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer (per Sidelink process except for the SL broadcast process): the maximum duration until an SL retransmission is received; - sl-drx-StartOffset: defines the subframe where the SL DRX cycle starts; - sl-drx-Cycle/sl-DRX-GC-BC-Cycle: the Sidelink DRX cycle; - sl-drx-HARQ-RTT-Timer/sl-DRX-GC-HARQ-RTT-Timer (per Sidelink process except for the SL broadcast process): the minimum duration before an SL HARQ retransmission is expected by the MAC entity.

Here, we discuss the issue 1 raised in CR [1]. It is very clear that neither sl-drx-HARQ-RTT-Timer nor sl-drx-GC-HARQ-RTT-Timer is directly configured in RRC because those IE names are not in ASN.1. This is also true for sl-drx-StarOffset and sl-drx-Cycle, as they have to be derived based on RRC-configured sl-drx-CycleStartOffset.

The sl-drx-HARQ-RTT-Timer is actually a UE internal variable can be set according to RRC configured IEs, but also can be set to some other values by UE itself, as shown by the following procedure text in TS 38.321 [4]:

From the above procedure text, we observed:

- SL-DRX-GC-HARQ-RTT-Timer is not a “variable” or “parameter” used in SL-DRX procedure, as sl-DRX-GC-HARQ-RTT-Timer1 or sl-DRX-GC-HARQ-RTT-Timer2 are potential value settings for “sl-drx-HARQ-RTT-Timer”

- sl-drx-HARQ-RTT-Timer does not always take the RRC-configured value(s).

From a), we can see there is no need to keep “SL-DRX-GC-HARQ-RTT-Timer” in the SL-DRX section at all, because it only appears in 5.28.1, but nowhere else.

From b), we think the current text “RRC controls Sidelink DRX operation by configuring the following parameters“does not fully match the SL DRX HARQ RTT timer case. To take this into account, maybe we can add “value” after “sl-drx-HARQ-RTT-Timer “.

Proposal 1	In 5.28.1, remove “SL-DRX-GC-HARQ-RTT-Timer” and add “value” after "sl-drx-HARQ-RTT-Timer"

## 2.2	DRX-parameter determination procedure for GC/BC

For issue 2 and 3 in [1], we discuss them together in this section as they are sort of related. In 5.28.2, the procedure has described how to decide the SL-DRX configuration for GC/BC case for the following three parameters one-by-one:

- sl-DRX-GC-BC-Cycle

- sl-DRX-GC-BC-OnDurationTimer

- sl-DRX-GC-InactivityTimer (for GC only)

The three sets of procedure are quite similar in the current TS 38.321 [4], and all share the same common logic errors.

Let us analyze the following text as a typical example:

1> if a single sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or a single sl-DRX-GC-BC-Cycle that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List is associated to a Destination and the associated cast type is groupcast or broadcast: 2> select the sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List or the sl-DRX-GC-BC-Cycle configured in sl-DefaultDRX-GC-BC associated with the Destination.

By trimming the relative clauses, we can see the highlighted yellow text (the main trunk of the procedure text) is equivalent to say:

“if a single sl-DRX-GC-BC-Cycle… or a single sl-DRX-GC-BC-Cycle….is associated to a Destination and the associated cast type is groupcast or broadcast, (The MAC entity shall) select the sl-DRX-GC-BC-Cycle associated with this Destination”

Logical Error 1: The primary error in above sentence is that there is no any configured “association” between a destination and a DRX cycle. Based on RAN2 agreements [5], what the Sidelink RX UE shall do is for each GC/BC destination, determine the DRX cycle based on the QoS profiles associated with this destination, and then use the determined DRX parameter to guide the UE behavior when receiving SL traffic destinated to this Destination Layer-2 ID. In other words, the Destination Layer 2 ID itself, has nothing to do with these three DRX parameters derivation. Hence, we think the above sentence is misleading and confusing by suggesting the UE shall select a destination-specific DRX cycle. In fact, the whole “Destination” part should be moved out of this detail procedure text to the top-level, as:

“When one or multiple SL DRX is configured, the MAC entity shall, for each destination:”

Similarly, the “associated cast type is groupcast or broadcast” part can also be moved to the top-level, w/o repeating itself in every parameter derivation.

Logical Error 2: let us check the highlighted condition clause related to QoS profile mapping. Firstly,

a single sl-DRX-GC-BC-Cycle that is mapped with one or multiple SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List

The highlighted green part suggests this condition is to let UE to evaluate whether a configured sl-DRX-GC-BC-Cycle can be mapped with “any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List”. However, the configured look-up table sl-DRX-GC-BC-PerQoS-List is constructed to associate one or more SL-QoS-Profile with several SL_DRX configuration parameters which includes a sl-DRX-GC-BC-Cycle parameter. So, Logically, such a sl-DRX-GC-BC-Cycle will always match one or more “SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List” because both are the parameters in the same entry in the configured look-up table. So, such an evaluation is always true and meaningless. Then,

a single sl-DRX-GC-BC-Cycle that is configured in sl-DefaultDRX-GC-BC and cannot be mapped with any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List

The highlighted cyan part condition includes the evaluation of whether the configured default DRX cycle can be mapped with “any SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List”. However, this check is weird and irrelevant because the sl-DefaultDRX-GC-BC is a separate configuration out of the look-up table. No matter whether this check result is true or false, it is not the reason for UE to choose the default cycle. According to RAN2 agreement [5], the UE shall check if the SL QoS Profile passed from V2X layer of SL RX UE associated to the Destination Layer-2 ID match “any “SL-QoS-Profile in sl-DRX-GC-BC-PerQoS-List”. If there is no such match, then the default DRX parameter will be considered. But this critical part is not specified in TS 38.321.

Therefore, we think the above conditions are also wrong and must be corrected.

Logical Error 3 is about handling multiple different SL-DRX parameter values, which is highlighted below:

It is wrong to mandate the “sl-DRX-GC-BC-Cycle configured in sl-DefaultDRX-GC-BC” to be always included in the set of evaluation for the “shortest” one because maybe every SL QoS profile for the SL destination has found its corresponding matching in sl-DRX-GC-BC-PerQoS-List and there is no need to use the default configuration. This needs to be fixed with adding “if applicable” or by restructuring the whole writing with more concise text to avoid this pitfall.

We think the logical error#1 and #2 can be avoided if we regard the QoS-DRX mapping as a RRC layer behaviour. After RRC layer conducts the mapping, MAC layer has only been provided with “Destination layer 2 ID” and associated “SL-DRX parameters”. From this perspective, then we can understand that the current MAC procedure text can be largely simplified to focus on choose a single value among potentially multiple parameter candidates. Literarily, the SL RX UE behaviour is:

RRC layer: For each destination ID, -maps QoS profile(s) to SL-DRX-GC-BC cycle(s) based on RRC configuration. - pass those SL-DRX-GC-BC cycle(s) along with the destination L2 ID to lower layers MAC layer: For each destination ID, - choose a single SL-DRX cycle (among multiple, if needed).

It can been seen that there is no “QoS” related text through TS 38.321 except in the SL-DRX sections. This also implies that any handling of QoS parameters ought not to be exposed to the MAC layer of the UE.

Therefore, we proposal to clarify that the split of RRC and MAC for Rel-17 and then make corresponding specification changes.

Proposal 2-1 	Mapping QoS profile associated to a GC/BC Destination to SL-DRX parameters based on RRC configuration is an RRC layer behaviour and not to be mentioned in MAC.

With this principle, then the MAC layer procedure text can be reduce to the following:

<text omitted> When one or multiple SL DRX is configured, the MAC entity shall, for each Destination Layer 2 ID associated with Groupcast or Broadcast: - if there are multiple different sl-DRX-GC-BC-Cycle values provided based on RRC configuration (including the one in sl-DefaultDRX-GC-BC, if applicable), as specified in TS 38.331 [5]: 2> select the sl-DRX-GC-BC-Cycle whose length is the shortest one among multiple sl-DRX-GC-BC-Cycle. 1> else: 2> select the single sl-DRX-GC-BC-Cycle. 1> if there are multiple different sl-DRX-GC-BC-OnDurationTimer values provided based on RRC configuration (including the one in sl-DefaultDRX-GC-BC, if applicable), as specified in TS 38.331 [5]: 2> select the sl-DRX-GC-BC-OnDurationTimer whose length is the longest one among multiple sl-DRX-GC-BC-OnDurationTimer. 1> else: 2> select the single sl-DRX-GC-BC-OnDurationTimer. 1> if Destination Layer 2 ID is associated with Groupcast: 2> if there are multiple different sl-DRX-GC-InactivityTimer values provided based on RRC configuration (including the one in sl-DefaultDRX-GC-BC, if applicable), as specified in TS 38.331 [5]: 3> select sl-DRX-GC-InactivityTimer whose length is the longest one among multiple sl-DRX-GC-InactivityTimer. 2> else: 3> select the single sl-DRX-GC-InactivityTimer. The MAC entity shall: <text omitted>

Proposal 2-2	Adopted the above simplified TP to 38.321 for the procedure to determine sl-DRX-GC-BC-Cycle, sl-DRX-GC-BC-OnDurationTimer and sl-DRX-GC-InactivityTimer .

Then, regarding the RRC specification, we notice that the following field descriptions exist in current spec [6]:

If we think the field description above is clearly enough, then we can leave RRC spec as it is w/o introducing the additional procedure text to clarify RRC layer behaviour. Otherwise, we can also consider the option to add some TP to explain the RRC layer behaviour more clearly, for example:

##### 5.2.2.4.13 Actions upon reception of SIB12 <text omitted> 2> if sl-DRX-ConfigCommonGC-BC is included in SIB12-IEs: 3> store the NR sidelink DRX configuration, map the QoS profile(s) associated with the GC/BC Destination to SL-DRX configuration(s) based on sl-DRX-ConfigCommonGC-BC, and configure lower layers to perform sidelink DRX operation for groupcast and broadcast as specified in TS 38.321 [3]; #### 5.3.5.14 Sidelink dedicated configuration <text omitted> 1> if sl-DRX-ConfigGC-BC is included in sl-ConfigDedicatedNR within RRCReconfiguration: 2> store the NR sidelink DRX configuration, map the QoS profile(s) associated with the GC/BC Destination to SL-DRX configuration(s) based on sl-DRXConfigGC-BC, and configure lower layers to perform sidelink DRX operation for groupcast and broadcast as specified in TS 38.321 [3];

So, there are two options in regards of RRC changes:

Option 1: No change needed.

Option 2: Adopt the above TP to clarify RRC layer behaviour for SL-DRX configuration determination for GC/BC.

Proposal 2-3	RAN2 discuss which option is to pursue for RRC CR: 1) no need of CR 2) adopt procedure enhancements in 5.2.2.4.13 and 5.3.5.14.

## 2.3	Missing “The MAC entity shall” in 5.28.2

In section 5.28.2, the last part of procedure, which is immediately after DRX Start offset determination formulas for GC/BC case, starts with a leve-1 bullet without subject, as below:

When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, or for the reception of discovery message [26], the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms).

sl-drx-SlotOffset (ms) = (Destination Layer-2 ID modulo the number of slots in one subframe)
/ (the number of slots in one subframe) (ms).

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle or sl-DRX-GC-BC-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

It is obvious that the highlighted sentence above starts to describe a new SL-DRX behaviour which is not limited to GC/BC case, but for all cast types. “The MAC entity shall” part, which is supposed to precede this sentence, was probably wrongly removed by an earlier CR. We need to fix this.

Proposal 3	Add “The MAC entity shall:” after the formula calculating sl-drx-SlotOffset in 5.28.2.

# 3 Conclusion

Proposal 1	In 5.28.1, remove “SL-DRX-GC-HARQ-RTT-Timer” and add “value” after "sl-drx-HARQ-RTT-Timer"

Proposal 2-1 	Mapping QoS profile associated to a GC/BC Destination to SL-DRX parameters based on RRC configuration is an RRC layer behaviour and not to be mentioned in MAC.

Proposal 2-2	Adopted the above simplified TP to 38.321 for the procedure to determine sl-DRX-GC-BC-Cycle, sl-DRX-GC-BC-OnDurationTimer and sl-DRX-GC-InactivityTimer .

Proposal 2-3	RAN2 discuss which option is to pursue for RRC CR: 1) no need of CR 2) adopt procedure enhancements in 5.2.2.4.13 and 5.3.5.14.

Proposal 3	Add “The MAC entity shall:” after the formula calculating sl-drx-SlotOffset in 5.28.2.

# 4 References

[1] R2-2409961	Correction on SL DRX procedure	Apple	CR	Rel-17	38.321	17.10.0	1982	-	F	NR_SL_enh-Core

[2] R2-2409962	Correction on SL DRX procedure	Apple	CR	Rel-18	38.321	18.3.0	1983	-	A	NR_SL_enh-Core

[3] Chairman’s Notes, RAN2#128

[4] TS 38.321 v17.11.0

[5] RAN2#114 Chairman’s Notes: “ RAN2 assumes that the V2X layer of Rx UE passes the PC5 QoS parameters together with the corresponding destination layer-2 ID(s) for reception to the AS layer, as per TR 23.776 conclusion, and will further discuss SL DRX design based on this working assumption”.

[6] TS 38.331 v17.11.0