3GPP TSG-RAN2 Meeting #129	R2-2500429

# Athens, Greece, 17-21 February 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2025 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on SL DRX procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2024-02-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For SL-DRX in MAC specification, issues are found as follow: Issue 1: Similar to “sl-drx-Cycle“, “sl-drx-HARQ-RTT-Timer “ is not an ASN.1-defined field, but another variable which can take one of the following 4 RRC configurations: sl-drx-HARQ-RTT-Timer1 , sl-drx-HARQ-RTT-Timer2, sl-DRX-GC-HARQ-RTT-Timer1 or sl-DRX-GC-HARQ-RTT-Timer2. So, there is no need to say “sl-DRX-GC-HARQ-RTT-Timer “ in 5.28.1, which is not used anywhere else. Issue 2: The mapping between QoS profile (passed from V2X layer) to the SL-DRX cnfigurations is RRC layer behaviro. There is no need to describe thte detailed mapping rules in MAC spec.The MAC spec shall only focus on how to select one value from multiple allowed configuration values. Issue 3: The SL-DRX procedure in 5.28.2 was split into several parts, with each ending with period “.” , but then immediately started with a bullet which means a different SL-DRX case involving different cast types. For example, the UE procedure sequence after the following paragraph does not all mean to be only used by GC/BC case, and it is wrong to start the procedure with 1> without the subject “UE” or “The MAC entity”. When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, or for the reception of discovery message [26], the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations: <equations omitted> We think the signalling procedure after the equations are used for all cast types (GC/BC/UC), not directly connected to the formula and GC/BC conditions described above.This problem through this section needs to be fixed by adding “The MAC entity shall:”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Removed “sl-DRX-GC-HARQ-RTT-Timer” in 5.28.1 and added “value”. - Simplify the GC/BC SL DRX parameters (Cycle, ondurationTimer, inactivityTimer) determination to assume one or more values are already given to MAC layer. - Added the “The MAC entity shall:” in needed space in 5.28.2 Impact analysis Impacted 5G architecture options: NR SA Impacted functionality NR Sidelink DRX Inter-operability: If the UE implements the CR but not the network, there is no inter-operability issue. If the network implements the CR but not the UE, there is no inter-operability issue. If one UE implements the CR but not the other UE, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will not implement the SL-DRX procedures correctly for GC/BC/UC SL DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.28.1 5.28.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

### 5.28.1	General

The MAC entity may be configured by RRC with an SL DRX functionality that controls the UE's SCI (i.e., 1st stage SCI and 2nd stage SCI) monitoring activity for unicast, groupcast and broadcast. When using SL DRX operation, the MAC entity shall also monitor SCI (i.e., 1st stage SCI and 2nd stage SCI) according to requirements found in other clauses of this specification.

Sidelink DRX and UE procedure on Dedicated SL-PRS resource pool are not applied at the same time.

RRC controls Sidelink DRX operation by configuring the following parameters:

-	sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer: the duration at the beginning of an SL DRX cycle;

-	sl-drx-SlotOffset: the delay before starting the sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer;

-	sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer (except for the SL broadcast communication): the duration after the first slot of SCI (i.e., 1st stage SCI and 2nd stage SCI) reception in which an SCI indicates a new SL transmission for the MAC entity;

-	sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer (per Sidelink process except for the SL broadcast process): the maximum duration until an SL retransmission is received;

-	sl-drx-StartOffset: the slot where the SL DRX cycle starts;

-	sl-drx-Cycle/sl-DRX-GC-BC-Cycle: the Sidelink DRX cycle;

-	sl-drx-HARQ-RTT-Timer value (per Sidelink process except for the SL broadcast process): the minimum duration before an SL HARQ retransmission is expected by the MAC entity.

### 5.28.2	Behaviour of UE receiving SL-SCH Data

When SL DRX is configured, the Active Time includes the time while:

-	sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer or sl-drx-InactivityTimer/sl-DRX-GC-InactivityTimer is running; or

-	sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer is running; or

-	period of sl-LatencyBoundCSI-Report configured by RRC in case SL-CSI reporting MAC CE is not received; or

-	the time between the transmission of the request of SL-CSI reporting and the reception of the SL-CSI reporting MAC CE in case SL-CSI reporting MAC CE is received; or

-	Slot(s) associated with the announced periodic transmission(s) by the UE transmitting SL-SCH Data; or

-	the time between transmission/reception of Direct Link Establishment Request message (TS 24.587 [28]) or ProSe Direct Link Establishment Request message (TS 24.554 [29]) and reception of RRCReconfigurationSidelink message including initial DRX configuration or the link establishment procedure being aborted by upper layer; or

-	the time between transmission of RRCReconfigurationSidelink message including initial DRX configuration and reception of corresponding RRCReconfigurationCompleteSidelink or RRCReconfigurationFailureSidelink message.

When one or multiple SL DRX is configured, the MAC entity shall, for each Destination Layer-2 ID associated to groupcast or broadcast:

.

1>	if there are multiple different sl-DRX-GC-BC-Cycle values provided based on RRC configuration (including the one in sl-DefaultDRX-GC-BC, if applicable), as specified in TS 38.331 [5]:

2>	select the sl-DRX-GC-BC-Cycle whose length is the shortest one among multiple sl-DRX-GC-BC-Cycle values.

1> else:

2> select the single sl-DRX-GC-BC-Cycle

1> if there are multiple different sl-DRX-GC-BC-OnDurationTimer values provided based on RRC configuration (including the one in sl-DefaultDRX-GC-BC, if applicable), as specified in TS 38.331 [5]:

2>	select the sl-DRX-GC-BC-OnDurationTimer whose length is the longest one among multiple sl-DRX-GC-BC-OnDurationTimer values.

1> else:

2> select the single sl-DRX-GC-BC-OnDurationTimer.

1> if Destination Layer 2 ID is associated with Groupcast:

2>	if there are multiple different sl-DRX-GC-InactivityTimer values provided based on RRC configuration (including the one in sl-DefaultDRX-GC-BC, if applicable), as specified in TS 38.331 [5]:

3> select sl-DRX-GC-InactivityTimer whose length is the longest one among multiple sl-DRX-GC-InactivityTimer values.

2> else:

3> select the single sl-DRX-GC-InactivityTimer.

The MAC entity shall:

1>	if an sl-drx-HARQ-RTT-Timer expires:

2>	if the data of the corresponding Sidelink process was not successfully decoded or if the HARQ feedback (i.e., negative acknowledgement) is not transmitted for unicast due to UL/SL prioritization:

3>	start the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process in the first slot after the expiry of sl-drx-HARQ-RTT-Timer.

When the cast type is groupcast or broadcast as indicated by upper layer, or the cast type is unicast for the reception of Direct Link Establishment Request message [28] or ProSe Direct Link Establishment Request message [29] as indicated by upper layer, or for the reception of discovery message [26], the sl-drx-StartOffset and sl-drx-SlotOffset are derived from the following equations:

sl-drx-StartOffset (ms) = Destination Layer-2 ID modulo sl-DRX-GC-BC-Cycle (ms).

sl-drx-SlotOffset (ms) = (Destination Layer-2 ID modulo the number of slots in one subframe)
/ (the number of slots in one subframe) (ms).

The MAC entity shall:

1>	if the SL DRX cycle is used, and [(DFN × 10) + subframe number] modulo (sl-drx-Cycle or sl-DRX-GC-BC-Cycle) = sl-drx-StartOffset:

2>	start sl-drx-onDurationTimer/sl-DRX-GC-BC-OnDurationTimer after sl-drx-SlotOffset from the beginning of the subframe.

1>	if an SL DRX is in Active Time:

2>	monitor the SCI (i.e., 1st stage SCI and 2nd stage SCI).

2>	if the SCI indicates a new SL transmission:

3>	if Source Layer-1 ID of the SCI is equal to the 8 LSB of the intended Destination Layer-2 ID and Destination Layer-1 ID of the SCI is equal to the 16 LSB of the intended Source Layer-2 ID and the cast type indicator in the SCI is set to unicast:

4>	start or restart sl-drx-InactivityTimer for the corresponding Source Layer-2 ID and Destination Layer-2 ID pair in the first slot after SCI reception.

3>	if Destination Layer-1 ID of the SCI (i.e., 2nd stage SCI) is equal to the 16 LSB of the intended Destination Layer-2 ID and the cast type indicator in the SCI is set to groupcast:

4>	start or restart sl-DRX-GC-InactivityTimer for the corresponding Destination Layer-2 ID in the first slot after SCI reception.

2>	if the SCI indicates an SL transmission:

3>	if a next retransmission opportunity is indicated in the SCI:

4>	derive the sl-drx-HARQ-RTT-Timer from the retransmission resource timing of the next retransmission resource in the SCI.

3>	else if PSFCH resource is configured for the SL grant associated to the SCI:

4>	set the sl-drx-HARQ-RTT-Timer based on sl-drx-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer1 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is enabled, or based on sl-drx-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is unicast or sl-DRX-GC-HARQ-RTT-Timer2 configured by upper layer if the cast type associated with the SCI is groupcast when HARQ feedback is disabled, for resource pool configured with PSFCH.

3>	else (i.e., if PSFCH resource is not configured for the SL grant associated to the SCI):

4>	set the sl-drx-HARQ-RTT-Timer as 0 slots.

3>	if PSFCH resource is not configured for the SL grant associated to the SCI:

4>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

3>	if PSFCH resource is configured for the SL grant associated to the SCI:

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is unicast:

5>	if sl-NumPSFCH-Occasions is configured as specified in TS 38.331 [5]:

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of corresponding PSFCH transmission carrying the SL HARQ Feedback when the SL HARQ feedback is successfully transmitted in one of PSFCH occasions from sl-NumPSFCH-Occasions; or

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ Feedback when the SL HARQ feedback is not transmitted in all PSFCH occasions from sl-NumPSFCH-Occasions.

5>	else:

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization or SL LBT failure.

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and positive-negative acknowledgement is selected:

5>	if sl-NumPSFCH-Occasions is configured as specified in TS 38.331 [5]:

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ Feedback.

5>	else:

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization.

4>	if HARQ feedback is enabled by the SCI and the cast type associated with the SCI is groupcast and negative-only acknowledgement is selected;

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is not transmitted due to UL/SL prioritization; or

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH resource for the SL HARQ feedback when the SL HARQ feedback is a positive acknowledgement.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is not scheduled in the SCI:

5>	if sl-NumPSFCH-Occasions is configured as specified in TS 38.331 [5]:

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion.

5>	else:

6>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSFCH resource.

4>	if HARQ feedback is disabled by the SCI and the resource(s) for one or more retransmission opportunities is scheduled in the SCI:

5>	start the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the slot following the end of PSSCH transmission (i.e., currently received PSSCH).

NOTE:	Void.

3>	stop the sl-drx-RetransmissionTimer/sl-DRX-GC-RetransmissionTimer for the corresponding Sidelink process.

1>	if an SL DRX Command MAC CE is received for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast:

2>	stop sl-drx-onDurationTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast;

2>	stop sl-drx-InactivityTimer for the Source Layer-2 ID and Destination Layer-2 ID pair of a unicast.

End of the change