# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500432
Athens, Greece, February 17-22, 2025

Agenda item:	8.13.2

Source:	Apple

Title:	Relay discovery and selection for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

RAN2#127 [1] has made the following agreements on relay discovery and selection:

- From RAN2 perspective, models A and B can both be supported. - Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying. - Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying. - Reuse existing resource pools defined for discovery message transmission and reception - Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE. - Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18. - If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. - If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). - FFS discovery conditions for the intermediate relay UEs.

RAN2#128[2] meeting has further reached the following agreements for relay discovery and selection:

- A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. - If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. - The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. - The following cases are supported for L2 multi-hop relay: - - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. - FFS if the last relay UE can use the same L2ID for both of the connections in either case. - Cases with two indirect paths to the gNB for the same remote UE are not supported. - An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links. - RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

In this paper, we continue to discuss some controversial issues and FFS(s) in relay discovery and selection aspects for Multi-hop UE-to-NW relay.

# 2 Discussion

### 2.1 Uu threshold for operating as Intermediate relay UE

Regarding the Uu threshold issue for intermediate relay UE’s transmission of U2N relay discovery message, RAN2 has agreed that a lower bound is not needed and NW may optionally configure a single upper bound. RAN2 also agrees that if the upper bound is not provided, then the following agreement is still applicable as a restriction to intermediate relay UE:

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE

However, is the above agreement still applicable if an upper bound is configured? We believe it should be applicable. too. However, depends on whether the upper bound is weaker or stronger than the Uu RSRP lower bound for being a “Last relay UE”, this may be confusing as a UE may be located in a region which is both eligible as a last relay UE and as an intermediate relay UE based on the RRC configuration. Therefore, we think RAN2 need to clarify this.

Proposal 1 	RAN2 clarify that the agreement “If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE “ always take precedence regardless of whether upper bound (for operation as intermediate relay UE) is configured or not.

Regarding the FFS issue “FFS if there would be additional restrictions in the case of no upper bound.”, we need to consider two cases:

- (a) There is no configured upper bound for being last relay UE, as shown in case (a) of Figure 1.

- (b) There is a configured upper bound (e.g., threshHighRelay-r17) for being the last relay UE, as shown in case (b) of Figure 1.

Figure 1: Relationship between intermediate relay UE location region and Uu RSRP

(a) w/o threshHighRelay-r17 configured vs (b) w/ threshHighRelay-r17 configured

If there is no configured upper bound for being the last relay UE, then it means any UE which satisfied the lower bound to being a relay UE will be acting as a relay UE. For this case, there is no additional restriction needed.

But for the other case, according to the current agreement, an intermediate relay UE may happen to locate very close to the centre of the cell as long as its Uu RSRP measurement is larger than threshHighRelay-r17. In this case, per the current RAN2 agreement, as the UE is not eligible to be a last relay UE, then it is eligible to be an in-coverage intermediate relay UE. But this is not a reasonable configuration, if the motivation to configure threshHighRelay-r17 is to prevent strong SL transmission in UL band from an in-coverage relay UE from interfering gNB reception, then this motivation is also applicable to the intermediate relay UE. Therefore, we think even if the upper bound for intermediate relay UE operation is not configured, there should always be an implicit restriction of keeping intermediate relay UE at least outside of the in-coverage last relay UE eligible region.

Proposal 2 	When Uu RSRP upper bound for operating as an intermediate relay UE is not configured, a relay UE only operate as intermediate relay UE towards the same cell, if its Uu RSRP measurement is less than the configured lower bound of Uu RSRP for operating as a last relay UE.

Regarding “FFS if operation as an intermediate relay UE towards a different cell would be supported“, our understanding is that this is a valid scenario. An in-coverage relay UE can act as an intermediate relay UE towards another cell, especially after SA2 has answered “yes” in Reply LS [3] to the RAN2 question “whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message”. Basically, the intermediate relay UE can establish PC5 link with a last relay UE camped in a different cell, and received the “Announced info” in model A relay discovery message. Then, it can announce  itself as one-hop intermediate relay UE towards that last relay UE. This is a very useful case especially if there is no eligible last relay UE in the current cell.

Proposal 3 	An in-coverage intermediate relay UE can announce itself as the intermediate relay towards a cell different from its camped cell after it established PC5 link with a last relay UE camped in that cell.

Logically, in this case, the intermediate relay UE’s camped cell may provide a Uu RSRP upper bound to intermediate relay UE operation. According to RRC norm, the UE shall comply to this NW-configured threshold even when it act as intermediate relay UE towards a different cell.

Proposal 4 	Upper bound of Uu RSRP configured in current cell applies to the intermediate relay UE for the case when it serves as an intermediate relay UE towards a different cell.

Finally, it is reasonable to assume the restriction “If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE “ is not to be used to restrict the relay UE if it would operate as intermediate relay UE towards a different cell. For example, the current cell may only support Layer-3 multi-hop U2N relay operation, but not supporting L2 U2N relay, so it is valid for a Layer-2 intermediate relay UE to announce itself towards the (different) cell supporting L2 operation.

Proposal 5 	A relay UE eligible to be the last relay UE in its camped cell may serve as an intermediate relay UE towards a different cell.

### 2.2 Cross-Path Scenarios

Cross-path scenarios discussed in email discussion [4] for relay discovery and selection, with no consensus whether it is to be supported in Rel-19. For example, they are two possible scenarios are shown in Figure 2 below:

Figure 2: Cross-path scenarios

In scenario (a), Remote UE has a single path Remote →relay_first→relay_last1→gNB1 to reach the NW, and Remote UE 2 has its respective single multi-hop path Remote 2→ relay_first→relay_last2→gNB2 to reach the NW. In scenario (b), similarly, there are two different paths between remote UE(s) and their respective gNBs, although sharing a common PC5 link between the two intermediate relay UEs.

It is a misunderstanding that the above scenarios are multi-path scenarios (e.g., there are two indirect path between remote UE ang gNB(s). As can be seen above, for each Uu DRB between remote UE and its corresponding gNB, (i.e., between the pair of Uu PDCP entities), there is only a single path. The multi-path concern is only meaningful is the PDCP entity is on the intermediate relay UE. But for end-to-end traffic transported by intermediate relay UEs, only SRAP entity is operating and there is no need to establish Uu PDCP entity in . Even in that case, the Uu PDCP entity in intermediate relay UE can still use just a single path to reach one of the preferred gNBs for its own traffic.

Proposal 6	Cross-path discovery scenarios (i.e., the same intermediate relay UE providing different path options to different cells) shall be supported in Rel-19.

### 2.3 Resource allocation for discovery transmission

Regarding the FFS in the following agreement [1]:

Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

We think for remote UE in Multi-hop R19 scenario, it does not have a direct Uu link and it cannot receive DCI signal transmitted by gNB. Therefore, mode 1 resource allocation is anyway crippled this remote UE as not all mode 1 mechanisms are working. It is much simpler to not support mode 1 and only allow remote UE to use resource allocation mode 2.

It is also worth clarifying that the agreement “Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs;” means for legacy R17/R18 relay which has to be the last relay UE. For intermediate relay UE which connects to the gNB via multiple hops, mode 1 support is also questionable. Hence, a much cleaner design is to only have mode 2 resource allocation for both remote UE and intermediate relay UEs.

Proposal 7 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

### 2.4 Intermediate Relay UE reselection

Any PC5 link of intermediate relay UE may fail. If such a RLF is detected (for example, as shown in Figure 4) To consider the forward extensibility of relay discovery design, we consider the scenario with two hops as an example:

Figure 3: PC5 RLF detected between 2nd intermediate relay UE and last relay UE

There are two options can be considered:

Option 1: The intermediate relay UE performs relay reselection and find a proper last relay UE.

Option 2: The intermediate relay UE sends notification message to remote UE, and let remote UE to re-initiate Model B discovery.

In our understanding, both options are feasible, and an intermediate UE can even do both. Nonetheless, for the sake of fast recovery, it is desirable to support intermediate relay UE to immediately start relay reselection procedure to find an alternative path towards gNB.

Similarly, if a Uu RLF is detected, the last relay UE will inform the Uu RLF to the intermediate relay UE. In this case, the intermediate relay UE shall also trigger relay re-selection for fast recovery.

Proposal 8	Support intermediate relay UE to trigger relay reselection if PC5 RLF is detected or Uu RLF is informed.

# 3 Conclusion

In this contribution, we discuss the Relay discovery and selection for Multi-hop UE-to-NW Relay, and have the following proposals:

Proposal 1 	RAN2 clarify that the agreement “If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE “ always take precedence regardless of whether upper bound (for operation as intermediate relay UE) is configured or not.

Proposal 2 	When Uu RSRP upper bound for operating as an intermediate relay UE is not configured, a relay UE only operate as intermediate relay UE towards the same cell, if its Uu RSRP measurement is less than the configured lower bound of Uu RSRP for operating as a last relay UE.

Proposal 3 	An in-coverage intermediate relay UE can announce itself as the intermediate relay towards a cell different from its camped cell after it established PC5 link with a last relay UE camped in that cell.

Proposal 4 	Upper bound of Uu RSRP configured in current cell applies to the intermediate relay UE for the case when it serves as an intermediate relay UE towards a different cell.

Proposal 5 	A relay UE eligible to be the last relay UE in its camped cell may serve as an intermediate relay UE towards a different cell.

Proposal 6	Cross-path discovery scenarios (i.e., the same intermediate relay UE providing different path options to different cells) shall be supported in Rel-19.

Proposal 7 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

Proposal 8	Support intermediate relay UE to trigger relay reselection if PC5 RLF is detected or Uu RLF is informed.

# 4 References

[1] Chairman’s notes, RAN2#127

[2] Chairman’s notes, RAN2#128

[3] S2-2501334, Reply LS on relay discovery announcement

[4] R2-2410981, Report of [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)