# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500433
Athens, Greece, February 17-22, 2025

Agenda item:	8.13.3

Source:	Apple

Title:	Discussion on Control Plane for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_multihop– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#128 meeting [1], the following agreements has been made for control plane design:

- In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE. FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE. - The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted. - In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED. - In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection. - The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted - For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation. This does not preclude the possibility of assistance information from the UE(s). - If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network. FFS how to split the PDB over each of the individual remaining hops. - In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s). FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE). - Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED. - 1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell. - 2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. - All agreements in this WI apply to both these cases unless otherwise specified.

In this paper, we continue to discuss the control plane design issues for Layer-2 Multi-hop UE-to-NW relay.

# 2 Discussion

### 2.1 Remote UE Connection Setup

In RAN2#128 meeting, RAN2 has agreed that the CONNECTED state intermediate U2N relay UE is a case which has to be supported. But there are also some understandings that the process to bring those intermediate relay UEs into CONNECTED can be further discussed. It may not need to be in CONNECTED state right at the very beginning when remote UE triggers the connection setup procedure, as Approach 1 mandates.

We have analyse the pros and cons of the Approach 1 and Approach 2 in R2-2409968 [2]. Here we provide some additional perspectives to clarify the benefits of Approach 2:

Let us begin with the Approach 1 of Layer-2 two-hop U2N relay case:

Figure 1: End-to-End RRC Connection Setup for MH (1 extra hop) case with all UE started in RRC_IDLE state and ended in RRC_CONNECTED state

From Figure 1, we can see that from the last relay UE perspective it has been configured twice in regards of L2 U2N relay operation:

- In the first reconfiguration, the last relay UE has reported intermediate relay UE to gNB via its SUI message, the gNB will assign local ID for intermediate relay UE and corresponding RRC configuration (SRAP, Relay RLC channels) will be provided to the last relay UE to support the SRB/DRBs to the intermediate relay UE.

- In the second reconfiguration, the intermediate relay UE has reported the remote UE to gNB via SUI message from the intermediate relay UE, the gNB will assign local ID for the remote UE. Now, for the last relay UE, it has to be reconfigured again by the gNB with SRAP and Relay RLC channel configurations to support the e2e SRB/DRBs towards the remote UE.

Literally, the last relay UE is configured N times sequentially for N-hop U2N relay in Approach 1, while each time the SRAP configuration is completely overwritten by the gNB, as there is no “delta configuration” for “SL-SRAP-Config” in ASN.1, according to TS 38.331..

Observation 1	In Approach 1, the SRAP configuration has been fully reconfigured repetitively for the last relay UE, but only the last one is meaningful for the relay UE’s UP operation. Same is true for any intermediate relay UEs which are not immediately adjacent to the remote UE.

Some may argue that the those RRC configurations can provide the SRAP configurations for allowing the the last relay UE to handle the end-to-end SRB/DRB traffic from the intermediate relay UE(s) to NW. Nonetheless, as intermediate relay UE does not have its own traffic, the only useful part in those configurations is the SRAP configuration to support end-to-end Uu SRB from the intermediate relay UE to the gNB. So, those RRCReconfiguration messages only provide a short-term SRAP mapping for Uu SRB1 originated from intermediate relay UE before those SRAP configuration is (re-)overwritten again by another round of RRCReconfiguration. However, it is questionable that whether such an iterative configuration is needed, due to the following reasons:

- If there is a need for forwarding of intermediate relay UE’s message during the connection setup process, there could be some default PC5 Relay RLC channel to be used (e.g., SL-RLC0, SL-RLC1). Anyway, there is no NW-controlled QoS split for the multi-hop Uu SRB1. So, any default PC5 relay RLC channels provided by NW is as good as the dedicated configurations.

- If the intermediate relay UE stays in IDLE/INACTIVE, then there is no any Uu SRB1 signalling to be transmitted/forwarded.

Observation 2	During connection setup procedure, CP signalling passing through the intermediate UE can utilize default SL-RLC0/SL-RLC1 w/o relying on dedicated RRC configurations.

One shortcoming of Approach 1 is that all the repeated RRCReconfiguration messages to the relay UE(s) will also trigger corresponding PC5-RRC reconfiguration procedure to potentially re-configure the PC5 relay RLC channels, as some gNB configuration may require UEs to configure a new PC5 Relay RLC channel for the Uu SRB(s) of the newly added downstream intermediate relay(s), thereby requiring more PC5-RRC signalling involved.

Observation 3	Excessive NW configuration signalling also result into excessive PC5-RRC signalling, which adds to the overall signalling overhead.

Regarding with the overall latency of the whole remote UE’s end-to-end connection setup procedure, it can be approximately assessed by the equivalent number of per-hop transmission needed for all the control signalling involved.

As shown in Table 1 below, the total number of Uu & PC5-RRC signalling transmissions (per-hop) increase by O(n2) with the increasing number of extra hops in Approach 1, while the signalling overhead for Approach 2 is still linear with O(n).  The detailed mathematical analysis behind those equation can be found in [2].

| Approach 1 | 22+25.5N+4.5N2 |
| --- | --- |
| Approach 2 | 22+10N |

Table 1 Overall Signalling overhead for N-hop U2N relay: Approach 1 vs. Approach 2

To mitigate the latency concern, it is critical to consider to alert/inform the last relay UE earlier instead of waiting for the recursive triggering of each intermediate relay UE one-by-one to enter CONNECTED state and sending corresponding SUI message.

Observation 4	Latency can be signification reduced if the last relay UE is informed about remote UE’s connection request (SRB0) as early as possible.

It is also worth noting that there could be different design options to realize this expeditated notification procedure of remote UE trigger. In option (a) shown in Figure 2, a new signalling encapsulating the Uu SRB0 message and Remote UE L2 ID can be originated by remote UE and fast-forwarded to the last relay UE using SL-RLC0 channels. Alternatively,  Option (b) design in Figure 2 does not need any change of the remote UE behaviour. In (b) of Figure 2, the remote UE can send its first RRC message (SRB0 message) as usual in the default SL_RLC0 channel, then the first intermediate relay UE will immediately forward this message along with remote UE’s L2 ID extracted from SL MAC header to the next-hop relay UE, until this message reaches the last relay UE along the path. With this approach, even a legacy Rel-17 Remote UE can enjoy the benefits of this expediated connection setup process.

Observation 5	Fast forwarding of RRC trigger among the relay UEs can be compatible with legacy U2N remote UE.

Figure 2 “Expediated” SRB0 trigger forwarding design options

Regarding this expediated trigger, we can see at least the “remote UE’s L2 ID” needs to be forwarded hop-by-hop towards the last relay UE, so that the last relay UE can report this remote UE in its SUI message to solicit appropriate SRAP configurations. This can be done with several different variants, as shown above. One can consider an enhanced SRAP design to encapsulate this information. It is a small signalling impact to be added in Rel-19 specification but providing overwhelming benefits to reduce the overall signalling overhead and latency.

Note that it is a misconception that Approach 1 and Approach 2 are two competing approaches to multi-hop U2N relay design. On the contrary, Approach 1 and 2 can be complementary to each other. If the intermediate relay UE is already in RRC_CONNECTED state, then the existing single-hop U2N frame work can be applied;  When the relay UE is in IDLE/INACTIVE, as an alternative to triggering itself for initiating RRC connection establishment/resumption, the intermediate relay UE can expediate the triggering towards the NW by forwarding the triggering message (e.g., Uu SRB0 message) immediately to its next-hop relay in the default relay RLC channel (e.g., SL-RLC0). Then, the last relay UE can report the “true” trigger in its SUI message and get the ultimate configuration needed for end-to-end forwarding once instead of being repetitively (re-)configured by gNB.

Proposal 1	Support fast SRB0 message forwarding with IDLE/INACTIVE intermediate relays.

### 2.2 Paging and SIB forwarding by Intermediate relay UE

Note that during the multi-hop U2N relay discovery, there is a clear rule to be applied for a relay UE to determine whether it serves as the last relay UE or serves as an intermediate relay UE. We think this role distinction is also applicable to paging and SI forwarding, in a similar way.

It is clear that an intermediate relay UE shall not pretend to be the last relay UE by providing its own SI or intercept paging on behalf of remote UE, or pretend to be the remote UE by originate its own SI or Paging request. Even if the UE alteasdy has a Uu connection to the gNB (i.e., in CONNECTED state), it can still at as same as OOC intermediate relay UE which do not bother to listen/receive paging or SI information. Instead, for paging & SI forwarding, it would be very simple and clear to keep the intermediate relay UE only playing a “forwarder” role. The paging and SI forwarding framework can be largely as same as legacy U2N relay design with remote UE and (last) relay UE involved to play its legacy roles in the legacy procedures, plus the new forwarder(s) of PC5-RRC signalling involved. From this perspective, the intermediate relay UE is only responsible to forwarding the SI/Paging request/response between remote UE and the last relay UE. To be more specific, the RemoUEInformationSL and UuMessageTransferSL can be forwarded “blindly” by the intermediate relay UE w/o even looking into the contents of those messages. For example, the intermediate relay UE even does not need to check whether RemoteUEInformationSL is asking for paging or SI request or both, as it can simply forward the message unaltered to its next-hop node along the path. The intermediate relay UE does not need to register or remember any remote UE interests and maintain no states about SI/Paging requests for the remote UE.

Proposal 2	Intermediate relay UE only act as “stateless forwarder” for SI/Paging transactions between remote UE and last relay UE, w/o checking/consuming the contents of the SI/Paging.

There is an issue about how the last relay UE deliver SI and Paging (Uu Messages) to its final destination (remote UE). This is a unique problem for multi-hop relay UE because there exist “stateless” forwarders between last relay UE and remote UE. We think this can be solved by the following two options:

- Option 1: Forward the UuMessageTransferSL to every connected downstream node, as similar to an Ethernet bridge.

- Option 2: Forward the UuMessageTransferSL for SI/Paging based on L2 address (Src L2 ID of Remote UE).

The benefit of Option 1 is that no signalling change is expected, the only behaviour needs to be added is the intermediate relay UE’s behaviour upon the reception of UuMessageTransferSL. It may deliver some Paging records or SI to some remote UE not interested, but this can be a trade-off between small singaling overhead increase vs. the cost of maintaining soft states in intermediate relay UE(s).

For Option 2, Remote UE L2 ID can be added in SRAP header which is pre-pended to the SL PDCP PDU containing PC5-RRC message RemoteUEInfromationSL or UuMessageTransferSL. The benefit of Option 2 is that there will be no signalling waste to “flood” the UuMessageTransferSL, as the relay UE will indicate the final destination (Remote UE L2 ID) to the forwarders. If this option is adopted, this can be unified with the proposed SRAP enhancement needed for Uu SRB0 (for Proposal 1), so that there would be no new SRAP changes needed on the top of the implementation of Proposal 1.

Proposal 3	RAN2 consider two signaling delivery options to support the forwarding of SI/Paging: 1) Last relay UE forward UuMessageTransferSL to every connected downstream node; or 2) last relay UE provide L2 address (Src L2 ID of Remote UE) to allow address-based L2 forwarding of UuMessageTransferSL towards the remote UE.

# 3 Conclusion

In this contribution, we discuss the CP design for Multi-hop SL relay, and have the following observations:

Observation 1	In Approach 1, the SRAP configuration has been fully reconfigured repetitively for the last relay UE, but only the last one is meaningful for the relay UE’s UP operation. Same is true for any intermediate relay UEs which are not immediately adjacent to the remote UE.

Observation 2	During connection setup procedure, CP signalling passing through the intermediate UE can utilize default SL-RLC0/SL-RLC1 w/o relying on dedicated RRC configurations.

Observation 3	Excessive NW configuration signalling also result into excessive PC5-RRC signalling, which adds to the overall signalling overhead.

Observation 4	Latency can be signification reduced if the last relay UE is informed about remote UE’s connection request (SRB0) as early as possible.

Observation 5	Fast forwarding of RRC trigger among the relay UEs can be compatible with legacy U2N remote UE.

Based on the discussion, we propose:

Proposal 1	Support fast SRB0 message forwarding with IDLE/INACTIVE intermediate relays.

Proposal 2	Intermediate relay UE only act as “stateless forwarder” for SI/Paging transactions between remote UE and last relay UE, w/o checking/consuming the contents of the SI/Paging.

Proposal 3	RAN2 consider two signaling delivery options to support the forwarding of SI/Paging: 1) Last relay UE forward UuMessageTransferSL to every connected downstream node; or 2) last relay UE provide L2 address (Src L2 ID of Remote UE) to allow address-based L2 forwarding of UuMessageTransferSL towards the remote UE.

# 4 References

[1] Chairman’s Notes, RAN2#128

[2] R2-2409968	, Discussion on End-to-End Connection Setup Approaches for Multi-hop UE-to-NW Relay, Apple