# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500434
Athens, Greece, February 17-22, 2025

Agenda item:	8.13.3

Source:	Apple

Title:	Discussion on SRAP for Multi-hop Layer-2 U2N Relay

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

.In RAN2#128 meeting [1], the following agreements has been made for SRAP:

- Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel. This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs. - Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

In this paper, we discuss the SRAP design for Layer-2 Multi-hop UE-to-NW relay

# 2 Discussion

## 2.1	SRAP Entity in Intermediate Relay UE

For the new “intermediate relay UE” introduced in Rel-19 work, we think it would just have a single PC5 SRAP entity which handles ingress and egress PC5 Relay RLC Channels. There is no Uu SRAP entity used for intermediate relay UE operation.

It has been raised in RAN2#128 [1] that in case Rel-19 SRAP header is different from legacy Rel-17 SRAP header, then the intermediate relay UE may need to encapsulate two versions of SRAP header in its forwarding operation, thereby resulting into the two SRAP entities in the same relay UE. This is only possible if the last relay UE in Rel-19 Scenario is the legacy Rel-17 relay UE. Nonetheless, the legacy “SL-L2RelayUE-Config-r17” structure in Rel-17 RRCReconfiguration [2] cannot support multiple downstream paths, with each of the path containing multiple local IDs assigned, as required by Approach 1 baseline connection setup procedure. Thus, there is no chance to have a legacy Rel-17 relay UE acting as the last relay UE for Rel-19 scenario. So, we can safely conclude that such a case is infeasible and only Rel-19 SRAP header will be used by the last relay UE which has to supports Rel-19 specification.

Given that, we think it is true that Layer-2 intermediate relay UE only has a single PC5 SRAP entity.

Proposal 1	 Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

## 2.2	PC5 Relay RLC channel configuration for Intermediate Relay UE

Then, in legacy L2 U2N Relay design, only one set of PC5 Relay RLC channel configurations are provided to a L2 U2N remote UE or L2 U2N relay UE in legacy single-hop relay design, as specified in RRC [2]. Then, for L2 remote UE, it is supposed to create PC5 Relay RLC channels based on the RRC configuration on the PC5 link between the remote UE the relay UE. For relay UE, it should create PC5 Relay RLC channels based on the configuration on each of the PC5 link to each of the connected remote UE(s). This works in legacy single-hop case because either remote UE or relay UE in Rel-17/Rel-18 design only has one direction to deliver the SRAP traffic, either towards the remote UE or towards the relay UE (and gNB). But In Rel-19 topology, each intermediate relay UE has two directions to deliver traffic, then it is unclear which one of the following two options would apply:

- Option 1: gNB provide two set of PC5 Relay RLC channel configurations to each intermediate relay UE, one for the direction to NW, one for the direction to remote UE;

- Option 2: gNB still provide a single set of PC5 Relay RLC channel configurations to each intermediate relay UE, but the UE need to establish (at least) two identical PC5 Relay RLC channels in both directions (one towards a downstream UE and one towards an upstream UE) for the end-to-end delivery.

Both of the above two options are feasible. For Option 2, it seems that the intermediate UE are obliged to establish symmetric PC5 Relay RLC channels in both UL and DL directions. However, in SRAP mapping of end-to-end traffic, it is very possible that UL and DL mapping may use different number of egress PC5 Relay RLC channels, and are not symmetric. This means some PC5 Relay RLC channels configured are not needed, at least in one of the directions. However, even if a PC5 Relay RLC channel is established, it can still be released by UE if not used. So, Option 2 can still work with probably less signalling overhead, no matter whether the PC5 Relay RLC channel configurations is provided by dedicated RRC signalling or SIB/Pre-configuration. So, we think Option 2 can be adopted.

Proposal 2	For SRAP mapping in intermediate relay, a single set of PC5 Relay RLC channel configurations is used for both directions, whereas different SRAP mapping could be used in each direction.

## 2.3	SRAP operation after NW providing dedicated configuration

Regarding relay UE’s handling of SRAP PDU to determine forwarding, the legacy U2N relay UE design is very simple:

- If an SRAP PDU received from Uu interface (Uu SRAP), then it is supposed to be mapping to an egress PC5 Relay RLC channel based on SRAP mapping

- If an SRAP PDU is received from PC5 interface (PC5 SRAP), then it is supposed to be mapping to an egress Uu relay RLC channel based on SRAP mapping.

However, for intermediate relay UE in multi-hop scenario, there is some new complexity in decision-making of traffic forwarding. Since there is only one PC5 SRAP entity associated with multiple PC5 links, the intermediate relay UE needs to determine which PC5 link should be used for forwarding the SRAP PDU based on SRAP mapping. If the SRAP header itself does not have an indication of “direction”, the relay UE need to dig into the source layer-2 address to understand which PC5 link the packet comes from and determine that the Src L2 ID of the received packet is in its upstream or downstream. This means the intermediate relay UE has to maintain additional context of L2 IDs of each PC5 link and maintain another mapping table per each remote UE for those L2 IDs. This would be inconvenient for Sidelink UE as L2 ID(s) may need change from time to time for a PC5 link. We think a simple enhancement to streamline the SRAP forwarding operation is to add a “directional” indication bit in SRAP header, this single-bit indicator can tell the relay whether this is UL traffic or DL traffic, so the relay UE can decide this in SRAP layer solely based on the SRAP mapping and SRAP header, w/o checking additional lower layer information.

Proposal 3	A “direction” bit to discern UL/DL is included in SRAP header.

## 2.4	SRAP operation in the absence of NW dedicated configuration

In legacy single-hop Layer-2 U2N relay design, SRAP is only used as a L2 relaying mechanism to transport end-to-end radio bearers, with the specific SRAP header identifying end-to-end bearer(s) and remote UE identity (i.e., local ID). It may be misunderstood that SRAP operation is only possible after dedicated RRC configuration (e.g., local ID assignment).  However, SRAP operation does not need to be limited to always happen after dedicated RRC configuration. In Rel-17 U2N relay design, forwarding operation in SL-RLC0 occurs in SRAP layer, but there is no any SRAP header involved. This is because SRB0 traffic are diverted into a fixed LCID (for SL-RLC0). With this way, RAN2 avoids the need to carry an SRAP header to indicate Uu SRB0.

In multi-hop U2N scenarios, there are plenty of circumstances to consider the SRAP operation before UE receiving dedicated RRC configuration. This may require SRAP entity to only identify remote UE ID of “end-to-end” traffic w/o bearer association or w/o local ID assigned. For R19 U2N procedures before any e2e DRB establishment (i.e., before NW providing dedicated SRAP configuration and dedicated PC5 Relay RLC channel), there is some essential need to transport control signalling message crossing the numerous PC5 hops (such as SI and Paging) in such a way. In those scenarios, instead of specifying exact UE behaviour based on the message contents/semantics (e.g., based on which SIB is requested by which remote UE), an address-based forwarding scheme can be very convenient.  This kind of forwarding can be enabled by tweaking the SRAP header design to contain a “Remote UE Layer-2 ID”, with UE ID and BEARER ID absent.

An example enhanced SRAP header design is shown in Figure 1.

Figure 1. enhanced Rel-19 SRAP header option (used in absence of dedicated NW configuration)

Note that 24-bit L2 ID will adds the signalling overhead for SRAP PDU. So, this is supposed to be only used for CP procedures crossing multiple hops, when NW has not provided dedicated SRAP configuration yet.

Proposal 4	Rel-19 support both legacy SRAP header (used after dedicated SRAP configuration is provided) and enhanced SRAP header containing remote UE L2 ID (used before dedicated SRAP configuration is provided).

## 2.5	SRAP Control PDU

Finally, we think although the SRAP control PDU is not used in legacy U2N relay design, it could be a useful tool in multi-hop relay scenario to piggyback some information among UEs along the multi-hop path. For example, after last relay UE has received the QoS split results from the gNB, to propagate the PDB split to each IDLE/INACTIVE intermediate relay UE, SRAP control PDU can be used instead of modifying PC5-RRC signalling.

Proposal 5	Support SRAP Control PDU as a light-weight alternative to PC5-RRC signalling to carry some control information in Rel-19 multi-hop U2N relay.

# 3 Conclusion

In this contribution, we discuss the SRAP design for Multi-hop SL relay, and we have the following proposals:

Proposal 1	Each Layer-2 Intermediate Relay UE has a single PC5 SRAP entity.

Proposal 2	For SRAP mapping in intermediate relay, a single set of PC5 Relay RLC channel configurations is used for both directions, whereas different SRAP mapping could be used in each direction.

Proposal 3	A “direction” bit to discern UL/DL is included in SRAP header.

Proposal 4	Rel-19 support both legacy SRAP header (used after dedicated SRAP configuration is provided) and enhanced SRAP header containing remote UE L2 ID (used before dedicated SRAP configuration is provided).

Proposal 5	Support SRAP Control PDU as a light-weight alternative to PC5-RRC signalling to carry some control information in Rel-19 multi-hop U2N relay.

# 4 References

[1] Chairman’s Notes, RAN2#128

[2] TS 38.331 v18.4.0