3GPP TSG-RAN WG2 Meeting #129	R2-2500435

# Athens, Greece, 17-21 February 2025

Agenda item:	8.0

Source:	Apple, Vodafone, InterDigital, Ericsson

Title:	RAN2 impact of WID ECRATU

WID/SID:	ECRATU – Release 19

Document for:	Discussion and Decision

# 1 Introduction

With the LS on UE usage of the RAT restrictions [1] (C1-247193/R2-2500004; contact: Apple) CT1 has asked RAN2 to study the impact of the WID ECRATU (Enhancement of controlling RAT utilization) on the RAN2 specifications.

The feature aims to reduce the network signaling load and service interruptions if an operator restricts the access to specific access technologies of its PLMN, as with the current NAS protocol the network needs to reject the registration with cause code #15 "no suitable cells in tracking area". Upon receiving #15 the UE stores the current TAI in the list of "forbidden tracking areas for roaming" and shall search for a suitable cell in another tracking area or in another location area. When the UE is moving into a new tracking area it might again select a cell of the restricted RAT and the network needs to reject the UE again with #15. This leads to unnecessary RAT changes and registration failures.

# 2 Discussion

# 2.1 NAS aspects

With ECRATU, CT1 has introduced a new MM IE RAT utilization control in TS 24.301 [2] section 9.9.3.3A:

#### 9.9.3.3A	RAT utilization control

The purpose of the RAT utilization control information element is for the network to communicate the restricted access technologies information to the UE.

The RAT utilization control information element is coded as shown in figure 9.9.3.3A.1 and table 9.9.3.3A.1.

The RAT utilization control is a type 4 information element.

Editor's note:	The specification of encoding RAT utilization control for roaming partner PLMNs is FFS.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 | 3 |  |  |  | 2 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RAT utilization control IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |
| Length of RAT utilization control contents |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |
| 0 spare | 0 spare |  | 0 Spare |  | 0 spare |  | 0 spare |  |  |  | 0 spare | Type of RAT utilization control |  |  |  | octet 3* |
| 0 spare | 0 spare |  | Sat-NG-RAN |  | Sat-E-UTRAN |  | NG-RAN |  |  | E-UTRAN |  | UTRAN |  | GERAN |  | octet 4* |

Figure 9.9.3.3A.1: RAT utilization control information element

Table 9.9.3.3A.1: RAT utilization control information element

| Type of RAT utilization control (octet 3, bit 1 to 2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 2 |  | 1 |  |  |
| 0 |  | 0 |  | current PLMN |
| 0 |  | 1 |  | current PLMN and its equivalent PLMN(s) |
| 1 |  | 0 |  | Unused, shall be interpreted as "current PLMN" if received by the UE |
| 1 |  | 1 |  | Unused, shall be interpreted as "current PLMN" if received by the UE |
| If Type of RAT utilization control = “00”, octet 4 applies to the current PLMN. If Type of RAT utilization control = “01”, octet 4 applies to the current PLMN and PLMN(s) included in Equivalent PLMNs IE, if any. |  |  |  |  |
| GERAN (octet 4, bit 1) |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | GERAN is not restricted |
| 1 |  |  |  | GERAN is restricted |
| UTRAN (octet 4, bit 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | UTRAN is not restricted |
| 1 |  |  |  | UTRAN is restricted |
| E-UTRAN (octet 4, bit 3) |  |  |  |  |
| Bit |  |  |  |  |
| 3 |  |  |  |  |
| 0 |  |  |  | E-UTRAN is not restricted |
| 1 |  |  |  | E-UTRAN is restricted |
| NG-RAN (octet 4, bit 4) |  |  |  |  |
| Bit 4 |  |  |  |  |
| 0 |  |  |  | NG-RAN is not restricted |
| 1 |  |  |  | NG-RAN is restricted |
| Sat-E-UTRAN (octet 4, bit 5) |  |  |  |  |
| Bit |  |  |  |  |
| 5 |  |  |  |  |
| 0 |  |  | satellite E-UTRAN is not restricted |  |
| 1 |  |  | satellite E-UTRAN is restricted |  |
| Sat-NG-RAN(octet 4, bit 6) |  |  |  |  |
| Bit 6 |  |  |  |  |
| 0 |  |  | satellite NG-RAN is not restricted |  |
| 1 |  |  | satellite NG-RAN is restricted |  |

With this IE the network can inform the UE about the applicable restricted access technologies information. This information is applicable to the "current PLMN" or "current PLMN and its equivalent PLMN(s)".

The information is stored in list of "PLMNs with associated RAT restrictions” which is a list of “PLMN" +"RAT utilization control IE” entries, i.e. once the RAT utilization control IE is received it is stored together with the PLMN which sent the IE in the list.

The network can include the RAT utilization control IE in the

- Registration Accept message (here the current access technology may not be marked as restricted)

- Registration Reject message together with cause code #15 (here the current access technology may be marked as restricted)

If a RAT is marked as restricted for a PLMN/EPLMN, the UE shall not consider the PLMN-access technology combination as a candidate for PLMN selection and shall not select any cell of the PLMN deployed in the restricted access technology during cell selection and re-selection for normal service.

As the network is aware of the applied access technology restrictions, the network will ensure in CONNECTED mode that the does not trigger a re-direction or a hand-over towards a cell deployed in the restricted access technology. Thus, there is no need that the UE updates the corresponding NAS and AS UE capabilities towards the network.

# AS aspects

While during PLMN selection NAS will ensure that a restricted PLMN-access technology combination is not selected, cells deployed in the restricted access technology might be selected by AS RRC without NAS interaction during out of service recovery or cell reselection.

In AS the criteria’s whether a UE may select a cell for normal services are defined in the term "Suitable Cell". The criteria for a suitable cell are a combination of parameters provided by NAS and AS parameters.

The NAS parameters are

- selected PLMN,

- EPLMN,

- "forbidden tracking areas for roaming" (#13 and #15 TAIs)

- allowed CSG/CAG IDs

In order to ensure that restricted PLMN-access technology combination is not selected, the NAS parameters for the suitable cell check need to be extended by the information of the applicable access technology restrictions which NAS maintains in the list of "PLMNs with associated RAT restrictions” and pass it to AS layer.

Access technology restrictions can be configured by the network for the following access technologies (access technology is defined in TS 23.122 [3]):

- GERAN

- UTRAN

- E-UTRAN (terrestrial cell)

- satellite E-UTRAN (non-terrestrial cell)

- NG-RAN (terrestrial cell)

- satellite NG-RAN (non-terrestrial cell)

As the definition of the suitable cell is defined for each radio access technology in separate specifications, the following 3GPP TS need to be changed: TS 43.022, TS 25.304, TS 36.304 and 38.304.

Also, as the “suitable cell” definition is to be changed, there would be also necessary modification of stage-2 TS, such as in section 9.2.1.1 “cell selection” of TS 38.300.

Note that the draftCRs [4][5][6][7] to TS 36.304, TS 38.304, TS 36.300, TS 38.300 are provided along with this paper to show the necessary changes to RAN2 specifications. CRs to GERAN/UTRAN are not under RAN2 responsibility, but similar changes are also expected.

Proposal 1	RAN2 confirms the AS layer impact of “RAT utilization control” for cell (re)selection procedure and agree the corresponding Rel-19 draftCR(s) for affected RAT(s).

# 3 Conclusion

In this contribution, based on the incoming CT1 LS, we discuss UE aspects on RAT utilization control and have the  following proposal:

Proposal 1	RAN2 confirms the AS layer impact of “RAT utilization control” for cell (re)selection procedure and agree the corresponding Rel-19 draftCR(s) for affected RAT(s).

# 4 References

[1] C1-247193 LS to RAN2 “UE usage of the RAT restrictions“

[2] TS 24.301 v19.1.0 Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3

[3] TS 23.122 Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode

[4] R2-2500436, Rel-19 draftCR TS 36.304, “Restriction on RAT utilization”

[5] R2-2500437, Rel-19 draftCR TS 36.384, “Restriction on RAT utilization”

[6] R2-2500967, Rel-19 draftCR TS 36.300, “Restriction on RAT utilization”

[7] R2-2500438, Rel-19 draftCR TS 38.300, “Restriction on RAT utilization”