3GPP TSG-RAN WG2 Meeting #129	R2-2500448

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.2.3

Source:	Panasonic

Title: 	Considerations for paging re-access on D2R failure

Document for:	Discussion

# Introduction

As per WID in [1], A-IoT work item phase in Rel-19 begins their discussions from the decisions already made in TR 38.769, with some refinements. Some of the RAN2 agreements related to random access and re-access triggering for D2R failure handling are listed below, as discussed during the study phase.

RAN2#127bis agreements - In case of D2R data transmission failure, device follows the reader instruction: - it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and - reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R). - Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data. - For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access. The re-access is always controlled by reader. - For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. - R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used. RAN2#128 agreements - RAN2 assumes, A-IoT reader can send a Msg2 transmission to a specific device for the random ID, if it doesn’t receive msg3 from that ID - If msg2 contains multiple devices, re-transmitted Msg2 will only include the random IDs of failed devices (if supported). - The "R2D failure/success feedback indication" also applies to "following D2R data" after the random access procedure for re-access purpose. - Capture the following option in TR: The devices can be triggered by R2D message to re-access before subsequent paging retransmissions.

This document discusses issues to be addressed for the failure scenarios of D2R transmissions as encountered by the device, including Msg3 transmission failure during random access procedure and further proposes few enhancements for the paging re-access triggering intended for such failure handling.

# Discussion

RAN2 has agreed upon a baseline procedure for A-IoT as seen from the table below, the data flow and the steps involved in R2D/ D2R depends on the type of service triggered for an A-IoT device, ‘inventory only’ or ‘inventory and command’

| Inventory | Step A | Initial trigger message or A-IoT paging |
| --- | --- | --- |
|  | Step B | Random access and transmission of device ID (FFS on details) |
| Command (followed by inventory) | Step C | Reader to device transmission (e.g. command in R2D) |
|  |  | Device to reader data transmission for the corresponding R2D triggered (e.g. response to the command in D2R) |

Inventory procedure can be executed as per Step A and Step B, while Command procedure in Step C is currently expected to be followed after the inventory operations in Steps A and B (since the details of command only procedure is yet to be discussed in RAN2). And the details for random access-like procedure in step B is further under discussion, with a longer 3-step format consisting of Msg1 and Msg3 transmitted on D2R and a shorter 2-step format consisting of only Msg1 sent in D2R. Further, from the current agreements it is implied that for each D2R message sent during inventory or command procedure, the reader can optionally send a feedback message in R2D to indicate the success or failure of the last transmitted data in D2R, which can accommodate along with new command operation triggered in R2D (i.e. command operation after completing step B, or continue with new command operation in step C).

Issues related to D2R transmission and explicit R2D feedback indication

The explicit feedback sent in R2D is an optional indication from the reader, which could benefit in some cases as it allows the reader to restrain from sending additional messages, for example, when no further messages are intended for the device as part of inventory or command operations. But the device not receiving this optional R2D feedback message after the last transmitted D2R data could mean that either the reader has completed the inventory or command procedure or the R2D message is lost and has failed to reach the device, possibly with a negative feedback indication to trigger re-access of the detected D2R failure, figure 1 illustrates one such failure scenario.

Figure 1: D2R failure illustration

The time duration for which devices wait for the feedback message could be proportional to the [D2R to R2D] max time specified for the A-IoT devices, which indicates the time duration where devices can expect the next corresponding R2D message following the D2R response.

Even during the random-access procedure when triggered to A-IoT devices, Msg3 transmission in D2R can encounter a similar failure situation as described in Figure 1. Furthermore, when multiple A-IoT devices are triggered to perform random access procedure, here the contention between those devices are expected to be resolved during Msg1 and Msg2 transmissions (example in a 3-Step random access), but it cannot be ruled out completely that two (or more) devices can choose the same random ID when transmitting Msg1 and the contention is resolved to all the A-IoT devices with this single random ID value echoed in Msg2 from the reader, see figure 2 for detailed illustration. Such situation can occur rarely in a larger group of devices, even though sufficient code space is available from the 16-bit random ID generation (the rarity shouldn’t be ignored).

It is a possible case where the reader detects these multiple messages from many A-IoT devices sent with single random ID value, but the reader cannot detect the number of devices transmitted on the same occasion. For example, if the transmitted information (with same values) from multiple devices traverses the same path and encountered less interference, then the receiver detects this information as a single message, therefore, the reader can send one Msg2 response upon detecting this random ID.

Figure 2: Msg3 failure during 3-step CBRA

As a consequence of resolving contention to multiple A-IoT devices using single Msg2 in R2D, if all the contention resolved devices sends Msg3 in D2R it is most likely to undergo contention again, as seen in figure 2. At this stage, assuming if the reader detects only some (or even none) of the Msg3 contents while the remaining (or all) be lost, for example if one device transmission power is received stronger than the others, then those devices which did not receive the follow-up R2D message (e.g. in step-C), could wait for a follow up message or a feedback in R2D indefinitely since it is not aware of the D2R failure. Also, here the reader will not be aware of the failed device to send explicit failure message in R2D to these devices.

Therefore, it is essential that the Msg3 failure detection is implicitly detected at the device, for example using a timer, to address all the failure conditions of random-access procedure.

Observation 1: A-IoT devices cannot rely completely on the explicit feedback message sent on R2D to trigger for re-accessing of paging occasions, due to failure scenarios with D2R transmission or R2D feedback messages or mislead contention resolution to A-IoT devices during a random-access procedure.

Proposal 1: Support implicit failure detection by A-IoT devices to perform re-access upon D2R transmission failures or Msg3 transmission failure, when paging re-access occasions are available from the reader.

Proposal 1a: A-IoT device performs implicit failure detection of D2R transmission when the follow-up R2D messages are not received from the reader after the D2R to R2D maximum time (T[D2R-R2D]max) defined for the device.

Proposal 2: When the reader intends the device stops implicit failure detection and re-access attempt, R2D success feedback indication should be sent to A-IoT device, within D2R to R2D maximum time, even if the reader does not intend to trigger subsequent R2D messages to the device (e.g. after completion of inventory or command procedure).

Further, if the reader detects the D2R message successfully (also applies to Msg3 in D2R) and wants to trigger a subsequent R2D message to continue services/ requests with the device, then the device can identify the success of the last transmitted D2R data using the follow-up messages triggered in R2D. Assuming the follow-up messages after the random access procedure will be addressed to the device using the device ID, and the devices receiving R2D messages addressed using its device ID can be used to determine success indication from the reader.

Proposal 3: Subsequent R2D messages received at the device following a D2R transmission can be used as the explicit success indication of the last transmitted D2R. Hence success indication in R2D is optional in case the reader schedules subsequent R2D messages.

Consideration for D2R re-transmission and implicit feedback detection

The implicit failure detection by A-IoT devices is especially important for handling re-transmission of D2R data which can be triggered by the reader, both for the case of D2R segment re-transmissions or for the Msg3 re-transmission, which are described further. Since RAN2 is discussing segmentation for D2R data after the random access procedure in a parallel agenda item in A-IoT, and to support segment re-transmission from devices to recover part of the data segments when not received at the reader is another aspect to consider. For such cases the device must store the last transmitted D2R messages within re-transmission buffer (at least at the upper layer) to facilitate re-transmission when requested by the reader. But it is not understood if the device should store this re-transmission data buffer until the paging re-access is triggered in the next opportunity or whether it can be cleared earlier, and later if the device does not receive a follow-up R2D message or a feedback message in R2D then it can imply that the D2R transmission has failed (also discussed in the previous section). Figure 3 shows one example where the re-transmission buffer is cleared after the D2R to R2D maximum duration defined for the device, which can also trigger the device to perform implicit failure detection to re-access in another opportunity.

Figure 3: Re-transmission buffer not cleared when device expects explicit feedback indication from reader

RAN2 also assumes A-IoT reader can send Msg2 transmission to a specific device in case the Msg3 is not found/decoded from the device on the D2R occasion scheduled for the corresponding random ID, repeated as part of re-transmission requests triggered by the reader to handle D2R failure scenarios. Which means it is assumed that the device can store Msg3 data in a re-transmission buffer, to repeat transmission on D2R when requested by the reader.

Observation 2: Implicit failure detection as seen in proposal 1 and 1a is useful for D2R transmission failures when re-transmission in D2R is supported, where the device stores re-transmission buffer for some time before it implicitly detects to re-access in another opportunity if no R2D feedback is received from the reader.

# Conclusions

Observation 1: A-IoT devices cannot rely completely on the explicit feedback message sent on R2D to trigger for re-accessing of paging occasions, due to failure scenarios with D2R transmission or R2D feedback messages or mislead contention resolution to A-IoT devices during a random-access procedure.

Proposal 1: Support implicit failure detection by A-IoT devices to perform re-access upon D2R transmission failures or Msg3 transmission failure, when paging re-access occasions are available from the reader.

Proposal 1a: A-IoT device performs implicit failure detection of D2R transmission when the follow-up R2D messages are not received from the reader after the D2R to R2D maximum time (T[D2R-R2D]max) defined for the device.

Proposal 2: When the reader intends the device stops implicit failure detection and re-access attempt, R2D success feedback indication should be sent to A-IoT device, within D2R to R2D maximum time, even if the reader does not intend to trigger subsequent R2D messages to the device (e.g. after completion of inventory or command procedure).

Proposal 3: Subsequent R2D messages received at the device following a D2R transmission can be used as the explicit success indication of the last transmitted D2R. Hence success indication in R2D is optional in case the reader schedules subsequent R2D messages.

Observation 2: Implicit failure detection as seen in proposal 1 and 1a is useful for D2R transmission failures when re-transmission in D2R is supported, where the device stores re-transmission buffer for some time before it implicitly detects to re-access in another opportunity if no R2D feedback is received from the reader.

# References

[1] RP-243312; New Work Item: Solutions for Ambient IoT (Internet of Things) in NR.