3GPP TSG-RAN WG2 #129                                             	    R2-2500485

Athens, Greece, Februrary 17th –21st, 2025

Agenda item:	   8.6.3

Source: 		Qualcomm Incorporated

Title: 	L1 event-triggered measurement reporting for LTM

Document for:	Discussion

# 1	Introduction

RAN2#128 has agreed on the following for L1 event-triggered measurements for LTM [1]:

Agreements on L1 event-driven MR - TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. - TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam. - TTT is applied to the leaving condition. - Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation. - Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1. - SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE. - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Support the truncated measurement report MAC CE.

This paper discusses further aspects related to L1 event-triggered measurement report enhancements for LTM.

# 2	CSI-RS measurement resource configuration

In RAN1#118, it was agreed to support periodic CSI-RS for both gNB scheduled reporting and event-triggered reporting for LTM. In RAN1#118bis, the support of semi-persistent CSI-RS became a working assumption and the LS about its feasibility for gNB scheduled LTM CSI report was sent to RAN3. The reply LS from RAN3 [2] indicated that specifying the signalling for coordination between serving cell and candidate cell(s) is feasible.

Observation 1: Based on RAN1 decisions and RAN3 response about semi-persistent CSI-RS for LTM CSI report, semi-persistent CSI-RS for event-triggered L1 measurement report for LTM can be also supported.

Proposal 1: Support both periodic and semi-persistent CSI-RS as a baseline for CSI measurement resource for event-triggered L1 measurement report for LTM. FFS on activation/deactivation MAC CE and UE procedures.

In RAN2#127b, it was agreed to reuse Rel-18 LTM CSI resource configuration as a baseline for event-triggered L1 measurement report for LTM. But in RAN1#119, the discussion on supporting the legacy CSI-RS resource for mobility for L1 measurement in LTM had the following outcome:

FL summary 3 of Measurements related enhancements for LTM from Thursday session [3] - No consensus to support CSI-RS for mobility for L1 measurement in Rel-19 LTM. - Note: From the actual gNB transmission viewpoint, CSI-RS for mobility and CSI-RS for BM may be the same.

Even though the legacy CSI-RS for mobility for L3 measurement is same as NZP-CSI-RS for L1 measurement from the DL signal transmission perspective, both are quite different from RRC signaling point of view. Thusmixing these two different CSI-RS resource indices for LTM measurement resource signaling could create unnecessary complications. It would be much simpler to use Rel-18 LTM CSI resource configuration for both CSI-RS based measurement and event-triggered L1 measurement report for LTM.

Observation 2: Re-using legacy CSI-RS resource for L3 mobility and measurement as also LTM CSI measurement resource would create too much complications from RRC signaling perspective.

Proposal 2: RAN2 to agree to use L1 CSI resource configuration (i.e., either the enhanced Rel-18 LTM CSI resource (i.e. LTM-CSI-ResourceConfigId-r18) or a new Rel-19 LTM CSI resource) for both CSI-RS based measurement and event-triggered L1 measurement report for LTM.

# 3	LTM event evaluation

## 3.1	Per-beam event evaluation

In RAN2#127, it was agreed that any beam in the candidate RS configuration can be used for event evaluation. And in RAN2#128, it was agreed that TTT is evaluated per beam, and the measurement report is only triggered by the beam that has satisfied the condition (entering/leaving) for the whole duration of TTT. Thus, these imply that the event evaluation would need to be done per-beam level, and it’s also possible that multiple beams meet the event condition around the same time. For a particular event, if multiple beams satisfy the condition for measurement report, then UE doesn’t need to trigger multiple measurement reports corresponding to multiple triggering beams. Instead, a single measurement report can be generated with multiple triggering beams included. This is also in line with RAN2#128 agreement of max N number of beams included in measurement report MAC CE.

Observation 3: Previous RAN2 agreements imply that the event evaluation is performed per-beam level, and consequently there can be multiple beams that satisfy the event condition around the same time.

When multiple beams trigger the event condition for the same event at or around the same time, the UE can select one or more of the beams and send the report.

Proposal 3: The event evaluation is performed at a per-beam level. When multiple beams fulfills the condition for a particular event, the UE sends a single L1 measurement report with one or more beam information for that event.

## 3.2	Prohibit timer

As discussed in the previous section, when the event is evaluated per-beam level, it is possible that another beam satisfies the event condition, shortly after the measurement report for that event has been transmitted. Even though it is possible to let UE trigger a new measurement report whenever the event condition is satisfied, it may lead to too many event-triggered L1 measurement reports for each event configuration. And as more event configurations provided to UE, the overall number of event-triggered reports could be unnecessarily large.

To achieve timely event-triggered but less frequent report transmissions, a new event-triggered measurement report can be only triggered after a “sufficient amount of time” has passed since the last measurement report for the same event. This sufficient amount of time can be controlled by a prohibit timer which starts after the measurement report for the triggered event is sent, and while this timer is running, no new measurement report can be triggered. Triggering event-triggered measurement reports for every triggering beam can be also supported by setting this prohibit timer value to zero.

Observation 4: Per-beam level event evaluation may lead to an event condition being met by different beams several times with a short time interval. Triggering the event-triggered measurement reports in each time instance could lead to too many event-triggered reports transmitted. Therefore, it would be beneficial to control the amount of time between two consecutive event-triggered measurement reports for a particular event.

Proposal 4a: RAN2 to introduce a prohibit timer with the following behavior:

- The timer starts after an event-triggered measurement report is triggered based on fulfilling an event condition.

- While the timer is running, no new event-triggered measurement report can be reported at least for the same event.

Proposal 4b: RAN2 to discuss whether the granularity of configuration of a prohibit timer for an event can be at

- Option (a): per event

- Option (b): per candidate cell

- Option (c): per beam

- Option (d): Option (a) through Option (c) based on configuration

## 3.3	Different HYS/TTT values for leaving condition

In RAN2#127b, it was agreed that a measurement report can be sent when the leaving condition is met based on NW configuration, and in RAN2#128, it was agreed that TTT is applied to the leaving condition.

In L3 event-based measurement reports, one set of HYS and TTT values are configured per event configuration, and they are applied to both entering and leaving conditions as illustrated in Figure 1. Applying the same HYS and TTT values to both entering and leaving conditions wouldn’t make much difference in triggering the L3 measurement reports and subsequent L3 HOs because (i) the cell metric is used for the evaluation hence the metric is less sensitive to the beam level metric differences, (ii) the measurement report through RRC signaling has a larger time scale than lower layer signaling (UCI or event-triggered L1 measurement MAC CE), and (iii) L3 HO has a longer overall switching latency than LTM switch (i.e., from the measurement report trigger to the cell switch complete).

Observation 5a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Figure 1. Illustration of entering and leaving condition in L3 measurement Event A3

In event-triggered L1 measurement reports for LTM, however, the situation becomes different. For a seamless LTM cell switch, it is crucial to keep track of the best candidate beam among those who fulfilled the entering condition for the potential cell switch so that serving cell can perform the candidate TCI state activation and/or early UL synchronization with that chosen beam. While a candidate beam satisfying the entering condition can be chosen conservatively via HYS/TTT, any candidate beam that is no longer good for cell switch (i.e., leaving condition) should be identified by serving cell through a measurement report as quickly as possible. In this aspect, applying the same HYS and TTT could delay the leaving condition to be triggered significantly.

Observation 5b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

In order to expedite the event-triggered L1 measurement report for leaving condition, there can be two options to be considered:

- Option 1: Keep one set of HYS and TTT for both entering and leaving conditions, and set them to smaller values

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

Option 1 can be the simplest solution without any specification impact, but the smaller HYS and TTT values would impact the entering condition to be triggered quicker as well, which could lead to unexpectedly frequent measurement reports triggered due to potential noisy L1 measurements (without any spec-defined filtering applied). Both Option 2 and 3 provide the configurability of different HYS and/or TTT values between entering and leaving conditions, but Option 3 could be preferrable due to its flexibility in not providing additional HYS and/or TTT for leaving condition if NW chooses not to.

Observation 5c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 5: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

# 4	Measurement report MAC CE

## 4.1	Report configuration

RAN#127 agreed that the event-triggered report configuration should be provided in the serving cell configuration. This agreement can be evaluated by considering the scenario where the UE is configured with CA. In this case, different UE behaviours can be considered:

- Option A: UE has one report configuration per serving cell group:

  - Option A1: UE may only send the MAC CE report via the PCell.

  - Option A2: UE may send the MAC CE report via any serving carrier.

- Option B: UE may have one report configuration per carrier, and UE issues MAC CE reports on a carrier using the respective report configuration.

Observation 6a: For a UE configured with CA, the following options are possible:

- Option A: UE has one report configuration per serving cell group

- Option B: UE may have one report configuration per serving carrier, and UE sends a MAC CE report on any carrier using the respective report configuration of that carrier.

Since it is the same serving gNB-DU that would receive the MAC CE reports regardless of which carrier is used to carry the report, one report configuration (Option A) is sufficient. This implies that the event-triggered report configuration does not need to be inside the serving cell configuration but rather may be within the CellGroupConfig (reverting the prior agreement).

Observation 6b: All MAC CE reports issued on any carrier of the same serving cell group are received by the same gNB-DU; therefore multiple report configurations are not needed.

Proposal 6a: UE receives one event-triggered report configuration for the serving cell group.

Proposal 6b: Event-triggered report configuration is within CellGroupConfig but outside the serving cell configuration (reverting prior RAN2 agreement).

## 4.2	Report content

### 4.2.1	Non-event triggering beam information

Regarding non-event triggering beams to be reported via measurement report MAC CE, RAN2#128 agreed that max N is the total number of all beams included into measurement report MAC CE, and via the value of N configuration, NW controls if the beam(s) not satisfying the event could be reported or not in MR MAC CE.

To our understanding, NW cannot fully control via N whether the beams not satisfying the event can be reported or not. As a simple example, assume that N is configured to be 2 and only 1 beam has triggered the event condition. In this situation, the value of N doesn’t provide any further information on whether UE can include another beam that didn’t satisfy the event condition into the measurement report MAC CE or not. The only scenario where the value N can control whether beams not satisfying the event can be reported or not is when N is set to 1, which means only single event triggering beam can be reported and non-event triggering beams cannot be included at all.

Observation 7a: The previous agreement on controlling via the value N whether beams not satisfying the event can be included in measurement report MAC CE can only work when the value N is set to 1, i.e., only single triggering beam is reported in measurement report MAC CE.

Even though the main purpose of the event-triggered measurement report is to know which beams have triggered the event and their measurement quantity, having additional information about other non-event triggering beams would be beneficial for serving cell to be prepared for further actions. When the value N is configured to UE, serving cell is expected to allocate the PUSCH resource to accommodate up to N beam reporting through measurement report MAC CE upon receiving a dedicated SR for event-triggered L1 measurement report for LTM. In this case, unless UE has any other logical channel data (MAC CE or ULSCH data), the allocated PUSCH payload would have to be filled with padding bits unnecessarily if we decide to include only event-triggering beams to measurement report MAC CE.

Observation 7b: For a configured value N (the maximum number of beams to be included in measurement report MAC CE), NW would try to schedule the PUSCH resource to accommodate the whole N beam information due to the lack of the exact number of beams that triggered the event. By sending the non-triggering beam information as well as the triggering beam(s), the allocated PUSCH resource can be used more efficiently and NW could get more information about other non-triggering beams for any further preparation.

Proposal 7: For the configured max N total number of beams, UE can include up to N-M non-triggering beams in the measurement report MAC CE where M is the number of beams satisfying the event condition. FFS on how to distinguish between triggering and non-triggering beams in the MAC CE.

### 4.2.2	L1-RSRP change rate information

In RAN2#127b, L1-RSRP was agreed to be included as the beam quantity in event-triggered L1 measurement report for LTM. While reporting the ‘snapshot’ of measurement results (filtered or unfiltered) has been traditionally adopted as the beam quantity included in L1 or L3 measurement reports, the lack of additional information such as the change rate of measurement quantity change has brought several challenges (e.g., handover failures and ping-pong handovers) in real deployment scenarios.

Observation 8a: The traditional measurement report with the ‘snapshot’ of measurement results can cause mobility problems such as handover failures or ping-pong handovers.

In Figure 2 below, the left and right figures show two different expectations after LTM execution which is assumed to be done based on the event-triggered measurement report from UE. If UE can include additional information of the L1-RSRP change rate (or “slope”) along with the L1-RSRP value, serving cell could make the better choice for the candidate beam & candidate cell for the LTM cell switch.

Figure 2. (left) negative L1-RSRP slope and (right) positive L1-RSRP slope in event-triggered L1 measurement report MAC CE transmission

Observation 8b: Additional signalling of the L1-RSRP change rate (or “slope”) can help serving cell choose the candidate beam that is expected to remain good after LTM cell switch.

Proposal 8: RAN2 to discuss adding the slope information to the event-triggered L1 measurement report MAC CE.

## 4.3	Truncated MAC CE

In RAN2#128, it was agreed to support the truncated MAC CE for event-triggered L1 measurement report. The truncated MAC CE has been supported for several MAC CEs (e.g., BSR or (Enhanced) BFR), and the main purpose of truncation is to send the essential information back to NW while the available payload size is not sufficient to accommodate the full MAC CE payload with the subheader. And the same purpose would hold true for event-triggered L1 measurement report MAC CE for LTM.

Observation 9: Similar to the existing truncated MAC CEs for BSR or (Enhanced) BFR, the truncated event-triggered L1 measurement report MAC CE is expected to be used when the allocated PUSCH resource is not sufficient enough to cover the entire measurement report payload from all triggered events.

Proposal 9: Truncated event-triggered L1 measurement report MAC CE is used when the available payload is not sufficient to accommodate the full MAC CE report with all triggered events plus subheader.

The truncation of event-triggered L1 measurement report MAC CE can be discussed in two different levels.

- Event-level truncation

  - Since NW can configure multiple event configurations with different sets of candidate beams and/or different event types, it should be noted that there can be multiple events to be transmitted through the same PUSCH allocation. And when the available payload size is not enough for all triggered events, then some events would need to be dropped. The dropping can be done via a prioritization rule among events. Other dropping rules are FFS.

- Content-level truncation

  - Even after the event-level truncation, it’s still possible that the available payload size is not sufficient to accommodate the full reports from one or more events. This is the similar case as truncated BSR or (Enhanced) BFR, where Buffer Size information of some Logical Channel Group IDs in BSR or octets containing the AC field in (Enhanced) BFR can be dropped. Similarly, some fields and/or values can be omitted in the content-level truncation for event-triggered L1 measurement MAC CE.

Observation 10a: Due to the multiple event configurations configured for UE, the truncation rule among triggered events needs to be discussed (called event-level truncation).

Observation 10b: Similar to the truncation case from BSR or (Enhanced) BFR, the truncation rule among fields and/or values within a triggered event needs to be discussed (called content-level truncation).

Proposal 10: RAN2 to discuss two level truncations as a baseline for truncated event-triggered L1 measurement report MAC CE:

- Event-level truncation

- Content-level truncation

# Conclusion

This contribution discussed aspects related to measurement enhancements for LTM. The following observations and proposals have been made:

Observation 1: Based on RAN1 decisions and RAN3 response about semi-persistent CSI-RS for LTM CSI report, semi-persistent CSI-RS for event-triggered L1 measurement report for LTM can be also supported.

Proposal 1: Support both periodic and semi-persistent CSI-RS as a baseline for CSI measurement resource for event-triggered L1 measurement report for LTM. FFS on activation/deactivation MAC CE and UE procedures.

Observation 2: Re-using legacy CSI-RS resource for L3 mobility and measurement as also LTM CSI measurement resource would create too much complications from RRC signaling perspective.

Proposal 2: RAN2 to agree to use L1 CSI resource configuration (i.e., either the enhanced Rel-18 LTM CSI resource (i.e. LTM-CSI-ResourceConfigId-r18) or a new Rel-19 LTM CSI resource) for both CSI-RS based measurement and event-triggered L1 measurement report for LTM.

Observation 3: Previous RAN2 agreements imply that the event evaluation is performed per-beam level, and consequently there can be multiple beams that satisfy the event condition around the same time.

Proposal 3: The event evaluation is performed at a per-beam level. When multiple beams fulfills the condition for a particular event, the UE sends a single L1 measurement report with one or more beam information for that event.

Observation 4: Per-beam level event evaluation may lead to an event condition being met by different beams several times with a short time interval. Triggering the event-triggered measurement reports in each time instance could lead to too many event-triggered reports transmitted. Therefore, it would be beneficial to control the amount of time between two consecutive event-triggered measurement reports for a particular event.

Proposal 4a: RAN2 to introduce a prohibit timer with the following behavior:

- The timer starts after an event-triggered measurement report is triggered based on fulfilling an event condition.

- While the timer is running, no new event-triggered measurement report can be reported at least for the same event.

Proposal 4b: RAN2 to discuss whether the granularity of configuration of a prohibit timer for an event can be at

- Option (a): per event

- Option (b): per candidate cell

- Option (c): per beam

- Option (d): Option (a) through Option (c) based on configuration

Observation 5a: For L3 measurement report, the same HYS and TTT are applied to both entering and leaving conditions. Since L3 measurement report and HO deal with the cell level measurement (not beam level), it won’t be sensitive to identifying the leaving condition quicker.

Observation 5b: For L1 measurement report, keeping track of the best candidate beam is crucial due to the possible TCI state activation and/or early UL synchronization. Therefore, any candidate beam that is not satisfying the better quality than the serving beam (i.e., leaving condition) needs to be identified as quickly as possible.

Observation 5c: In order to achieve rather conservative entering condition but a bit aggressive leaving condition, separate configurations of HYS and/or TTT for leaving condition is desirable.

Proposal 5: Support the configurability of different HYS and/or TTT values between entering and leaving conditions with following options:

- Option 2: Keep two sets of HYS and TTT, one for entering condition and the other for leaving condition

- Option 3: Keep one set of HYS and TTT for both entering and leaving conditions as a baseline, but NW can optionally configure another set of HYS and/or TTT for leaving condition (i.e., HYS and/or TTT for leaving condition can be overridden by additional configuration, if provided)

Observation 6a: For a UE configured with CA, the following options are possible:

- Option A: UE has one report configuration per serving cell group

- Option B: UE may have one report configuration per serving carrier, and UE sends a MAC CE report on any carrier using the respective report configuration of that carrier.

Observation 6b: All MAC CE reports issued on any carrier of the same serving cell group are received by the same gNB-DU; therefore multiple report configurations are not needed.

Proposal 6a: UE receives one event-triggered report configuration for the serving cell group.

Proposal 6b: Event-triggered report configuration is within CellGroupConfig but outside the serving cell configuration (reverting prior RAN2 agreement).

Observation 7a: The previous agreement on controlling via the value N whether beams not satisfying the event can be included in measurement report MAC CE can only work when the value N is set to 1, i.e., only single triggering beam is reported in measurement report MAC CE.

Observation 7b: For a configured value N (the maximum number of beams to be included in measurement report MAC CE), NW would try to schedule the PUSCH resource to accommodate the whole N beam information due to the lack of the exact number of beams that triggered the event. By sending the non-triggering beam information as well as the triggering beam(s), the allocated PUSCH resource can be used more efficiently and NW could get more information about other non-triggering beams for any further preparation.

Proposal 7: For the configured max N total number of beams, UE can include up to N-M non-triggering beams in the measurement report MAC CE where M is the number of beams satisfying the event condition. FFS on how to distinguish between triggering and non-triggering beams in the MAC CE.

Observation 8a: The traditional measurement report with the ‘snapshot’ of measurement results can cause mobility problems such as handover failures or ping-pong handovers.

Observation 8b: Additional signalling of the L1-RSRP change rate (or “slope”) can help serving cell choose the candidate beam that is expected to remain good after LTM cell switch.

Proposal 8: RAN2 to discuss adding the slope information to the event-triggered L1 measurement report MAC CE.

Observation 9: Similar to the existing truncated MAC CEs for BSR or (Enhanced) BFR, the truncated event-triggered L1 measurement report MAC CE is expected to be used when the allocated PUSCH resource is not sufficient enough to cover the entire measurement report payload from all triggered events.

Proposal 9: Truncated event-triggered L1 measurement report MAC CE is used when the available payload is not sufficient to accommodate the full MAC CE report with all triggered events plus subheader.

Observation 10a: Due to the multiple event configurations configured for UE, the truncation rule among triggered events needs to be discussed (called event-level truncation).

Observation 10b: Similar to the truncation case from BSR or (Enhanced) BFR, the truncation rule among fields and/or values within a triggered event needs to be discussed (called content-level truncation).

Proposal 10: RAN2 to discuss two level truncations as a baseline for truncated event-triggered L1 measurement report MAC CE:

- Event-level truncation

- Content-level truncation

# References

- Chairman Notes; TSG RAN WG2 Meeting #128

- R3-247911, Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells, CATT

- R1-2410188, FL summary 3 of Measurements related enhancements for LTM, Fujitsu