3GPP TSG-RAN WG2 #129                                             	    R2-2500486

Athens, Greece, February 17th –21st, 2025

Agenda item:	   8.6.4

Source: 		Qualcomm Incorporated

Title: 	Conditional intra-CU LTM

Document for:	Discussion

# 1	Introduction

The WID for mobility enhancements includes the following objective [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

RAN2#128 agreed the following [2]:

Agreements on C-LTM - The triggering condition of conditional LTM can be based on L3 measurement. - CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. - The L1 execution condition of a candidate cell is associated to only one triggering event. - For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. - To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. - The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer. - For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

RAN2#127b agreed the following [3]:

Agreements on C-LTM - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

This paper discusses further aspects related to conditional intra-CU LTM.

# 2	LTM & C-LTM coexistence

In L3 mobility, a UE may receive an L3 HO command towards a candidate cell for which the UE has previously received a stored CHO configuration. Similarly, the network should be allowed to trigger LTM towards a target cell that is a C-LTM candidate cell. This implies that C-LTM candidate cells are a subset of LTM candidate cells.

Observation 1: In L3 mobility, the network may trigger L3 HO towards a CHO candidate cell.

Proposal 1a: C-LTM candidate cells are a subset of LTM candidate cells. Network-triggered LTM towards a C-LTM candidate cell is always supported.

The UE determines which subset of the LTM candidate cells are also C-LTM candidate cells based on configuration. Specifically, if the LTM candidate cell configuration also includes configuration for C-LTM condition evaluation, then the LTM candidate cell is also a C-LTM candidate cell.

Proposal 1b: A UE identifies an LTM candidate cell as a C-LTM candidate cell if the LTM candidate cell is included in a configuration for C-LTM condition evaluation.

# 3	C-LTM event evaluation

In Rel-18 LTM, the cell switch latency reduction is achieved by performing early candidate TCI state activation and early UL synchronization for the best candidate beam identified through L1 or L3 measurement reports. Taking the Rel-18 LTM as a reference, the following principles can be considered for C-LTM event evaluation:

- Beam level evaluation (not cell level): As agreed in event-triggered L1 measurement report for LTM from RAN2#128, TTT is evaluated per beam and the measurement report is triggered by a beam that has satisfied the condition (entering/leaving) with HYS for the whole TTT duration. Hence, for Event LTM3-like or LTM5-like conditions, the evaluation of each candidate beam and the corresponding C-LTM trigger would be a sensible solution.

  - Unified design between L1 and L3: Having the beam level evaluation as a baseline, for L1-based C-LTM evaluation, L1-RSRP value is used (filtering is up to RAN1 decision). And for L3-based C-LTM evaluation, filtered beam-level RSRP is used.

- Single beam-based C-LTM trigger: when NW-triggered LTM or C-LTM is triggered, the beam to be used for RACH or UL transmission (RACH-less case) is just one, not multiple. This means no matter how many other good candidate beams there are for that candidate cell, one beam eventually needs to be selected and used for the (C-)LTM execution. Waiting for more than one candidate beam to satisfy the C-LTM event condition for a particular candidate cell may provide a higher reliability after C-LTM execution, but this in turn could delay the C-LTM execution timing significantly, which leads to too late cell switch situation. The reliability issue of the selected candidate beam can be guaranteed via HYS and TTT configuration. Therefore, the C-LTM execution needs to be triggered when at least one beam fulfils the C-LTM event condition. When multiple beams satisfy the condition simultaneously, it can be up to UE implementation.

Observation 2: Following the Rel-18 LTM principle (i.e., TCI state activation, early UL synchronization, LTM cell switch command based on a target candidate beam), the beam level evaluation should be the baseline for L1 based C-LTM event evaluation.

Proposal 2: For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

# 4	RACH-less C-LTM

## 4.1	Beam selection for CG

In the previous RAN2#128 meeting, one of the discussion items was the beam selection for RACH-less C-LTM with CG transmission. Looking back the Rel-18 LTM procedure, the decision of RACH-less LTM upon receiving LTM Cell Switch Command MAC CE is based on whether UE has a valid TA value, i.e., either (i) receiving a Timing Advance Command value not set to FFF via LTM Cell Switch Command MAC CE, or (ii) UE is configured with UE-based Timing Advance measurement and the UE has successfully measured the Timing Advance for the candidate cell.

As already discussed in the previous section, the candidate beam to be used for C-LTM execution procedure shall be the one that has triggered the C-LTM event condition(s), which would provide the best channel quality after the cell switch. Taking this into account along with the Rel-18 LTM procedure, we can conclude that RACH-less C-LTM with CG transmission can be done when the valid TA is available for the triggering candidate beam for which C-LTM execution condition(s) is satisfied. It should be noted that the candidate beam here refers to any beam from candidate RS set, irrespective of pre-TCI state activation or early UL synchronization. Figure 1 illustrates the flow chart for determining between RACH-less and RACH-based C-LTM.

Choosing the candidate beam only among those whose TCI state have been activated and/or early TA values are available could reduce the overall LTM execution latency, but it could cause the non-optimal candidate beam/cell selection which may lead to the cell switch failure. Or, performing RACH-less C-LTM only when UE has activated the TCI state for the candidate beam won’t be necessary, because Rel-18 LTM already supports the similar execution, i.e., RACH-less LTM with TCI state for which pre-activation is not performed.

Figure 1. RACH-less C-LTM decision procedure

Observation 3: In order to provide the best channel quality after C-LTM cell switch, the candidate beam for C-LTM execution should be the one that has triggered the event condition, irrespective of TCI state being activated or early UL synchronization performed for a candidate beam.

Proposal 3: When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set), UE performs RACH-less C-LTM using CG transmission with that triggered beam if UE has a valid TA for the associated candidate cell (i.e., either UE has received a TA and a TA timer is still running or UE has successfully measured a TA). Otherwise, UE performs RACH-based C-LTM with the triggered beam.

## 4.2	Early C-LTM target cell notification

When it comes to CG-based first UL transmission on target cell for RACH-less C-LTM, due to the unknown C-LTM cell switch timing to both serving cell and multiple candidate cells, CG time/frequency resources would have to be reserved earlier than actual C-LTM timing and most of them would be left unused, i.e., those CG resources cannot be used for other PUSCH allocations due to the potential collision with C-LTM UE’s first UL transmission. As more potential target candidate cells configure such CG resources, the overall resource inefficiency could become even more severe.

Therefore, it would be beneficial to introduce a new signaling procedure (“early target cell notification”) for UE to notify the potential target cell(s) and beam(s) for C-LTM to the serving cell such that the candidate cells can start monitoring CG resources only for a certain time period after receiving the notification message from the serving cell. This procedure would allow other candidate cells (which were not selected) to reuse the configured CG resources for other PUSCH allocations without any collision issue.

Observation 4: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 4: For RACH-less C-LTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target C-LTM cell(s) and beam(s). Detailed signaling/condition are FFS.

## 4.3	DG support

From the resource utilization point of view, DG-based RACH-less C-LTM has an advantage over CG-based approach since the PUSCH allocation can be done in a fully dynamic manner (i.e., without any pre-allocation of PUSCH resources). However, the biggest challenge with the DG-based approach is the uncertainty of C-LTM timing due to which a target candidate cell doesn’t know when DG allocation needs to be made for C-LTM UE. Additionally, the uncertainty of beam (or TCI state) to be used for C-LTM is another challenge. In order for DG-based RACH-less C-LTM to work, there should be a closer interaction between serving cell and candidate cells.

With the early notification mechanism discussed in Proposal 11a, the uncertainty of the timing and the beam for DG allocations can be significantly alleviated. Once the target candidate cell receives the notification from the serving cell, then it can start sending DG allocations for C-LTM UE for a certain time period.

Observation 5: DG-based RACH-less C-LTM can be more efficient in resource allocation compared to CG-based RACH-less C-LTM, but the uncertainty of execution time and selected beam makes DG scheduling more challenging. However, the early target cell notification in Proposal 11 can alleviate this uncertainty significantly.

Proposal 5: Support DG-based RACH-less C-LTM.

# 5	Early TA signalling and maintenance

In RAN2#128, it was agreed that the candidate cell’s TA information to UE is informed via new MAC CE. There was also a discussion on the UE request information to assist serving cell to trigger PDCCH ordered RACH, but it was decided that serving cell can utilize the existing measurement reports to decide the PDCCH ordered RACH without any UE’s assistance.

## 5.1	New TA MAC CE

Without a UE assistance information, serving cell may need to prepare multiple PDCCH ordered RACH procedures towards multiple candidate cells to guarantee RACH-less C-LTM, and this means multiple TA values from those multiple candidate cells would need to be sent to UE. For the sake of efficient signalling of multiple TA values, the new TA MAC CE should be able to include more than one TA values.

Observation 6: In C-LTM without any UE assistance information for early TA acquisition, serving cell may need to trigger multiple PDCCH ordered RACH procedures towards multiple candidate cells, and those multiple TA values would need to be sent to UE.

Proposal 6: New TA MAC CE should support one or more TA values signalling.

## 5.2	Candidate cell TA group

For the successful RACH-less C-LTM, it is necessary for UE to have a valid TA value for the target candidate cell. Through measurement reports, serving cell would be able to roughly know which candidate cells are potentially target candidate cells for C-LTM. However, if the number of those candidate cells is not small enough, these multiple PDCCH ordered RACH procedures would become overhead such that DL and/or UL performance would degrade due to interruptions.

To alleviate the PDCCH ordered RACH overhead issue, the candidate cell TA group can be considered where UE can assume an acquired TA value from one candidate cell (either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells within the TA group. By adopting this TA group, serving cell can trigger only a few PDCCH ordered RACH procedures for all candidate cells within the TA group, hence the RACH overhead could be significantly reduced.

Observation 7: For successful RACH-less C-LTM, serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could become huge overhead and interruptions in data transmission and reception at UE.

Proposal 7: RAN2 to discuss the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

## 5.3	TA maintenance

In RAN2#128, it was agreed that the candidate cell TA is maintained by a timer, and more detailed procedures would need to be discussed. The TA validation procedure at UE has been discussed in other topics before such as CG-SDT transmission or SRS transmission for positioning, and the existing validation procedure can be used as a baseline.

According to Section 5.72.2 in TS 38.321 MAC specification, the following procedure is defined for the TA validation for CG-SDT.

The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running

From above three conditions, the second (RSRP change threshold) and the third (TA timer) would be relevant for C-LTM. The first condition (valid RSRP) is to cover UE in RRC_INACTIVE state, so it is not necessary for C-LTM.

In addition to the TA timer that has been already agreed, checking the RSRP change (L1 or L3) can be used to track UE’s pathloss change which impacts the propagation latency.

Observation 8: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in C-LTM execution.

Proposal 8: RAN2 to agree the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

# Conclusion

This contribution discussed aspects related to conditional intra-gNB LTM. The following observations and proposals have been made:

Observation 1: In L3 mobility, the network may trigger L3 HO towards a CHO candidate cell.

Proposal 1a: C-LTM candidate cells are a subset of LTM candidate cells. Network-triggered LTM towards a C-LTM candidate cell is always supported.

Proposal 1b: A UE identifies an LTM candidate cell as a C-LTM candidate cell if the LTM candidate cell is included in a configuration for C-LTM condition evaluation.

Observation 2: Following the Rel-18 LTM principle (i.e., TCI state activation, early UL synchronization, LTM cell switch command based on a target candidate beam), the beam level evaluation should be the baseline for L1 based C-LTM event evaluation.

Proposal 2: For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

Observation 3: In order to provide the best channel quality after C-LTM cell switch, the candidate beam for C-LTM execution should be the one that has triggered the event condition, irrespective of TCI state being activated or early UL synchronization performed for a candidate beam.

Proposal 3: When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set), UE performs RACH-less C-LTM using CG transmission with that triggered beam if UE has a valid TA for the associated candidate cell (i.e., either UE has received a TA and a TA timer is still running or UE has successfully measured a TA). Otherwise, UE performs RACH-based C-LTM with the triggered beam.

Observation 4: CG resource reservations from multiple target cells cany lead to significant inefficiency in time/frequency resources due to a large number of resource reservations.

Proposal 4: For RACH-less C-LTM, introduce an “early target cell notification” signaling procedure where UE informs the serving cell of potential target C-LTM cell(s) and beam(s). Detailed signaling/condition are FFS.

Observation 5: DG-based RACH-less C-LTM can be more efficient in resource allocation compared to CG-based RACH-less C-LTM, but the uncertainty of execution time and selected beam makes DG scheduling more challenging. However, the early target cell notification in Proposal 11 can alleviate this uncertainty significantly.

Proposal 5: Support DG-based RACH-less C-LTM.

Observation 6: In C-LTM without any UE assistance information for early TA acquisition, serving cell may need to trigger multiple PDCCH ordered RACH procedures towards multiple candidate cells, and those multiple TA values would need to be sent to UE.

Proposal 6: New TA MAC CE should support one or more TA values signalling.

Observation 7: For successful RACH-less C-LTM, serving cell may have to trigger PDCCH ordered RACH procedures for all potential target candidate cells, which could become huge overhead and interruptions in data transmission and reception at UE.

Proposal 7: RAN2 to discuss the candidate cell TA group where UE can assume that a TA value from one candidate cell (acquired either from new TA MAC CE or UE-measured TA) can be applied to all other candidate cells in that TA group.

Observation 8: The TA validation procedure for CG-SDT can be used as a reference for the TA maintenance procedure in C-LTM execution.

Proposal 8: RAN2 to agree the following as baseline for TA maintenance at UE:

- The TA timer value and RSRP change threshold are configured in RRC (which can be candidate cell specific or candidate cell group specific).

- When UE successfully receives the new TA MAC CE or measures UE-measured TA

  - Start the TA timer

  - Store the RSRP value of the candidate beam of the candidate cell

- The TA value is considered to be valid when the following conditions are true:

  - The RSRP change between the stored value and the current value has not increased/decreased by more than RSRP threshold; AND

  - The TA timer is running.

# References

- RP-242356; Revised Work Item: NR mobility enhancements Phase 4; 3GPP TSG RAN meeting #105; Melbourne, Australia, Sep 9-12, 2024

- Chairman Notes; TSG RAN WG2 Meeting #128

- Chairman Notes; TSG RAN WG2 Meeting #127bis