3GPP TSG-RAN WG2 Meeting #129	R2-2500487

Athens, Greece, Feb. 17th – 21st, 2025

# Agenda item:	8.6.4

# Source:	ETRI

# Title:	Further discussion on conditional LTM

# Document for:	Discussion, Decision

- Introduction

In RAN#128 meeting, agreements on conditional LTM design have been made as the followings [1]:

Agreements on Conditional LTM - CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution. - The L1 execution condition of a candidate cell is associated to only one triggering event. - For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. - To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition: - The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. - The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell. - The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH. - For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration. - The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4. - The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM. - Candidate cell TA is maintained by a new timer. - For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this paper, we discuss open issues to be considered to support conditional intra-CU LTM.

- Discussion

  - Number of LTM/CLTM candidate configurations

During the RAN2#127-bis meeting, several contributions regarding the maximum number of candidate configurations were submitted. For example, it was proposed that the network could configure up to 8 CLTM and LTM candidate configurations in total, or alternatively, configure up to 8 CLTM candidate configurations independently from the 8 LTM candidate configurations. Since this issue was not discussed in previous meetings, now is the right time to address it. Furthermore, the RRC rapporteur highlighted this issue in the running CR document [2] as follows:

“Editor’s Note: FFS whether the 8 is the number of LTM plus CLTM candidate configurations or we have 8 LTM candidate configurations and 8 CLTM candidate configurations”

There is no fundamental difference between CLTM and LTM. The key distinction between CLTM and network triggered LTM lies in which node triggers the cell switch to a target cell. In both cases, candidate cells are determined based on L3 measurement reports from the UE. Since the selection criteria for CLTM and LTM candidate cells are unlikely to be significantly different, there seems to be no strong reason to configure candidate cells separately for each LTM type. Instead, the network may configure some or all of the network triggered candidate cells as CLTM candidate cells. In this case, the existing LTM-config IE and LTM-Candidate IE can be reused with minimum modifications. The only required modification is the inclusion of execution conditions for CLTM candidate cells.

Observation 1: Since candidate cells for both conditional and network triggered LTMs are determined and configured based on the L3 measurement results, there is no reason for a network to configure candidate cells separately for each LTM type.

In a scenario where some or all of the network-triggered LTM candidate cells are also configured as CLTM candidate cells, the network-triggered LTM serves as the primary LTM, while the CLTM acts as a complementary mechanism to enhance the robustness of LTM-based cell switching. In most cases, the network selects a target cell based on L1 measurement reports and instructs the UE to perform the cell switch by transmitting a CSC MAC CE. However, if the channel conditions between the UE and the serving cell deteriorate rapidly, the UE may autonomously switch to a target cell that meets the execution conditions, without explicit instructions from the serving cell.

Observation 2: If some or all of the network triggered LTM candidate cells are also configured as CLTM candidate cells, the network triggered LTM serves as the primary LTM, while the CLTM can be used as a complementary mechanism to enhance the robustness of LTM-based cell switching.

If CLTM candidate cells are configured independently from network triggered LTM candidate cells, RAN2 would need to reassess the maximum number of LTM candidate cells to account for CLTM. Increasing the maximum number of LTM candidate cells would require additional network resources to support both conditional and network triggered LTM simultaneously. Conversely, if CLTM candidate cells are configured as a subset of network triggered LTM candidate cells, both LTM types can be supported without additional network resource requirements.

Proposal 1: the maximum number of LTM candidate configurations is 8 in total, regardless of whether they are network triggered LTM or CLTM.

Proposal 2: an LTM candidate cell can be configured as a CLTM candidate cell simultaneously.

Proposal 3: the UE identifies a candidate cell as a CLTM candidate cell if the LTM candidate configuration includes execution conditions for CLTM.

  - TA maintenance

In the last meeting, it was agreed that the validity of an early TA for a candidate cell would be maintained by a new timer in the UE. It was also agreed that the network could configure measurement reports to trigger PDCCH-ordered early RACH. Based on these agreements, the PDCCH-ordered early TA acquisition procedure is initiated by the source cell based on measurement reports from the UE. The UE then performs RACH with the candidate cell indicated in the PDCCH and receives a TA for the candidate cell from the source cell. Upon receiving the TA, the UE activates the timer to maintain the validity of the candidate cell’s TA. If the timer has not expired when CLTM is triggered, the UE considers the TA valid and applies it to perform a RACH-less CLTM to the candidate cell. However, if the timer expires before the CLTM cell switch is triggered, the UE considers the TA invalid. In this case, if CLTM to the candidate cell is triggered immediately after the timer expires, the UE performs a RACH-based CLTM.

If CLTM is not triggered for a considerable period after the timer expires, the question arises: what should the UE do? Rather than giving up on RACH-less CLTM, it would be more desirable to acquire a new TA for a potential upcoming CLTM cell switch. However, the method for re-triggering PDCCH-ordered early TA acquisition has not been discussed yet. As mentioned earlier, the PDCCH-ordered early TA acquisition is triggered by the source cell. However, in this case, the source cell cannot trigger the procedure because it is unaware of whether the timer for the TA has expired.

One possible solution is for the UE to inform the source cell of the timer expiration, allowing the source cell to decide on PDCCH-ordered early TA acquisition based on measurement reports and TA validity. While an RRC message could be used for this notification, defining a new MAC CE to notify the expiration of the timer would be preferable, as the timer is maintained at the MAC layer.

This notification could also help prevent the waste of radio resources allocated for CG-based first UL transmission on the target cell for RACH-less CLTM. Given the uncertainty of CLTM cell switch timing for both the serving cell and multiple candidate cells, radio resources for CG-based RACH-less CLTM would need to be reserved, with most likely going unused. If the UE does not have a valid TA with a candidate cell, the candidate cell does not need to prepare for CG-based RACH-less CLTM. Therefore, information on whether the UE has a valid TA is helpful for determining which candidate cell should prepare or release radio resources for CG-based RACH-less CLTM.

Proposal 4: if the timer for TA validity is expired, the UE notify the source cell of the timer expiration via a new MAC CE.

Proposal 5: if the early TA of a candidate cell becomes invalid, the source cell may re-trigger PDCCH ordered early RACH based on the measurement reports from the UE.

  - Mixture of conditional LTM and network triggered LTM

Co-existence of CLTM and network triggered intra-CU LTM

Since CLTM, like network-triggered intra-CU LTM, only allows intra-CU cell switching, the cell switch of one LTM type does not affect the candidate cell configuration of another LTM type. Additionally, according to WID [3], subsequent LTM should be supported in CLTM. As a result, conditional LTM can be configured simultaneously with intra-CU LTM. Furthermore, the UE does not need to release the CLTM cell configuration after an intra-CU LTM and should retain the CLTM cell configuration following a CLTM for subsequent LTM, unless explicitly released by the network.

Proposal 6: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 7: The UE keeps its CLTM cell configuration after a CLTM or intra-CU LTM for subsequent CLTM unless there are explicitly released by a network.

Co-existence of CLTM and network triggered inter-CU LTM

In the RAN2#125-bis meeting, it was agreed that the UE can be configured with a mixture of intra-CU and inter-CU candidate cells. If the previous proposals for supporting a mixture of CLTM and intra-CU LTM are agreed, the possibility of supporting a mixture of CLTM and inter-CU LTM should also be considered.

This scenario differs somewhat from a mixture of CLTM and intra-CU LTM. If an inter-CU LTM cell switch occurs while both CLTM and inter-CU LTM are configured, the target cell in a different CU may become a new source cell. Since CLTM is limited to cells within the same CU, the current CLTM configurations become irrelevant if the CU of the CLTM cells differs from the CU of the new source cell.

However, the UE does not necessarily need to release its CLTM candidate cell configurations, as it may perform a cell switch back to the previous serving cell sometime. Instead, the UE simply stops evaluating the execution conditions for CLTM candidate cells associated with the previous serving CU and begins evaluating the execution conditions for other CLTM candidate cells belonging to the current serving CU.

In the RAN2#127 meeting, it was agreed to introduce a new Rel-19 ID in RRC for UE to decide whether to perform RLC and PDCP re-establishment when performing LTM cell switch. After a cell switch, the UE can use the same Rel-19 ID to decide whether to continue evaluating the execution conditions for CLTM. Specifically, the UE stops evaluating the execution conditions for a LTM candidate cell if the candidate cell’s Rel-19 ID differs from that of the current serving cell. Conversely, if the Rel-19 ID of an LTM candidate cell matches that of the current serving cell, the UE starts evaluating the execution conditions for that cell after the cell switch.

Proposal 8: a mixture of CLTM and network triggered inter-CU LTM is supported.

Proposal 9: The UE keeps its CLTM cell configuration after a CLTM or inter-CU LTM for subsequent CLTM unless there are explicitly released by a network.

Proposal 10: the UE evaluates the execution conditions only for the candidate cells whose Rel-19 ID matches that of the current serving cell.

- Conclusions

In this paper, we discuss CLTM related issue. Following are the observations and proposals made in this paper:

Observation 1: Since candidate cells for both conditional and network triggered LTMs are determined and configured based on the L3 measurement results, there is no reason for a network to configure candidate cells separately for each LTM type.

Observation 2: If a subset of network triggered LTM candidate cells is also configured as CLTM candidate cells, the network triggered LTM serves as the primary LTM, while the CLTM can be used as a complementary mechanism to enhance the robustness of LTM-based cell switching.

Proposal 1: the maximum number of LTM candidate configurations is 8 in total, regardless of whether they are network triggered LTM or CLTM.

Proposal 2: an LTM candidate cell can be configured as a CLTM candidate cell simultaneously.

Proposal 3: the UE identifies a candidate cell as a CLTM candidate cell if the LTM candidate configuration includes execution conditions for CLTM.

Proposal 4: if the timer for TA validity is expired, the UE notify the source cell of the timer expiration via a new MAC CE.

Proposal 5: if the early TA of a candidate cell becomes invalid, the source cell may re-trigger PDCCH ordered early RACH based on the measurement reports from the UE.

Proposal 6: conditional LTM and intra-CU LTM can be configured simultaneously.

Proposal 7: The UE keeps its CLTM cell configuration after a CLTM or intra-CU LTM for subsequent CLTM unless there are explicitly released by a network.

Proposal 8: a mixture of CLTM and network triggered inter-CU LTM is supported.

Proposal 9: The UE keeps its CLTM cell configuration after a CLTM or inter-CU LTM for subsequent CLTM unless there are explicitly released by a network.

Proposal 10: the UE evaluates the execution conditions only for the candidate cells whose Rel-19 ID matches that of the current serving cell.

- References

- Chair notes RAN2#128

- R2-2500992 Running RRC CR for inter-CU and conditional LTM

- RP-242356 Revised Work Item: NR mobility enhancements Phase 4; Apple, China Telecom