3GPP TSG-RAN WG2 #129	R2-2500543

Athens, Greece, 17th – 21st Feb. 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on DSR enhancements for XR

Document for:        	Discussion and decision

Agenda Item:         	8.7.4.2		DSR enhancements

- Introduction

In RAN #106, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding scheduling enhancements.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

And the following agreements related to DSR enhancements were reached in RAN2#128 meeting.

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution, we provide the discussion on DSR enhancements for XR.

- Discussion

RAN2 agreed that Rel-19 DSR can report multiple pairs of remaining time and buffer size information based on multiple remaining time thresholds and one-bit indication can be included to indicate the presence of the pair. One remaining issue is whether the one-bit indication is used for indicating a certain pair or a further pair. In the former case, it is necessary to introduce the indication for each pair configured based on the multiple thresholds. On the other hand, in the latter case, it is only necessary to introduce the indication if there is a further pair where BS > 0, which may slightly save bits. From this perspective, we prefer that the one-bit indication indicates whether a further pair of remaining time information and buffer size information is present in Rel-19 DSR MAC CE. For the detailed design of Rel-19 DSR MAC CE, the reserved bit in Rel-18 DSR can be reused for the one-bit indication.

Proposal 1:	One-bit indication can indicate whether a further pair of remaining time information and buffer size information is present in Rel-19 DSR MAC CE.

Proposal 2:	For the detailed design of Rel-19 DSR MAC CE, the reserved bit in Rel-18 DSR can be reused for one-bit indication.

In the current specification, if pdu-SetDiscard is configured, any PDCP SDU which belongs to the same PDU set as a delay-critical PDCP SDU is also considered delay-critical, even if the remaining time of those SDUs does not fall below the DSR threshold. The delay information of these SDUs is reported in the same buffer size and remaining time field of Rel-18 DSR. In our understanding, the same principle should be applied to Rel-19 DSR. That is, if a PDCP SDU is reported in a specific portion of the Rel-19 DSR, all PDCP SDUs belonging to the same PDU set should be reported in the same portion. For example, their delay information can be reported in the portion corresponding to the smallest remaining value of discardTimers among all PDCP SDUs in the same PDU set.

Proposal 3:	If pdu-SetDiscard is configured, the delay information of all PDCP SDUs in the same PDU set can be reported in the same portion of Rel-19 DSR that corresponds to the smallest remaining value of discardTimers among those SDUs.

In addition, the delay-critical data volume reported in Rel-18 DSR is defined as follows, and the data highlighted in yellow is considered delay-critical regardless of the remaining time.

TS38.322 For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment; - RLC data PDUs that are pending for retransmission (RLC AM). In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting and as part of delay-critical RLC data volume for MAC delay status reporting.

TS38.323 For the purpose of MAC delay status reporting, the transmitting PDCP entity shall consider the following as delay-critical PDCP data volume: - the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

For example, in the RLC layer, RLC data PDUs pending for retransmission and RLC control PDUs such as STATUS PDU are considered as delay-critical data volume. The UE can prioritize the transmission of these yellow data even over the transmission of data with the remaining time below the threshold. Therefore, in order for the NW to allocate sufficient resources to accommodate all the delay-critical data, it would be beneficial to include and report the yellow data in Rel-19 DSR, just as in Rel-18. However, since the reported data in Rel-19 DSR would be divided into multiple portions based on the remaining time, it may not be clear in which portion the yellow data can be reported. Therefore, we need to discuss whether and how to report the yellow data that is considered delay-critical regardless of the remaining time in Rel-19 DSR.

Observation 1:	In Rel-18 DSR, there is data reported as delay-critical data volume regardless of the remaining time, such as RLC/PDCP control PDU.

Observation 2:	In Rel-19 DSR, the reported data would be divided into multiple portions based on the remaining time.

Proposal 4:	RAN2 to discuss whether/how to report the data that is considered delay-critical regardless of the remaining time in Rel-19 DSR.

There is still FFS on whether Rel-18 and Rel-19 DSR can be used at the same time. Essentially, the size of Rel-19 DSR can be larger than that of Rel-18 DSR because it can include multiple pairs of remaining time and buffer size. Therefore, if UL resources are not sufficient to accommodate Rel-19 DSR, it would be better to report Rel-18 DSR instead to inform the NW of the delay information as soon as possible. On the other hand, there is also the opinion that Rel-19 DSR can have flexibility in its size due to the one-bit indication, allowing it to be transmitted even if UL resources are insufficient. However, in this case, additional spec change would be required to determine which portions should be included in Rel-19 DSR with the limited resources. Moreover, in scenarios with limited resources, reporting Rel-18 DSR would be more beneficial as it can include delay information of all delay-critical data with a single buffer size and remaining time field, which means it can report more buffered data. For example, the following spec change can be considered to use Rel-18 and Rel-19 DSR at the same time.

TS38.321 If there is at least one DSR pending, the MAC entity shall: 1> if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the Enhanced DSR MAC CE plus its subheader as a result of logical channel prioritization and if for at least one LCG configured with remainingTimeThreshold-r19: 2> instruct the Multiplexing and Assembly procedure to generate the Enhanced DSR MAC CE as specified in clause 6.1.3.X. 1> else if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization: 2> instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72. 1> else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR: 2> trigger a Scheduling Request.

Observation 3:	There could be the cases where UL resources are not sufficient to accommodate Rel-19 DSR that is larger in size than Rel-18 DSR. This may delay reporting delay information.

Proposal 5:	Rel-19 DSR can be used in case there is at least one LCG configured with multiple reporting thresholds and UL resources can accommodate it. Otherwise, Rel-18 DSR can be used.

- Conclusion

In this contribution, we provided the following observations and proposals.

Proposal 1:	One-bit indication can indicate whether a further pair of remaining time information and buffer size information is present in Rel-19 DSR MAC CE.

Proposal 2:	For the detailed design of Rel-19 DSR MAC CE, the reserved bit in Rel-18 DSR can be reused for one-bit indication.

Proposal 3:	If pdu-SetDiscard is configured, the delay information of all PDCP SDUs in the same PDU set can be reported in the same portion of Rel-19 DSR that corresponds to the smallest remaining value of discardTimers among those SDUs.

Observation 1:	In Rel-18 DSR, there is data reported as delay-critical data volume regardless of the remaining time, such as RLC/PDCP control PDU.

Observation 2:	In Rel-19 DSR, the reported data would be divided into multiple portions based on the remaining time.

Proposal 4:	RAN2 to discuss whether/how to report the data that is considered delay-critical regardless of the remaining time in Rel-19 DSR.

Observation 3:	There could be the cases where UL resources are not sufficient to accommodate Rel-19 DSR that is larger in size than Rel-18 DSR. This may delay reporting delay information.

Proposal 5:	Rel-19 DSR can be used in case there is at least one LCG configured with multiple reporting thresholds and UL resources can accommodate it. Otherwise, Rel-18 DSR can be used.

- References

[1]	RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3