3GPP TSG-RAN WG2 #129	R2-2500544

Athens, Greece, 17th – 21st Feb. 2025

Source:                    	DENSO CORPORATION

Title:  	Discussion on RLC enhancements

Document for:        	Discussion and decision

Agenda Item:         	8.7.5		RLC enhancements

- Introduction

In RAN #106, revised WID on Rel-19 XR enhancements was approved [1], and the following objective was specified regarding RLC enhancements.

- Specify the following user plane enhancements [RAN2]: - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

And the following agreements related to RLC enhancements were reached in RAN2#128 meeting.

There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer Unless critical issue is identified, no Tx to Rx indication will be introduced Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In this contribution, we provide the discussion on RLC enhancements.

- Discussion

Avoid unnecessary RLC retransmissions

In RAN2#128 meeting, it was agreed that a new RLC timer at the Rx side would be introduced to determine obsolete RLC SDUs and the new timer would be started when the gap is detected at RLC layer. One issue is whether the new timer operates per RLC SDU or per RLC entity. If the new timer operates per RLC SDU, the RLC entity would need to maintain many timers, which may complicate the UE operation. From this perspective, it would be preferable that the new timer is configured and running per RLC entity. In other words, only one new timer per RLC entity is running at a given time, similar to t-reassembly.

Proposal 1:	A new RLC timer should be configured and running per RLC entity.

When the new timer expires, the receiving window can be advanced to abandon the obsolete RLC SDUs. According to the current specification, RX_Next serves as the lower edge of the receiving window. Therefore, RX_Next can be updated based on the expiry of the new timer. Furthermore, after the new timer expires, it is necessary to transmit a status report to notify that the receiving window has advanced by indicating positive acknowledgements for the obsolete SDUs. This allows the transmitting window to be advanced as well, addressing the window stalling issue. Thus, it would be beneficial to trigger the status report based on the expiry of the new timer to advance the transmitting window as soon as possible. If the status report is triggered, the RLC entity constructs a STATUS PDU for the RLC SDUs with SN which is determined based on RX_Next and RX_Highest_Status. Therefore, the status report should be triggered after both RX_Next and RX_Highest_Status are updated based on the expiry of the new timer.

Proposal 2:	When a new RLC timer expires, the receiving window can be advanced to abandon obsolete RLC SDUs, i.e., RX_Next can be updated.

Proposal 3:	When a new RLC timer expires, a status report can be triggered to advance the transmitting window as soon as possible.

Observation 1:	If the status report is triggered, the RLC entity constructs a STATUS PDU for the RLC SDUs with SN which is determined based on RX_Next and RX_Highest_Status.

Proposal 4:	A status report should be triggered after RX_Next and RX_Highest_Status are updated based on the expiry of a new RLC timer.

For the combined approach, the Tx side can stop transmissions of obsolete RLC SDUs independently of the Rx side to avoid unnecessary transmissions. However, it is still under discussion how to determine the obsolete SDUs. A simple option is to use discardTimer associated with the SDUs. If the discardTimer expires and the associated SDU has already been delivered to the lower layer, the PDCP entity provides a discard indication to the lower layer. The RLC entity can determine obsolete SDU based on this indication and stop its transmission.

Proposal 5:	The RLC transmitting entity can stop transmissions of obsolete RLC SDUs based on an indication from the PDCP entity. This indication can be based on the expiry of discardTimer associated with the SDUs.

Timely RLC retransmission

In the last meeting, RAN2 agreed that both autonomous retransmission and polling enhancement can be supported for timely RLC retransmission. In the current polling procedure, pollPDU and pollByte are used to trigger a polling. In addition, t-PollRetransmit is used to trigger the retransmission of a poll if the requested status report is not received. One possible approach to reduce the polling latency would be to introduce smaller values for these parameters. This approach is simple and has minimal impact on the specification. However, since it does not take into account the delay status, unnecessary polling and status reporting for non-delay-critical data may be frequently triggered. Another approach could be to introduce new polling triggering conditions based on the delay status. For example, the UE can include a poll in an AMD PDU containing a delay-critical RLC SDU. This allows timely providing retransmission opportunities only for delay-critical data with small remaining time.

Proposal 6:	For polling enhancement, the following options can be considered.

- Option 1: Introduce new smaller values of pollPDU, pollByte and/or t-PollRetransmit.

- Option 2: Introduce new polling triggering conditions based on the delay status.

For autonomous retransmission, the UE can retransmit RLC SDUs without receiving negative acknowledgements in a status report. However, it is not clear how the UE can determine which RLC SDUs should be considered for retransmission. Considering the impact on capacity due to the signalling overhead, frequent autonomous retransmission should be avoided. In our opinion, it would be preferable to trigger autonomous retransmissions based on the remaining time of the SDUs. In other words, the UE can consider an RLC SDU for retransmission if a positive acknowledgement for the SDU has not yet been received and its remaining time falls below a configured threshold. We believe that introducing a new threshold for this purpose and allowing it to be configured to a smaller value would minimize the overhead.

Proposal 7:	For autonomous retransmission, UE can consider an RLC SDU for retransmission based on its remaining time. And a new remaining time threshold can be introduced for this purpose.

- Conclusion

In this contribution, we provided the following observation and proposals.

Proposal 1:	A new RLC timer should be configured and running per RLC entity.

Proposal 2:	When a new RLC timer expires, the receiving window can be advanced to abandon obsolete RLC SDUs, i.e., RX_Next can be updated.

Proposal 3:	When a new RLC timer expires, a status report can be triggered to advance the transmitting window as soon as possible.

Observation 1:	If the status report is triggered, the RLC entity constructs a STATUS PDU for the RLC SDUs with SN which is determined based on RX_Next and RX_Highest_Status.

Proposal 4:	A status report should be triggered after RX_Next and RX_Highest_Status are updated based on the expiry of a new RLC timer.

Proposal 5:	The RLC transmitting entity can stop transmissions of obsolete RLC SDUs based on an indication from the PDCP entity. This indication can be based on the expiry of discardTimer associated with the SDUs.

Proposal 6:	For polling enhancement, the following options can be considered.

- Option 1: Introduce new smaller values of pollPDU, pollByte and/or t-PollRetransmit.

- Option 2: Introduce new polling triggering conditions based on the delay status.

Proposal 7:	For autonomous retransmission, UE can consider an RLC SDU for retransmission based on its remaining time. And a new remaining time threshold can be introduced for this purpose.

- References

[1]	RP-243318, Revised WID on XR (eXtended Reality) for NR Phase 3