3GPP TSG-RAN WG2 Meeting #129	R2-2500675

Athens, Greece, Feb. 17th – 21th, 2025

Agenda item:		8.8.4 Support of broadcast service

Title:		Remaining issues on the support of broadcast service in NTN

Source:		ETRI

Document for:		Discussion

# 1. Introduction

During the 102th meeting of RAN, a new WI on NTN for NR phase 3 was approved, including the following objective:

- Specify signalling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3]. - Specify SIB signalling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary between CN and NG-RAN. [RAN3]

The first discussion regarding the new WI took place during 125bisth meeting of RAN WG2 and then, the following agreements were made for the support of broadcast service in NTN.

Agreements: 1. For MBS broadcast service we don’t restrict the work to any satellite constellation type 2. We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS 3. We will cover at least the case where the indicated intended service area covers a portion of a NTN cell 4. The intended service area can cover the area of more than one NTN cells (or portions thereof) 5. Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended service area) 6. At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered): - Circles (like for TN coverage) - Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

During the 126th meeting of RAN WG2, the following agreements were made:

Agreements: 1. For MBS broadcast service, both EFC and EMC are supported. 2. RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area. 3. MBS broadcast intended service area is provided via system information 4. For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS 5. When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

During the 127th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s). 2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions 3. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) 4. For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

During the 127bisth meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

During the 128th meeting of RAN WG2, the following agreements were additionally made:

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this paper, we would like to provide our views on the highlighted aspects as well as remaining issues on support of broadcast service in NTN.

# 2. Discussion

2.1	Signalling of intended service areas

The range of a list of intended service areas

In the last meeting, RAN WG2 reached the following agreements regarding the definition and signalling of intended service areas for the support of broadcast service in NTN.

A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbour cell(s).

Introduce a new SIB to include a list of intended service areas and related point.

Based on these agreement, the following three options can be considered for the range of a list of intended service areas that should be signalled via the new SIB in a given cell.

- Option 1: The list of intend service areas covers the entire geographical areas set by network.

- Option 2: The list of intended service areas covers the geographic areas within the serving cell and all neighbouring cells.

- Option 3: The list of intended service areas covers the geographic areas within the serving cell and only the areas across the serving cell and overlapping neighbour cells.

Figure 1: the range of a list of intended service areas signalled in certain cell

The decision of one of these options will impact further discussions on remaining issues for the support of broadcast service in NTNt. For instance,

- If Option 1 is selected, the list of intended service areas does not need to be frequently updated, even in an EMC (Earth Moving Cell) scenario.

  - This allows the intended service areas to be referenced easily via an index in the list.

  - However, Option 1 requires high signalling overhead compared to other options.

- If Option 2 or Option 3 is selected, the list of intended service areas may require frequent updates in an EMC scenario.

  - This could introduce challenges in using an index in the list as a pointer due to potential list modifications.

  - Option 3, while reducing signalling overhead, may not fully support the intend service area information for neighbouring cells, impacting service continuity.

Given these trade-offs, ETRI believes that RAN WG2 should first decide on the appropriate range of the list of intended service areas signalled via the new SIB before addressing other related issues.

Proposal 1: RAN WG2 to decide on the range of intended service areas signalled via a new SIB in a certain cell, based on the following options:

- Option 1: to cover the entire geographical areas set by network.

- Option 2: to cover the geographic areas within the serving cell and all neighbouring cells.

- Option 3: to cover the geographic areas within the serving cell and only the areas across the serving cell and overlapping neighbour cells.

How to point to the intended services areas via the index in the list or with an ID or another way

In the last meeting, RAN WG2 agreed on the introduction of a new SIB to include a list of intended service areas and related points. However, no consensus was reached on the method for referencing the intended service areas, specifically whether to use an index in the list or an ID.

- Using index

  - Pros: smaller data size and simple lookup mechanism

  - Cons: dependency on the list order and no global uniqueness

- Using ID

  - Pros: stable reference, global uniqueness and more readability

  - Cons: larger data size and additional management overhead

If the entire geographical intended service areas are not indicated via a new SIB in a certain cell, the list of the intended service areas may need to be frequently updated, particularly when the cell is an EMC. Additionally, the intended service area information may be provided via NAS signalling as well as AS signalling. In such cases, the use of a unique identifier can provide more flexibility in handling references across different cells/networks. Given these considerations, ETRI believes that an ID-based approach would be a more suitable way to point the intended service areas.

Observation 1: a list of intended service areas may be frequently updated when a cell is an EMC and the intended service area information may be provided via both AS and NAS signalling

Proposal 2: RAN WG2 to decide on the use of an ID-based approach for pointing the intended service areas in a new SIB.

2.2	Service Continuity

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session as specified in TS 38.300 [2] as long as the two following conditions are fulfilled:

- 1) SIB1 scheduling information of the cell reselected by the UE due to frequency prioritization for MBS contains SIB20;

- 2) Either:

  - One or more MBS FSAI(s) of that frequency is indicated in SIB21 of the serving cell and the same MBS FSAI(s) is also indicated for this MBS broadcast service in MBS User Service Description (USD) as specified in TS 26.517 [20], or

  - SIB21 is not provided in the serving cell and that frequency is included in the USD of this service, or

  - SIB21 is provided in the serving cell but does not provide the frequency mapping for the concerned service, and that frequency is included in the USD of this service.

If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session as specified in TS 38.300 [2], as long as SIB1 scheduling information of the cell contains SIB20 on the MBS frequency which the UE monitors and as long as the condition 2) above is fulfilled for the serving cell.

Considering that SIB21 contains the mapping relationship between the current and/or neighbouring carrier frequencies and MBS FSAI, this legacy mechanism for MBS service continuity assumes that the MBS service area is mapped onto one or more cells. However, in NTN systems, this may not work effectively because the UE may not consider that frequency as the highest priority if the MBS intended service area(s) of that frequency do not include the intended service area of the MBS session that the UE is receiving or interested in, even if the above conditions 1 and 2 are met.

In the 127th meeting, the following agreements were reached regarding UE behaviour for the support of broadcast service in NTN.

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2).

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3).

Given this agreed UE behaviour, ETRI believes that enhancements to the legacy mechanism for MBS service continuity are necessary. Specially, the UE may consider that frequency of neighbouring cell providing the interested MBS as the highest priority only if it will be within the intended service areas after cell reselection. Conversely, the UE should not assign the highest frequency to that frequency if it will be outside the intended service areas.

To address these issues, the following two options can be considered.

- Option 1: To define a new mapping relationship between the current and/or neighbouring carrier frequencies and MBS intended service area IDs. This approach would involve modifying SIB21 to include:

  - Carrier frequency list (already existing in SIB21)

  - MBS intended service area ID(s) per each carrier frequency

- Option 2: To utilize intended service area information of neighbouring cells from a new SIB and MCCH. This approach would include:

  - A list of intended service areas within each neighbouring cell in a new SIB

  - The intended service area IDs of neighbouring cells in MCCH

In option 1, when the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session if the UE will be within the intended service areas corresponding to one or more intended service area ID(s) of that frequency indicated in SIB21. Conversely, the UE should not prioritize that frequency as the highest priority if it will be outside the intended service areas.

In option 2, the UE may consider that frequency as the highest priority during the MBS broadcast session if the UE will be within the intended service areas corresponding to one or more intended service ID(s) of neighbouring cells indicated in MCCH.

Proposal 3: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency as the highest priority during the MBS broadcast session only if it will be only within the intended service areas.

Proposal 4: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE should not consider that frequency as the highest priority during the MBS broadcast session if the UE will be outside the intended service areas.

Proposal 5: For service continuity, RAN WG2 to decide whether to

- Introduce intended service area information (e.g. intended service area ID(s)) associated to the current and/or neighbouring carrier frequencies in SIB21

- Utilize intended service area information associated to neighbouring cells, as indicated in MCCH.

# 3. Conclusions

Proposal 1: RAN WG2 to decide on the range of intended service areas signalled via a new SIB in a certain cell, based on the following options:

- Option 1: to cover the entire geographical areas set by network.

- Option 2: to cover the geographic areas within the serving cell and all neighbouring cells.

- Option 3: to cover the geographic areas within the serving cell and only the areas across the serving cell and overlapping neighbour cells.

Observation 1: a list of intended service areas may be frequently updated when a cell is an EMC and the intended service area information may be provided via both AS and NAS signalling

Proposal 2: RAN WG2 to decide on the use of an ID-based approach for pointing the intended service areas in a new SIB.

Proposal 3: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency as the highest priority during the MBS broadcast session only if it will be only within the intended service areas.

Proposal 4: When an MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(S) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE should not consider that frequency as the highest priority during the MBS broadcast session if the UE will be outside the intended service areas.

Proposal 5: For service continuity, RAN WG2 to decide whether to

- Introduce intended service area information (e.g. intended service area ID(s)) associated to the current and/or neighbouring carrier frequencies in SIB21

- Utilize intended service area information associated to neighbouring cells, as indicated in MCCH.