3GPP TSG-RAN WG2 Meeting #129	R2-2500678

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.3

Source:	Apple

Title:	Views on Enhancements Relating to RRM Measurement Gaps

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR, we have the following objective [1]:

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In RAN2 #127bis, we have reached the following agreements [2, 3]:

RAN2 #127bis Agreements: RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution RAN2 #128 Agreements: No MG-specific enhancements is needed on DSR operation. RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped No MG-specific enhancements is needed for DRX operation.

Furthermore, RAN4 has confirmed that DCI-based approach for MG skipping is feasible, based on the reply LS [4]:

R4-2420198: Response LS on gaps/restrictions that are caused by RRM measurements RAN4 discussed the above RAN1 working assumption and confirms that Alt 1-1 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability. It is also RAN4 understanding that the time offset is the time from the end of the last symbol of the PDCCH carrying the DCI to the start of the corresponding skipped measurement gap indicated by the DCI, where the start of the measurement gap is determined after applying Measurement Gap Timing Advance (mgta) configured by the network according to TS 38.331. Furthermore, from RAN4 RRM requirements perspective, Alt 1-1 is not supported for EN-DC and NE-DC scenarios. Lastly, in addition to Alt 1-1, RAN4 is also discussing the need to introduce semi-static indication based solutions to skip measurement gap occasions.

# Discussions

According to the Reply LS from RAN4 [3], although the DCI-based approach is feasible, it is actually insufficient for EN-DC and NE-DC scenarios. At the same time, both RAN2 and RAN4 are evaluating if RRC-based semi-static MG skipping control is also needed, in addition to the DCI-based approach introduced by RAN1, as a complementary mechanism.

Observation 1: From RAN4 perspective, the DCI-based MG skipping approach alone cannot fulfil RRM requirements in EN-DC and NE-DC scenarios.

Observation 2: Both RAN2 and RAN4 are discussing the need to introduce RRC-based MG skipping approach.

Since both RAN2 and RAN4 are considering RRC-based approach, we would like to provide some views about feasibility of RRC-based approach from RAN2 perspectives. In our understandings, the key motivation for MG skipping is to make sure that transmission/reception of packets from specific XR traffic flows will not be blocked by measurement gaps, in order to reduce the latency and thereby maximizing the XR user experience. To enable RRC-based approach, a prerequisite would be the gNB’s knowledge about the traffic arrival patterns. One could argue that the packets can arrive spontaneously and therefore a fixed skipping pattern configured by RRC may not be flexible enough to cope with it. However, since many XR traffics including video and audio streams are typically periodic, we think the network can have a good knowledge about when the packets arrival cadence based on the traffic periodicity, based on either the assistance information from the CN or the UE. In Rel-18, 3GPP has already specified several features including DRX for non-integer periodicity and Multi-PUSCH configured grants, based on the gNB’s knowledge relating to traffic patterns. Thus, we think it is reasonable for the gNB to further utilize its knowledge of XR traffic arrival to pre-configure a pattern of time-windows wherein the MG should be skipped. This is illustrated in Figure 1:

Figure 1 MG Skipping based on RRC-configured time window pattern.

Observation 3: Typical XR traffics are periodic, and it is feasible for the gNB to have the knowledge of packet arrival times and configure MG skipping patterns accordingly.

We have also observed some potential issues with DCI-based approach from RAN2 point of view. If only DCI-based approach is applied, the UE can only know that a MG will be skipped based on DCI that is signalled not long before the MG occasion (5ms or 3ms, depending on UE capability), and this may lead to some additional restrictions and complications for implementation for certain L2 procedures such as the UTO-UCI mechanisms specified in Rel-18 XR. According to TS 38.321, when UTO-UCI is configured for a configured grant (CG) configuration, the MAC layer should determine if a number of consecutive subsequent CG occasions are to be used or unused, and then indicate such decisions to the gNB via the UTO-UCI. Since this is specified as a L2 functionality, we are not sure if RAN1 has considered this aspect in their discussions.

TS 38.321 V18.2.0: For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI.

If the UE does not know that an upcoming MG will be skipped early enough, it may directly determine any CG occasions that overlap with this MG as “unavailable/unused”, because by default the UE is not allowed to transmit on UL-SCH resources during a MG. Thus, UTO-UCI may be derived in an inappropriate manner, as it may fail to take MG skipping into account (e.g. a UL-SCH resource that has been considered unavailable is actually available because of MG skipping). Moreover, the UE cannot reverse/update its decision for a CG occasion, if it is already indicated as “unused”. Consequently, the UE cannot use the CG occasion(s) even if the overlapping MG is skipped, which may result in undesirable latency for uplink transmission.

Such problems could be avoided, if the UE only derives UTO-UCI after it has confirmed that whether a MG will be skipped or not (e.g. after receiving a DCI that instructs the UE to skip a MG). However, this obviously creates a lot of constraints for UE implementation in terms of UTO-UCI derivation. The lead time of the DCI before the MG could be as short as 5ms, but the time required by the UE to derive UTO-UCI could be much longer than that (especially if the value of nrofBitsInUTO-UCI is large, e.g. 8). Ideally, we should not couple the processing timeline for UTO-UCI and MG skipping, as it would increase a lot of UE implementation complexity.

Observation 4: Solely relying on DCI for MG skipping may create some UE implementation problems for some L2 functionalities such as UTO-UCI determination, that may have been overlooked by RAN1.

Since the UE is able to know which MG occasions will be skipped much earlier based on such RRC configuration, this is advantageous in terms of UE implementation. In particular, the UE can derive UTO-UCI by directly taking the MG skipping into account, without abandoning a CG occasion unnecessarily. Thus, introducing RRC-based solution would provide the gNB more flexibility to configure MG skipping in different situations. For examples, the gNB may enable RRC-based solution instead of DCI-based solution when UTO-UCI is also configured.

Overall, we think both DCI-based and RRC-based signalling for MG skipping should be supported. The former allows the gNB to cancel a MG based on the spontaneous need, while the latter allows the UE to implement certain features (e.g. UTO-UCI) in a more sensible way with the knowledge of traffic periodicity and packet arrival times.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported in Rel-19.

# Conclusions

In this paper, we have provided some of our views about measurement gaps skipping, including the following observations and proposals:

Observation 1: From RAN4 perspective, the DCI-based MG skipping approach alone cannot fulfil RRM requirements in EN-DC and NE-DC scenarios.

Observation 2: Both RAN2 and RAN4 are discussing the need to introduce RRC-based MG skipping approach.

Observation 3: Typical XR traffics are periodic, and it is feasible for the gNB to have the knowledge of packet arrival times and configure MG skipping patterns accordingly.

Observation 4: Solely relying on DCI for MG skipping may create some UE implementation problems for some L2 functionalities such as UTO-UCI determination, that may have been overlooked by RAN1.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported in Rel-19.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #128 Chair’s Notes, Orlando, USA, November 2024.

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements (Response to R1-2407561), 3GPP RAN4 #113, Orlando, USA, November 2024.