3GPP TSG-RAN WG2 Meeting #129	R2-2500680

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.4.2

Source:	Apple

Title:	Views on DSR Enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

Based on the study in Rel-19 XR Phase 3, the following objective has been added to the work item [1]:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

For DSR enhancements, we have made the following agreements in previous meetings [2-4]:

RAN2 #127 Agreements: Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR RAN2 #127bis Agreements: We do not change the definition of delay-critical data For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - • Buffer size of data volume in each portion - • Shortest remaining time among PDCP SDUs buffered in each portion. A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds RAN2 #128 Agreements: Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

This paper presents some of our views on DSR enhancement.

# Discussions

## Data Volume Calculations

In Rel-18 DSR, if pdu-SetDiscard is configured, any PDCP SDU that belongs to the same PDU Set should also be considered as delay-critical. Furthermore, it should also be included in the delay-critical data volume for reporting, even though the remaining time for some of these PDCP SDUs may still larger than the threshold. This is described in the definition of Delay-Critical PDCP SDUs in TS 38.323:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

Since the buffer of a LCG is divided into multiple portions based on the multiple remaining time thresholds in Rel-19, RAN2 needs to discuss how the integrated handling of PDU Sets can be extended in such cases. When calculating the buffer size of portions corresponding to different remaining time ranges, we think all PDCP SDUs of a PDU Set should be included in the buffer size for the portion corresponding to the smallest remaining time among the PDCP SDUs of this PDU Set. Therefore, when the most urgent PDCP SDU of a PDU Set is being included in the buffer size calculation for a specific portion, all other PDCP SDUs of the same PDU Set should also be included in the same portion, if pdu-SetDiscard is configured.

Proposal 1: If pdu-SetDiscard is configured, all PDCP SDUs of a PDU Set should be included in the data volume calculation for the same buffer portion that corresponds to the smallest remaining time among the PDCP SDUs of this PDU Set.

Furthermore, as control PDUs are not associated with discard timer, it is questionable how the control PDUs should be sorted in data volume calculation procedures. More specifically, which “portion” should be chosen for buffer size reporting that includes control PDUs. In our views, this is the simplest to directly include control PDUs in the buffer size portion corresponding to the smallest remaining time threshold. This is reasonable as typically control PDUs should be transmitted as soon as possible.

Proposal 2: The Control PDUs are always included in the buffer size portion corresponding to the smallest remaining time threshold.

Based on the discussions above and the current agreements, we noted that in some cases a PDCP SDU is included in the buffer size corresponding to a remaining time range that does not match its own remaining time. For examples, a PDCP SDU is included in a buffer size portion based on the remaining time of another PDCP SDU belonging to the same PDU Set (as described in Proposal 1). Also, as agreed, the UE may include a non-delay-critical PDCP SDU ahead of delay-critical PDCP SDU in buffer size calculations.

In these cases, if a PDCP SDU has been included in the data volume calculation for a buffer portion, it should not be included again in the data volume calculation for another buffer portion, regardless of its remaining time and the remaining time range that these buffer portions correspond to. Apparently, this is needed as we should avoid “double counting” issues of data volume calculation that can result in inaccurate buffer size information of DSR.

Proposal 3: A PDCP SDU that has been included in the data volume calculation for one buffer portion should not be counted again in the data volume calculation for another buffer portion for DSR, regardless of its remaining time.

## Remaining Time Information for Non-Delay-Critical Data

For Rel-19 DSR, we have agreed that the network can configure DSR triggering threshold and reporting threshold without constraints. Thus, the UE may be configured to report remaining time information for data with remaining time larger than the DSR triggering threshold (i.e. non-delay-critical data). In our understanding, this is motivated by the intention to report both delay-critical and non-delay-critical data in the DSR, which allows the gNB to get a more completed picture about the buffer status, including the buffer size of data that is less urgent. For instance, it provides much more leeway for the scheduler to plan ahead for its resource allocation strategy, if it has the knowledge about delay status of both delay-critical and non-delay-critical data.

The Rel-18 DSR MAC CE has a 6-bits field for remaining time indication. According to the description in TS 38.321:

- Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in any MAC PDU, at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1) msec;

As a result, the UE can indicate the remaining time up to 63 msec. Furthermore, in TS 38.331, the maximum value of remaining time threshold level for DSR triggering that can be configured for an LCG is also 64 msec.

There could be cases where the remaining time of a buffered packet is larger than 64msec. If the UE intends to report the remaining time information for such data in Rel-19 DSR (e.g. as a part of reporting for non-delay-critical data), the 6-bits field in the DSR MAC CE may not be able to accommodate.

To solve this problem, instead of encoding the remaining time for non-delay-critical based on its absolute value, we think the remaining time information for non-delay-critical data can be encoded as a differential value in the DSR MAC CE. For example, it could be encoded based on the difference with respect to the DSR triggering threshold in the DSR MAC CE.  Thus, the UE can support a wider range of remaining time reporting without having to increase the number of bits in the remaining time field.

Proposal 4: For Rel-19 DSR MAC CE, the remaining time for non-delay-critical data (i.e. the data with remaining time larger than the DSR triggering threshold) can be encoded as the differential value with respect to the DSR triggering threshold, to support reporting of remaining time values larger than 63msec without increasing the number of bits.

# Conclusions

In this paper, we have the following proposals for Rel-19 DSR enhancements:

Proposal 1: If pdu-SetDiscard is configured, all PDCP SDUs of a PDU Set should be included in the data volume calculation for the same buffer portion that corresponds to the smallest remaining time among the PDCP SDUs of this PDU Set.

Proposal 2: The Control PDUs are always included in the buffer size portion corresponding to the smallest remaining time threshold.

Proposal 3: A PDCP SDU that has been included in the data volume calculation for one buffer portion should not be counted again in the data volume calculation for another buffer portion for DSR, regardless of its remaining time.

Proposal 4: For Rel-19 DSR MAC CE, the remaining time for non-delay-critical data (i.e. the data with remaining time larger than the DSR triggering threshold) can be encoded as the differential value with respect to the DSR triggering threshold, to support reporting of remaining time values larger than 63msec without increasing the number of bits.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127 Chair’s Notes, Maastricht, Netherlands, August 2024.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #128Chair’s Notes, Orlando, USA, November 2024.