3GPP TSG-RAN WG2 Meeting #129	R2-2500681

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.5

Source:	Apple

Title:	Views on Fast RLC Retransmissions

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR Phase 3, we have the following objective [1]:

-	Specify the following user plane enhancements [RAN2]

-	RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget.

According to the agreements made in RAN2 #125bis, it is confirmed that RAN2 will consider the following directions:

RAN2 #125bis Agreements: RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

For timely RLC retransmission, the following agreements have been made during RAN2 #126 and RAN2 #127bis [2-4]:

RAN2 #126 Agreements on Timely RLC Retransmissions: To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options: - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered) - Retransmission based on enhanced status report - Retransmission based on enhanced polling - FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism Impact on capacity should be considered RAN2 focuses on the enhancements for UL traffic RAN2 #127bis Agreements on Timely RLC Retransmissions: RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. Exclude enhanced status reporting. Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay RAN2 #128 Agreements on Timely RLC Retransmissions: Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

Since RAN2 has agreed that the solution should include the components of both autonomous retransmission and polling enhancements, this paper presents our views on the enhancements that should be introduced in each of these directions.

# Discussions

## Autonomous Retransmission

With Autonomous RLC retransmission, the transmitter can autonomously consider a packet for retransmission once it becomes delay-critical (i.e. when the remaining time of a packet becomes smaller than a threshold), without status feedback. From the latency perspective, such scheme may be beneficial as a more urgent RLC SDU could be retransmitted more proactively and quickly. Nonetheless, some concerns on impacts to the system capacity have also been brought up in the discussions, especially in uplink scenarios. For instance, the UE may not know the network status in general, blind retransmission may cause further congestions to other traffic flows and/or users.

When packets start to become delay-critical, implicitly some degree of congestion may be present in the network, and therefore the radio resource availability may be scarce. If we recklessly trigger more retransmissions without status feedback from the receiver under such situations, congestions may become even more severe. It is worth highlighting that, regardless of how frequently or how quickly a packet is considered for retransmission, if there is no radio resource available, the retransmission of this packet still cannot be performed over the air interface. For UL, since the network does not know when (and how many) autonomous RLC retransmissions will be triggered, the gNB may not be able to schedule appropriate uplink resource on time. Thus, we cannot guarantee that autonomous retransmission based on remaining time can always achieve the desired latency.

Furthermore, for XR use cases we may have to cope with PDU Sets, wherein a large number of packets of a PDU Set may trigger autonomous retransmission concurrently (due to their similar arrival times). With such a surge of data volume for retransmission, it is difficult to ensure that the gNB can allocate sufficient radio resources to accommodate both initial transmissions and retransmissions that have been triggered by the UE autonomously. Since RLC-AM shall always prioritize retransmissions over initial transmissions, in such cases any UL resource allocated by the network may be largely consumed by the retransmissions, which causes delay for the new transmission. Consequently, the initial transmissions cannot be performed immediately even if the radio resource is available, and they may subsequently become delay-critical as well. This eventually leads to a vicious cycle, and therefore triggering autonomous retransmission in an impetuous manner will only worsen the congestion.

Observation 1: It is questionable how the network can timely allocate sufficient UL resource for autonomous RLC retransmission, especially when the UE concurrently triggers autonomous retransmission for a large number of packets (e.g. packets of a PDU Set).

Observation 2: Regardless of how quickly/frequently autonomous RLC retransmission is triggered, if there is no available/sufficient radio resource, we cannot ensure retransmission can be performed immediately, so the benefits in terms of latency cannot be guaranteed.

Observation 3: Triggering autonomous retransmission in an impetuous manner (e.g. directly consider a RLC SDU for retransmission as soon as its remaining time becomes smaller than a threshold) may make the congestion even more severe.

Based on these observations, we think RAN2 needs to investigate the following two aspects relating to Autonomous Retransmissions, in order to make sure such feature can work effectively:

- How to minimize the impacts to capacity?

- How to make sure the gNB can allocate resource for Autonomous Retransmission timely ?

For the minimization of impacts to capacity, we think the UE should only apply autonomous retransmission under certain conditions. For examples, the UE may only consider autonomous retransmission as a last resort, i.e. when the window is stalled and when the transmitter cannot receive timely status feedback. In such situations, since new transmission cannot be performed anyway due to window stalling, the transmitter can endeavor to expedite the retransmissions.

On the other hand, we could restrict autonomous retransmission only to certain types of packets. Based on the AL-FEC study conducted by SA2, we understand that some packets of a PDU Set may become obsolete if the receiver already have sufficient packets to decode the contents. Since these obsolete packets are no longer useful from Application perspectives, autonomous retransmission of such packets seems to be a wastage of resource. Hence, we think RAN2 can consider refraining from performing autonomous retransmission for the obsolete packets.

Finally, the network may configure some upper limits about the number of autonomous retransmissions. The UE should only consider autonomous retransmission for a RLC SDU when the number of pending retransmissions is still below a threshold. We think RAN2 can consider all these options.

Proposal 1: To minimize the impacts of Autonomous Retransmission to capacity, RAN2 may consider the following some of the following options:

- Option 1: The UE only applies Autonomous Retransmission as a last resort, i.e. when the transmitting window is stalled, and status feedback cannot be received on time.

- Option 2: The UE only applies Autonomous Retransmission to certain packets, e.g. PDCP SDUs that are not obsolete yet.

- Option 3: The UE is configured with a maximum number of pending autonomous retransmissions.

To make sure that the UE can get timely UL resource allocation for autonomous retransmissions, we think the UE can trigger BSR in the MAC layer when there is a “surge” of data volume due to autonomous retransmission. For instance, if autonomous retransmissions are triggered for multiple packets of a PDU Set concurrently, it would be useful if the UE can immediately notify the gNB in order to get the uplink resource allocation on time. Of course, we could also have periodic BSR with very short periodicity, but this may be less efficient in terms of signalling overhead if autonomous retransmission is not triggered that often. Furthermore, one could argue that DSR can be used for such purposes, but since DSR is only triggered based on remaining time of PDCP SDU that has not been transmitted yet, we cannot guarantee this can help the resource allocation for autonomous retransmissions.

Proposal 2: To ensure/facilitate timely radio resource allocations, the UE may trigger BSR when Autonomous Retransmission is triggered for a certain amount of data.

## Polling Enhancements

For Polling enhancements, the triggering conditions of polling can be extended by taking delay-criticality of RLC SDUs into account. The transmitter can therefore trigger a poll faster when there is a critical need. Some examples of new triggering conditions for polling may include:

- When a delay-critical RLC SDU is present in the queue,

- When the number of RLC PDUs corresponding to delay-critical RLC SDUs sent since the last status report is received satisfies a threshold,

- When the number of bytes RLC PDUs corresponding to delay-critical RLC SDUs sent since the last status report is received satisfies a threshold.

Thus, we think RAN2 can introduce the new poll triggering conditions based on the presence of delay-critical RLC SDUs.

Proposal 3: For Polling Enhancements, RAN2 can consider introducing new poll triggering conditions based on the presence of delay-critical RLC SDUs.

Based on these new poll triggering conditions, a poll can be transmitted whenever there is an urgent need, and theoretically the receiver can provide the status report in a timely manner. For UL cases where the receiver is a gNB, we believe it is up to network implementation to send the status report. Thus, as long as the gNB is properly implemented, we think faster RLC retransmission can be achieved. Nevertheless, we cannot guarantee the status report is always received on time. Specifically, how quickly the transmitter can receive the status report may be constrained by the instantaneous DL channel quality and congestion status.

Observation 4: Faster RLC Retransmission can be achieved with Polling Enhancement in UL, as long as the gNB can provide status report in a timely manner. However, this cannot always be guaranteed.

With this in mind, we think a complementary mechanism may be introduced to deal with such situations. In particular, we can allow the transmitter to perform retransmission of delay-critical RLC SDUs when t-PollRetransmit expires and when the transmitting window is stalled. In fact, this is very aligned with an existing procedure. According to Clause 5.3.3.4 of TS 38.322, the specification has already provided some guidelines about if a UE can trigger retransmission when t-PollRetransmit expires, and which RLC SDU can be selected to be considered for retransmission. Hence, in our opinions, reusing such framework makes a lot of sense in terms of performance benefits and specification impacts.

An exemplary text proposal below shows how simple the specification change is for this complementary mechanism. Basically, only one additional sentence is needed in Clause 5.3.3.4 of TS 38.322 (Note that the number of retransmissions that can be triggered can be further discussed, it does not have to be N>=1):

#### 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider N>=1 delay-critical RLC SDUs which have not been positively acknowledged for retransmission, if delayCriticalReTx is configured; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Proposal 4: The transmitter should consider delay-critical RLC SDUs for retransmission upon expiration of t-PollRetransmit.

On the other hand, we also think it makes some sense to have different values of poll retransmit timer based on the poll triggering event. If the poll was triggered based on the presence of delay-critical RLC SDUs, the UE may apply a shorter poll retransmit timer, in order to enable faster transmissions of the delay-critical RLC SDUs.

Proposal 5: The transmitter should apply a shorter t-PollRetransmit if the poll is triggered based on the presence of delay-critical RLC SDUs.

# Conclusions

In this paper, we have presented our views on RLC-AM enhancements, with the following observations:

Observation 1: It is questionable how the network can timely allocate sufficient UL resource for autonomous RLC retransmission, especially when the UE concurrently triggers autonomous retransmission for a large number of packets (e.g. packets of a PDU Set).

Observation 2: Regardless of how quickly/frequently autonomous RLC retransmission is triggered, if there is no available/sufficient radio resource, we cannot ensure retransmission can be performed immediately, so the benefits in terms of latency cannot be guaranteed.

Observation 3: Triggering autonomous retransmission in an impetuous manner (e.g. directly consider a RLC SDU for retransmission as soon as its remaining time becomes smaller than a threshold) may make the congestion even more severe.

Observation 4: Faster RLC Retransmission can be achieved with Polling Enhancement in UL, as long as the gNB can provide status report in a timely manner. However, this cannot always be guaranteed.

Based on the observations, we propose the following:

Proposal 1: To minimize the impacts of Autonomous Retransmission to capacity, RAN2 may consider the following some of the following options:

- Option 1: The UE only applies Autonomous Retransmission as a last resort, i.e. when the transmitting window is stalled, and status feedback cannot be received on time.

- Option 2: The UE only applies Autonomous Retransmission to certain packets, e.g. PDCP SDUs that are not obsolete yet.

- Option 3: The UE is configured with a maximum number of pending autonomous retransmissions.

Proposal 2: To ensure/facilitate timely radio resource allocations, the UE may trigger BSR when Autonomous Retransmission is triggered for a certain amount of data.

Proposal 3: For Polling Enhancements, RAN2 can consider introducing new poll triggering conditions based on the presence of delay-critical RLC SDUs.

Proposal 4: The transmitter should consider delay-critical RLC SDUs for retransmission upon expiration of t-PollRetransmit.

Proposal 5: The transmitter should apply a shorter t-PollRetransmit if the poll is triggered based on the presence of delay-critical RLC SDUs.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #128 Chair’s Notes, Orlando, USA, November 2024.