3GPP TSG-RAN WG2 Meeting #129	R2-2500682

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.6

Source:	Apple

Title:	Views on XR Rate Control

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

During RAN #105, the following new objective has been approved for Rel-19 WI on XR for NR Phase 3 [1]:

-	Specify uplink congestion signalling [RAN2]:

-	Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

In previous meetings, we have started the discussions on this topic, and we have reached the following agreements [2]:

- RAN2 #127bis Agreements: FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - - Extend the Bit Rate field - - Define a new bit rate table to provide sufficient granularity for XR traffic - - Introduce new values for the bitRateMultiplier - RAN2 #128 Agreements: RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

# Discussions

## UE Assistance Information for per-QoS Flow Rate Throttling

To enable per QoS flow throttling based on DL message, we think it is essential for the gNB to know which of the UL QoS flows can be throttled. Obviously, not all applications can tolerate degradation of data rate in the presence of network congestion. Since the scope is focused on UL, we think the UE is well-positioned to provide such assistance information, as the UE knows the best about the instantaneous application activities. In Rel-18 XR, we have already introduced the UE Assistance Information (UAI) relating to uplink traffic information, i.e. ul-TrafficInfo, which allows the UE to indicate per QoS flow information including: UL jitter range, Burst Arrival Time, and PDU Set identifiability. We think this UAI can be reused for the UE to indicate which QoS flow has the flexibility of rate throttling under congestion.

Proposal 1: The UAI of ul-TrafficInfo can be extended to indicate whether the bit rate for a QoS flow could be throttled.

In addition to the indication of whether the rate can be adapted for a QoS flow or not, we think this is also useful for the UE to indicate the minimum bit rate that the Application can tolerate for this QoS flow. Even if some Applications can continue to operate with reduced rate, it may not acceptable if the rate is lower than a minimum level. Hence, the gNB can also take this into account when making the recommendation of bit rates.

Proposal 2: The UE may further indicate the minimum acceptable bit rate for a QoS flow.

## Flexible Indication of Bit Rate Multiplier

For rate indication per QoS Flow, we think it is helpful if the rate multiplier can be signalled in a more flexible manner. Currently, for the Recommended Bit Rate MAC CE, a bitRateMultiplier value chosen from the set {x40, x70, x100, x200} is configured for a logical channel. Then, the indicated rate for the concerned logical channel should be multiplied by the configured bitRateMultiplier value to obtain the actual bit rate recommendation. Due to the dynamic nature of XR applications and network congestion status, we think the multiplier value could be dynamically signalled in the MAC CE, rather than fixed to a pre-configured value.

In particular, each QoS flow may be configured with a specific set of possible rate multiplier values, and the MAC CE can be used to indicate one particular multiplier value out of this configured set. This naturally provides a flexible framework to cope with bit rate recommendation in dynamically changing environments.

Proposal 3: RAN2 can consider the direction based on dynamic indication of Bit Rate Multiplier in the MAC CE.

# Conclusions

In this paper, we presented the following proposals for XR rate control:

Proposal 1: The UAI of ul-TrafficInfo can be extended to indicate whether the bit rate for a QoS flow could be throttled.

Proposal 2: The UE may further indicate the minimum acceptable bit rate for a QoS flow.

Proposal 3: RAN2 can consider the direction based on dynamic indication of Bit Rate Multiplier in the MAC CE.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #128 Chair’s Notes, Orlando, USA, November 2024.