3GPP TSG-RAN WG2 Meeting #129	R2-2500715

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.3

Source:		Lenovo

Title:			Enabling TX/RX for XR during measurement gaps/restrictions

Document for:	Discussion and Decision

# Introduction

This contribution focuses on the following objective identified by the work item description (RP-241771):

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

  - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In the following, we provide our views on potential RAN2 impacts.

# Discussion

The following working assumption was made in RAN1#118:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

In RAN2#127bis some initial discussions took place with the following agreements

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

In RAN2#128 some further discussions took place with the following agreements：

1. No MG-specific enhancements is needed on DSR operation. 2. RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped 3. No MG-specific enhancements is needed for DRX operation.

In RAN4#113 a response LS has been agreed in R4-2420198, which mainly confirms that Alt 1-1 in LS R1-2407561 is feasible from RAN4 perspective with X=5ms as baseline, and X=3ms with optional UE capability.

In our understanding, based on RAN1/RAN4 discussions, the skipping of measurement gaps may be restricted to specific types of measurement gaps or only applicable to specific measurement gap configurations in order to minimize the impact on the accuracy of RRM measurements. Furthermore, the number of consecutive MG skipping indications which UE is expected to receive may be limited, e.g. UE is not expected to receive more than X consecutive MG skipping indications. Those restrictions when being introduced by RAN4/RAN1 will have some impact to the RAN2 specifications, e.g. new RRC signalling implementing the MG skipping restrictions.

Observation 1: Based on RAN4/RAN1 decisions, new RRC signalling may need to be introduced to enable a restriction of MG skipping, e.g. MG skipping based on DCI is only applicable to specific MG configurations or MG types.

However, in our view, RAN2 should wait for further progress in RAN1/4 before starting to discuss those new RRC configurations enabling MG skipping restrictions.

Proposal 1: RAN2 to wait for further progress in RAN1/4 before discussing new configurations, e.g. measurement gap skipping restrictions.

RAN1 also discussed about new UE assistance information related to measurements occasions and related channel conditions, traffic, UE mobility. As shown in the agreement below, RAN1 discussed whether to introduce some new UE assistance information for solutions to enable /x/Rx in measurement gaps/restrictions that are caused by RRM measurements. Since no conclusion on the feasibility and usefulness of such UAI could be reached in RAN1 a related LS was sent in R1-2405736 (Cc RAN2). Depending on whether new UAI signalling will be supported based on input from RAN4, RAN2 will have to implement the new UAI reporting in TS38.331.

RAN1#117 Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

Proposal 2: RAN2 to wait for further progress in RAN4 before discussing the introduction of new UAI signalling for enabling Tx/Rx in gaps/restrictions that are caused by RRM measurements.

In our view some new UE capability signalling should be introduced indicating the support of Measurement gap skipping. Similar to other features like PDCCH skipping, also MG skipping based on the reception of a DCI to enable data reception/transmission during such measurement gaps may not be supported by all UEs.

Proposal 3: RAN2 to discuss the introduction of UE capability signalling for the skipping of measurement gaps, e.g. based on DCI indication.

## Impact of MG skipping to Layer2 procedures

If UE is configured with measurement reporting events, UE should evaluate whether the measurement report event is fulfilled during the TimeToTrigger period associated with the event. After entering condition for a measurement report event being satisfied, UE continues to evaluate the event even if no latest measurement result is obtained due to the cancelled measurement gap occasion. Obviously, it may cause the false evaluation result when UE moves between the cell edge of two cells. For example, UE triggers Measurement Report (MR) based on the inaccurate evaluation and NW may trigger the handover procedure. In addition, the similar issue may happen to the conditional configuration event evaluation procedure. In legacy measurement procedure, the measurement based on measurement gap is only cancelled due to random access procedure or gap collision. The probability to cancel a measurement gap is quite low. However, with R19 MG cancelling function, the probability to cancel a gap occasion becomes higher. In RAN4#113, RAN4 has discussed impacts on measurement requirements due to a cancelled MG occasion, and majority supports extending measurement period and reuse existing measurement accuracy requirements. For example, to achieve the same number of measurements, the measurement period associated with a cancelled MG occasion is extended according to the cancelled MG occasion similar as a MG occasion dropped by a high priority MG occasion. Therefore, if RAN4 agrees to extend the measurement period according to the cancelled MG occasion, it is proposed RAN2 to discuss the impact of a cancellable MG to the measurement event evaluation procedure, whether to enhance the measurement event evaluation procedure, e.g., enhance TimeToTrigger due to a cancelled MG occasion.

Proposal 4: RAN2 to discuss impacts of a cancellable MG to the measurement event evaluation procedure, e.g. enhance TimeToTrigger due to a cancelled MG occasion.

In addition to the DCI-based skipping solution, it was also proposed to consider a semi-static solution where RRC signalling is used to enable Tx/Rx in gaps/restrictions.  In particular for cases where the arrival time of XR traffic is quasi-periodic and CG/SPS scheduling is used, the usage of a DCI-based solution may increase the signalling overhead and also UE power consumption unnecessarily. Since in Rel-18, CG used for UL XR traffic and SPS used for DL XR traffic were considered as important scheduling mechanisms for XR, some proposals for semi-staic MG skipping solution may be considered.

It should be noted that RAN4 is also discussing the need to introduce semi-static indication-based solutions to skip measurement gap occasions. Therefore, we think that RAN2 should wait for RAN4 feedback on this before further discussing the topic.

Proposal 4: RAN2 to wait for further progress in RAN4 before discussing semi-static indication-based solutions to skip measurement gap occasions

# Conclusion

In this contribution, we discuss potential impacts of RRM measurement gap skipping to RAN2. We have the following proposals:

Observation 1: Based on RAN4/RAN1 decisions, new RRC signalling may need to be introduced to enable a restriction of MG skipping, e.g. MG skipping based on DCI is only applicable to specific MG configurations or MG types.

Proposal 1: RAN2 to wait for further progress in RAN1/4 before discussing new configurations, e.g. measurement gap skipping restrictions.

Proposal 2: RAN2 to wait for further progress in RAN4 before discussing the introduction of new UAI signalling for enabling Tx/Rx in gaps/restrictions that are caused by RRM measurements.

Proposal 3: RAN2 to discuss the introduction of UE capability signalling for the skipping of measurement gaps, e.g. based on DCI indication.

Proposal 4: RAN2 to discuss impacts of a cancellable MG to the measurement event evaluation procedure, e.g. enhance TimeToTrigger due to a cancelled MG occasion.

Proposal 4: RAN2 to wait for further progress in RAN4 before discussing semi-static indication-based solutions to skip measurement gap occasions

# References

### TS 38.321 v18.0.0