3GPP TSG-RAN WG2 Meeting #129	R2-2500721

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.1

Source:		Lenovo

Title:			Enhanced Logical channel prioritization for XR

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss enhancements to the uplink scheduling for XR traffic. A detailed look is taking to the  logical channel prioritization (LCP) procedure.

# Discussion

The Rel-19 Work Item on XR Phase 3 was agreed in RP-240791.

The Rel-19 XR Phase 3 objectives are as follows: Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2].

In RAN2#126 the following agreements were reached:

- Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. - The solution should consider impact on UE complexity (as already indicated in SI objective description) - For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - Use additional priority configured to LCHs in case of these LCHs with delay-critical data - FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH - We try to avoid RAN1 impacts. - RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. - LCP prioritization within a logical channel will not be considered in RAN2 discussions - FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). - Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 the following agreements were reached:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127bis the following agreements were reached:

- As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation. - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

In RAN2#128 the following agreements were reached:1. As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. 2. As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

Enhanced LCP procedure

- In RAN2#126bis meeting it was concluded that the option of overriding/adjusting the priority of a LCH during LCP based on the remaining time of the data of a LCH is used as a baseline for a delay-aware LCP procedure. UE may use an additional LCH priority configured for a LCH in case this LCH has delay-critical data available for transmissions. Alternatively, the UE may be configured with an offset by how much the priority of a LCH is increased, e.g. LCH priority value is decreased by this offset, in case delay-critical data is pending for transmission. Both implementation options are very similar in our understanding, and it might be slightly simpler to configure a second LCH priority for a LCH which is eligible for a delay-aware prioritization mechanism.

Proposal 1: NW can configure a second logical channel priority for a LCH to prioritize delay-critical data of the LCH.

For the agreed baseline solution of an enhanced delay-aware LCP procedure, there could be different flavors on how the enhanced LCH prioritization is implemented. The general approach would be to adapt the LCH priority based on the remaining delay time of data within the LCH and perform the legacy LCP procedure with the (adapted) LCH priorities. UE may for example increase the priority of a LCH in case the remaining delay/time of data of this LCH is becoming lower than a threshold or use a second configured priority. It should be noted that RAN2 would need to further discuss when exactly MAC entity determines the priority of a LCH for the LCP procedure, e.g. when MAC determines which LCH priority to apply. MAC may for example only determine once the priority of a LCH - by considering the remaining time of data buffered for this LCH - before the (legacy) LCP procedure is executed. This determination step would be similar to determining a delay-critical LCH as mentioned above.

In order to provide more fairness, MAC may further determine the priority of a LCH additionally before executing the second step of the LCP procedure, i.e. assigning UL resource in a strict priority order regardless of Bj. This would ensure that a LCH is not further prioritized, e.g. using the second (higher) LCH priority, during the second step of the LCP procedure if all the delay-critical data of a LCH has been already multiplexed in the MAC PDU during the first step of LCP. RAN2 had some discussions and conclusions in RAN2#128 on whether to (re)-determine the LCH priority before the second round of the LCP procedure. RAN2#128 introduced some optional capability, which allows UE to fallback to the default priority in the 2nd round of LCP, e.g. when there is no delay-critical data left in the UE buffer for the corresponding LCH. Baseline behaviour is that UE uses the LCH priority throughout the complete LCP procedure. In particular the additional complexity caused by this re(determination) step was seen as a concern by some companies. In our opinion, the complexity of determining whether there is some delay-critical available for transmission after the first round of LCP procedure is rather a simple task for the UE. Since UE has anyway to quantify the amount of delay-critical data pending for transmission for a LCH before the execution of the LCP procedure and obviously UE/MAC is aware of the amount of data being allocated to a LCH in the first round of LCP, e.g. to update Bj, it is rather straight forward to check whether there are any remaining delay-critical data in the UE after the first round of LCP. Also, it should be noted that since LCP is a time-critical procedure, MAC doesn’t need to consider new indications of delay-critical data from PDCP while LCP procedure is being executed.

In our view it would be better to avoid introducing too many UE capabilities. Furthermore, since it is important to consider fairness for the enhanced LCP procedure, we prefer a baseline behaviour, where MAC determines the LCH priority to apply before every step of the LCP procedure. Therefore, we propose that RAN2 rediscusses the RAN2#128 agreements on enhanced LCP procedure.

Proposal 2: RAN2 to agree that UE/MAC determines the priority of a LCH before the second round of the LCP procedure to ensure more fairness, e.g., MAC determines whether there is delay-critical data pending for transmission before the second round of LCP.

RAN2 needs to also further discuss how to define a delay-critical LCH, e.g. when UE should apply a higher LCH priority for a LCH during LCP. In our understanding a LCH is considered a delay-critical LCH and should use a second higher LCH priority if there is data available for transmission for the LCH for which the remaining time is less than a configured threshold. In RAN2#127bis it was already agreed that the threshold for determining a delay-critical LCH is separate to the delay threshold used for the triggering of a DSR. The reason is that the DSR related remaining Time threshold needs to account for the delay incurred by SR transmission and reception of corresponding UL grant.

Proposal 3: RAN2 to agree that a delay-critical LCH applying a delay-critical second priority during LCP (if configured) is defined as a LCH having at least one PDU/PDU set available for transmission for which the remaining time till discardTimer expiry is less than a new configured threshold.

We think that NW should control/configure if and for which LCHs the remaining time should be considered when executing the LCP. For example, the remaining time doesn’t need to be considered during LCP for every kind of service/LCH, e.g. only for specific LCHs carrying XR traffic. As another example, the enhanced LCP procedure may be only used when DSR has been reported to the gNB in order to ensure that delay-critical data – as reported in the DSR – is transmitted in the corresponding UL grant, e.g. received in response to the DSR. It should be also further discussed whether the enhanced LCP procedure is explicitly activated/deactivated by NW, e.g. similar to the PSI-based discarding procedure by a MAC CE.

Proposal 4: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE. RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

The PDU set importance based PDCP discard has been introduced for Rel-18 XR. In case of UL congestion, PSI based discard can be activated, and a lower discard timer value, e.g. discardTimerForLowImportance is applied to the low importance PDCP SDUs.  When PSI based discarding is activated, UE only supports DSR reporting for high importance data. Following the same principle, when LCP procedure considers the remaining time/delay of data prioritization, the enhancement may also only be applied to high importance data, e.g. delay-critical LCH that has high importance data in its buffers.

Proposal 5: RAN2 to discuss whether the LCP enhancements are only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

The current LCP procedure has two stages, UE allocates in the first round uplink resources based on Bj in a logical channel priority order. If any UL resources remain after the first round, UE allocates the remaining resources regardless of Bj in a decreasing priority order during the second round.  It was proposed by some companies to also consider Bj when a higher LCH priority is applied during first round of LCP for delay-critical data. It was argued that if the data volume of the delay critical data is larger than Bj, it may happen that not the complete delay-critical data may be multiplexed into the TB during the first round of LCP which would diminish the benefits of the enhanced delay-aware LCP procedure. However, in our opinion, the issue mentioned by some companies is not critical. Since UE has anyways the chance to multiplex delay-critical data also during the second round of LCP into the UL grant, we don’t think that Bj needs to be specifically addressed for the enhanced LCP procedure. It should be noted that it would be basically unfair towards other LCHs if Bj is increased beyond the normal update process. Bj was introduced to ensure fairness and avoid starvation of low priority LCHs. Furthermore, it should be also noted that UE implementation has already today the freedom to use a negative value for Bj. Therefore, in our understanding the current LCP procedure is already flexible enough to address the issue brought up by some companies. Limiting (hard) factor of the LCP procedure is the LCH priority, and this aspect will be already taken care of by the enhanced LCP procedure agreed for Rel-19. Nevertheless, we are OK to further discuss this issue in RAN2 and see whether there are strong support to further enhance the LCP procedure w.r.t. handling of Bj. However as already agreed before, additional complexity should be considered for any LCP enhancements.

Proposal 6: RAN2 to further discuss whether to introduce enhancements for the handling of Bj during the enhanced LCP procedure.

Intra-UE prioritization

Similar to the considerations for the LCP procedure, also for the intra-UE prioritization mechanism which was introduced in Rel-16 it should be also discussed whether to consider the remaining time/delay of the data as a decisive parameter determining the priority of UL grant. According to the current specification the priority of an overlapping UL grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU. Essentially only the logical channel priority is considered to determine which UL grant is prioritized/deprioritized.

However, it doesn’t make sense to prioritize an UL grant which carries data for which the PSDB/PDB is already exceeded. Such UL grant should be rather deprioritized. Similarly, a MAC PDU/UL grant which carries delay-critical data or a DSR MAC CE should be treated as a prioritized grant in the intra-UE prioritization procedure.

Proposal 7: RAN2 should consider enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay of data when determining the priority of an UL grant (prioritized/deprioritized UL grant).

# Conclusion

In this contribution, we discuss enhancements to the uplink scheduling. We have the following proposals:

Proposal 1: NW can configure a second logical channel priority for a LCH to prioritize delay-critical data of the LCH.

Proposal 2: RAN2 to agree that UE/MAC determines the priority of a LCH before the second round of the LCP procedure to ensure more fairness, e.g., MAC determines whether there is delay-critical data pending for transmission before the second round of LCP.

Proposal 3: RAN2 to agree that a delay-critical LCH applying a delay-critical second priority during LCP (if configured) is defined as a LCH having at least one PDU/PDU set available for transmission for which the remaining time till discardTimer expiry is less than a new configured threshold.

Proposal 4: NW should control/configure whether the enhanced LCP procedure taking remaining delay/time into account is used or not by the UE. RAN2 to further discuss whether enhanced LCP procedure is explicitly activated/deactivated by NW.

Proposal 5: RAN2 to discuss whether the LCP enhancements is only applicable for high importance packets in case of UL congestion, e.g., PSI-based discard is activated.

Proposal 6: RAN2 to further discuss whether to introduce enhancements for the handling of Bj during the enhanced LCP procedure.

Proposal 7: RAN2 should consider enhancements to the intra-UE prioritization procedure, e.g. considering the remaining delay of data when determining the priority of an UL grant (prioritized/deprioritized UL grant).

# References

### TS 38.321 v18.0.0