3GPP TSG-RAN WG2 Meeting #129	 R2-240XXXX

Athens, Greece, 17th - 21st February 2025

Agenda Item:	8.3.4

Source:	KDDI Corporation

Title:	Discussion on simulation metric for RLF prediction

SID:	FS_NR_AIML_Mob – Release 19

Document for: 	Discussion and decision

- Introduction

In this contribution we provide our consideration on a metric for RLF prediction, with reference to the agreements [1] and Apple’s [2] contribution in RAN2 #128 meeting. The following agreements were endorsed in RAN2 #128 meeting.

Agreements on event prediction … 4 To agree following definition for true event prediction, false event detection and missed event detection for indirect measurement event prediction Counter n3(true event prediction): it increases by 1 when a real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] or vice versa Counter n1(false event detection): it increases by 1 when no real event occurs around a predicted event with ETD, whose range is [0, maximum ETD] Counter n2(missed event detection): it increases by 1 when no event is predicted around a real event with ETD, whose range is [0, maximum ETD] 5 To agree following definition for true event prediction, false event detection and missed event detection for direct measurement event prediction Counter n3’ (true event prediction): it increases by 1 when a real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n1’ (false event detection): it increases by 1 when no real event occurs within the occurrence window of predicted event whose possibility is higher than a predefined threshold Counter n2’ (missed event detection): it increases by 1 when a real event occurs, but it doesn’t fall in the occurrence window of any predicted event whose possibility is higher than a predefined threshold 6 Agree following definition of F1 score: F1 score = 2*Precision*Recall/(Precision + Recall) Formula_3 (13/14) 7 In addition to F1 score, precision and recall in following formula are optional metrics to report. (12/14) Precision = n3/(n1+n3) Formula_1 Recall =n3/(n2+n3) Formula_2 Note: for direct prediction, the counter should be n1’,n2’,n3’ Agreements on RLF prediction: 1 To agree following indirect RLF prediction definition: The future L1 SINR results are predicted based on actual L1 SINR results of the serving cell by following same way of intra-frequency temporal domain case A. Afterwards, RLF event at one time instance is determined based on predicted and optionally actual L1 SINR results within T310 duration, without further AI/ML models. As baseline L1 SINR refers to raw L1 SINR without L1 filtering. 2 Reuse agreed metrics for measurement event prediction for RLF prediction for both indirect and direct prediction respectively

- Discussion

2.1 Simulation metrics for RLF in temporal domain case A

As written in above agreements, metrics for RLF prediction are F1 score, precision and recall based on counter n1, n2, and n3 for indirect prediction, and counter n1’, n2’, and n3’ for direct prediction.

Let’s focus on temporal domain case A of indirect prediction, in our understanding, counter n1, n2, and n3 are illustrated as follows.

Figure 1. Metrics counters

In addition, in our view, time series data is evaluated as follows in temporal domain case A.

Figure 2. Event prediction in temporal domain case A

Intuitively, in such sequential evaluation, the counters increment occurrences tend to be concentrated in localized point, as follows.

Figure 3. Concentrated count

In addition, without handover because of non-SLS simulation, continuous occurrences of true event (RLF) may happen and increase localized counters increment (this may depend on how each company handles continuous OOS without handover.)

Our concern is whether there is a consensus among companies on such evaluation. This concentrated increase of counter can lead to over- or under-estimation of AI/ML performance. Of course, suitability of the metric depends on the objectives of the study.

From a perspective of actual commercial operation, on the contrary, this sequential evaluation is reasonable because the system tries to find RLF or another event sequentially. Anyway, we don’t know representative methods for continuous (indirect) classification tasks on time series data.

Temporal domain case A of RLF prediction, companies should confirm whether sequential data set evaluation is reasonable for study goal or not.

The point is that such evaluation is beneficial for actual operation or preparation of SLS.

On the other hand, we agree with that scenario and evaluation methodology should not be too much complex for early stage. If any company has strong concern that the evaluation of sequential data set is inappropriate, and has alternative evaluation idea with simple expression, which can be discussed.

The alternative evaluation idea to sequential data set evaluation is open for discuss.

The above discussions may be extended to direct prediction or other use cases.

2.2 Data set imbalance

We agree with Apple’s consideration on data set imbalance [2] where only small proportion of time is RLF (or measurement event) in whole datasets. Because this is also related to above concern, we think the ratio of the true RLF in overall time is also important information for this study. It is recommended that ratio of the true RLF in overall time be included in the results in some form. The counter n1, n2 and n3 itself could be a substitute for the ratio.

Ratio of the true RLF in overall time or the value of n1, n2 and n3 should be included in the result.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1　Temporal domain case A of RLF prediction, companies should confirm whether sequential data set evaluation is reasonable for study goal or not.

Proposal 2　The alternative evaluation idea to sequential data set evaluation is open for discuss.

Proposal 3　Ratio of the true RLF in overall time or the value of n1, n2 and n3 should be included in the result.

References

- R2_128_ChairNotes_24-11-22_17-00_eom

- R2-2409972, Model generalization, RLF evaluation assumptions, etc., Apple