3GPP TSG-RAN WG2 Meeting #129	R2-2500735

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.3

Source:	OPPO

Title:	Consideration on on-demand SIB1

Document for:	Discussion, Decision

Introduction

This paper outlines our further views on on-demand SIB1, which falls under the following objective.

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

Discussion

Validity of UL WUS configuration

RAN2 has decided to implement a new SIB, called SIB-X, to include the UL WUS configuration for multiple NES cells. The cells providing the UL WUS configuration could be Cell A and NES cells.

RAN2#126: - Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB RAN2#127bis: - A NES cell can include neighbouring NES cell’s WUS configuration. - NES cell’s WUS configuration, it is included in a new SIB (including its own WUS configuration).

The UE must assess the validity of the UL WUS configuration after it is obtained. The validity issue of UL WUS configuration was discussed in RAN2#128, leading to the following agreement.

RAN2#128: - UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

In legacy, after acquiring SIB1, the UE can use validity indication in SIB1 to verify whether the stored OSI remains valid. The validity indication parameters include areaScope, valueTag, etc.

The UE shall: 1> delete any stored version of a SIB after 3 hours from the moment it was successfully confirmed as valid; 1> for each stored version of a SIB: 2> if the areaScope is associated and its value for the stored version of the SIB is the same as the value received in the si-SchedulingInfo for that SIB from the serving cell: 3> if the UE is NPN capable and the cell is an NPN-only cell: 4> if the first NPN identity included in the NPN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 5> consider the stored SIB as valid for the cell; 3> else if the first PLMN-Identity included in the PLMN-IdentityInfoList, the systemInformationAreaID and the valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the systemInformationAreaID and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell; 2> if the areaScope is not present for the stored version of the SIB and the areaScope value is not included in the si-SchedulingInfo for that SIB from the serving cell: 3> if the UE is NPN capable and the cell is an NPN-only cell: 4> if the first NPN identity in the NPN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the NPN identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 5> consider the stored SIB as valid for the cell; 3> else if the first PLMN-Identity in the PLMN-IdentityInfoList, the cellIdentity and valueTag that are included in the si-SchedulingInfo for the SIB received from the serving cell are identical to the PLMN-Identity, the cellIdentity and the valueTag associated with the stored version of that SIB: 4> consider the stored SIB as valid for the cell;

Assume that the Rel-19 NES UE intends to reselect to an NES Cell, called Cell1, after receiving SIB-X from its current cell, Cell A. Once the UE acquires SIB 1 from Cell 1, it can check the validity indication for the OSI, such as valueTag and areaScope, if provided. It allows the UE to verify whether the stored OSI is still valid. If we permit the UE currently camping on Cell 1 to check the validity of SIB-X obtained from Cell A, it eliminates the need for the UE to re-acquire a new SIB-X each time it reselects to a new cell. Meanwhile, this approach requires minimal standardization effort since the existing SIB validity mechanism can be easily utilized.

RAN2 discusses if the legacy SIB validity mechanism (e.g. valueTag and areaScope) can be used to check the validity of SIB-X. If yes, after UE reselects from Cell A to the NES cell, UE decides whether to reacquire SIB-X based on valueTag and areaScope as in legacy.

Handling of UEs in RRC_CONNNECTED

RAN2 previously discussed how to handle the UEs in RRC_CONNNECTED and raised this issue at RAN plenary.

- Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

RAN#106 has endorsed the following way forward and requests RAN WGs to support the specific case of RLF. The procedure for the relevant OD-SIB1 request is the same as that for UE in RRC_IDLE or RRC_INACTIVE.

RP-243297 Way forward on NES RAN2 chair (Interdigital) vivo: looks like an upscoping of the WI Nokia: thinks only vivo had the misunderstanding that it is an upscoping as no new procedure is defined Qualcomm: fine with the problem, we still have cell A to rely on? RAN2 chair: yes, it is covered by "careful deployment" so operators will have to take care of this The document was endorsed.

Way forward on NES in RP-243297 - On RRC Connected mode - The scope of WID will not be updated to include RRC Connected. - However, RAN2 will address the specific case of RLF, to use the same procedure as idle/inactive to request OD-SIB1 transmission. No need to change the scope of WI to indicate this.

In TS 38.331, when RLF occurs, the UE remains in the connected mode and performs cell selection instead of cell reselection.

Clause 5.3.10.3 5> else: 6> if the UE supports RLF-Report for fast MCG recovery procedure and if T316 is configured: 7> if the SCG is deactivated at the moment of detecting RLF in the MCG: 8> set the mcgRecoveryFailureCause in the VarRLF-Report to scg-Deactivated; 7> else if SCG transmission is suspended at the moment of detecting RLF in the MCG: 8> set the pSCellId in the VarRLF-Report to the global cell identity of the PSCell, if available, otherwise to the physical cell identity and carrier frequency of the PSCell; 8> set the scg-FailureCause value in the VarRLF-Report according to 5.7.3.5; 8> set the elapsedTimeSCG-Failure in the VarRLF-Report to the time elapsed between SCG failure and the MCG failure; 6> if T316 is configured; and Editor´s note: The use of scg-Deactivated cause. 6> if SCG transmission is not suspended; and 6> if the SCG is not deactivated; and 6> if neither PSCell change nor PSCell addition is ongoing (i.e. timer T304 for the NR PSCell is not running in case of NR-DC or timer T307 of the E-UTRA PSCell is not running as specified in TS 36.331 [10], clause 5.3.10.10, in NE-DC): 7> initiate the MCG failure information procedure as specified in 5.7.3b to report MCG radio link failure. 6> else: 7> initiate the connection re-establishment procedure as specified in 5.3.7. Clause 5.3.7.2 1> else: 2> if the UE is capable of L2 U2N Remote UE: 3> perform either cell selection as specified in TS 38.304 [20], or relay selection as specified in clause 5.8.15.3, or both; 2> else: 3> perform cell selection in accordance with the cell selection process as specified in TS 38.304 [20].

To address the specific case of RLF, the most straightforward approach is to apply the existing agreement to cell selection. If the UE falls in the re-establishing of the RRC connection, it is recommended that the UE follow the procedure outlined for RRC_IDLE/RRC_INACTIVE to re-acquire SIB-X (when needed).

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration.

For Rel-19 NES UE in RRC_CONNECTED occurring RLF, the UE considers WUS configuration acquired from the serving cell only valid in directly succeeding cell selection. In case of RRC connection reestablishment failure, follow the procedure used in RRC_IDLE/RRC_INACTIVE to re-acquire SIB-X (when needed).

Another case for the connected UE requiring SIB1 is for acquiring the OSI. In the legacy system, if searchSpaceSIB1 is not configured, the UE depends on the network-dedicated RRC for the delivery of SIB1. However, if searchSpaceSIB1 is configured, the UE can monitor the broadcasted SIB1. For Rel-19 NES UE in the connected mode, this principle can still be applied. Additionally, we need to determine how to request OD-SIB1 when the searchSpaceSIB1 is configured. To address this issue, we can consider one of the following options and we prefer Option a) as it is a unified solution applicable to UEs in all RRC states: a) Rely on UE to acquire SIB1 via Msg-1 based on-demand SIB1 procedure; 2) Rely on UE to acquire SIB1 via a dedicated-RRC request; c) Rely on the network dedicated RRC to deliver SIB1.

For Rel-19 NES UE in RRC_CONNECTED while T311 is not running, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured, otherwise, the UE acquires SIB1 via the on-demand SIB1 procedure used in RRC_IDLE/RRC_INACTIVE.

Conclusion

We have the following proposals:

Proposal 1	RAN2 discusses if the legacy SIB validity mechanism (e.g. valueTag and areaScope) can be used to check the validity of SIB-X. If yes, after UE reselects from Cell A to the NES cell, UE decides whether to reacquire SIB-X based on valueTag and areaScope as in legacy.

Proposal 2	For Rel-19 NES UE in RRC_CONNECTED occurring RLF, the UE considers WUS configuration acquired from the serving cell only valid in directly succeeding cell selection. In case of RRC connection reestablishment failure, follow the procedure used in RRC_IDLE/RRC_INACTIVE to re-acquire SIB-X (when needed).

Proposal 3	For Rel-19 NES UE in RRC_CONNECTED while T311 is not running, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured, otherwise, the UE acquires SIB1 via the on-demand SIB1 procedure used in RRC_IDLE/RRC_INACTIVE.

Reference

[1] RP-242354 Revised WID: Enhancements of network energy savings for NR