3GPP TSG-RAN WG2 Meeting #129	R2-2500736

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.7.6

Source:	OPPO

Title:	Discussion on XR Rate Control

Document for:	Discussion, Decision

Introduction

This paper discusses the following objective in Rel-19 XR WID [1]:

- Specify uplink congestion signalling [RAN2]: - Specify in MAC layer XR rate control signalling over downlink per QoS flow/per DRB to enable faster source rate adaption to uplink congestion.

Discussion

RAN2 previously discusses the granularity of the rate control indication for XR in the congestion case. The ongoing debate about “per QoS flow or DRB” in RAN2 has continued through several meetings, and RAN2#128 eventually concludes to implement QoS flow level indication.

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB.

The follow-up question is whether the QoS flow level information is contained in a MAC CE or RRC signalling. Some companies prefer RRC signalling since they are concerned that having the QoS flow level awareness at the MAC layer could negatively impact the CU/DU split architecture. However, according to TS 38.473, the DU can now obtain information about the QoS flow(s) and the associated DRB through the interaction with CU. As a result, the MAC layer can derive/generate a MAC CE that includes QoS flow identification to control the data rate with minimal effort. Conversely, using RRC to indicate the QoS flows need to be throttled seems inflexible, ineffective and slow when congestion happens. This is because the QoS flow that should be throttled can vary from one congestion scenario to another. For instance, during slight congestion, the network might adjust one QoS flow associated with a single DRB, while in case of severe congestion, it may need to adjust multiple QoS flows of the same DRB. Consequently, the network has to frequently update the relevant QoS flow(s) via RRC before sending a MAC CE, which introduces unexpected latency and increases signalling overhead.

Introduce a new MAC CE that includes QFI information to indicate uplink rate adaption.

The gNB currently lacks information about the traffic types associated with each QoS flow. To determine which QoS flow should be controlled, the gNB needs assistance from either UE or CN to identify which QoS flow can be throttled.

- gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN

We understand that both methods are acceptable, meaning that the assistant information can be obtained from either UE or CN. However, we slightly prefer obtaining it from the UE, as the UL data is generated/come from the UE side.

Rely on the UE to indicate to the gNB which QoS flows can be throttled.

Another issue discussed in RAN#128 is whether a UE is permitted/required to send its recommendation for UL rate adaption.

- FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

The extent to which the codec bit rate of a specific QoS flow can be adjusted significantly impacts on the user experience and is closely tied to the encoding algorithm that can be supported. Therefore, we suggest providing some flexibility for the UE to recommend its preferred UL data rate.

Allow the UE to indicate the recommended UL bit rate for a specific QoS flow to the gNB.

Conclusion

We have the following proposals:

Proposal 1	Introduce a new MAC CE that includes QFI information to indicate uplink rate adaption.

Proposal 2	Rely on the UE to indicate to the gNB which QoS flows can be throttled.

Proposal 3	Allow the UE to indicate the recommended UL bit rate for a specific QoS flow to the gNB.

Reference

[1] RP-243318 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)

[2] RP-242827 Extension of XR WID scope for rate control, Xiaomi, Qualcomm Incorporated, Sony, ZTE Corporation