3GPP TSG RAN WG2 Meeting #129			R2-2500738

Athens, Greece, February 17th – 21st 2025

Agenda Item: 	8.2.2

Source:		Sony

Title:			Considerations on paging for Ambient IoT

Document for:	Discussion & Decision

# Introduction

At RAN#106 meeting, a new WID for Ambient IoT in NR was approved including the following RAN2 objectives [1].

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we discuss our view on some RAN2 related aspects related to subsequent paging.

# Discussions on subsequent paging for Ambient IoT

In legacy cellular systems, for the Network to reach the UE normally paging is used, for which a UE-id is needed. The UE-id is not only used for identification, but also to group the UEs in different wake-up groups for the system to page the UE without having to wake-up all UEs and for the UEs to save power, not having to listen for paging continuously.

In the case of Ambient IoT, the traffic assumptions have been defined as Device Terminated (DT), and Device Originated (DO). DO traffic only considers DO-DTT, Device Originated, Device Terminated Trigger, which means that the network wakes up the device but for expected or required uplink traffic. This could e.g., be sensor data that should be reported.

One issue that has been identified in the study item [3GPP TR 38.769 V2.0.0 (2024-12)] [2], is when the reader sends multiple subsequent paging messages associated with the same service request [ chapter 6.3.3].

The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader.

In this paper we discuss and list different alternatives on what type of information that can be included in the Paging response message to reduce and/or minimize duplicated responses from the AIoT device.

Proposal 1: Discuss different alternatives and options to avoid or minimize duplicated responses for the same service request.

The overall procedure between the A-IoT device and reader is according to figure X, [2].

Figure 6.3.1-1 Overall AS procedures between A-IoT device and reader

Uplink Response:

As a first option when the A-IoT device receives a subsequent paging message and detects it is for the same service request and is a repeated paging message. The AIoT device responds again, but it does not include any data or trigger any subsequent action if same as in the initial paging message, but it only replies with a simple indication, that the A-IoT device has already responded. The indication could be as simple as a one bit:

Already responded [1 bit]. TRUE Conditional

Proposal 2: The paging message includes information, that the AIoT device can respond with a one bit indication

Alternatively the Reader can include a repetition number in the A-IoT Paging message, and when the A.IoT device receives a subsequent paging message for the same service, the device responds with a repetition number, and does not include the same data again. Hence, the subsequent response message will be smaller in size.

Proposal 3: The paging message includes information such a repetition number, that the AIoT device can associate its response with.

Downlink Ack:

As a third option, the subsequent paging message includes either those devices that have responded, or alternatively those devices that have not responded.

This would not be as part of any RACH or STEP B procedure but included in the subsequent A-IoT paging message. For the case where the device -ID is included in the subsequent A-IoT paging message, the corresponding devices can simply ignore to respond again. The behavior can be indicated either that service or reader id is repeated, or a flag is set indicating that those A-IoT devices whether the device id is included can refrain from responding.

For the other case, the reader has prior information on what devices are to be paged and can then include the ID of those devices that have not responded to the initial A-IoT paging message. For the initial paging message, only group or type ID is included, to limit the message size.

Proposal 4: The paging message includes information, on which devices that has either responded or which have not responded.

Time window:

Additionally, a time window may be used as a fourth option or in conjunction with any of the alternatives above.

When the UE receives an A-IoT message is starts a timer, and if the device gets subsequent A.-IoT paging messages while the timer is running, the A-IoT device can ignore responding to those subsequent A-IoT paging messages for the same service request.

Proposal 5: During a configured time window, the A-IoT device can ignore responding to subsequent paging transmission for the same service request.

# Summary

In this contribution, we have discussed our views on aspects related to paging.

Proposal 1: Discuss different alternatives and options to avoid or minimize duplicated responses for the same service request.

Proposal 2: The paging message includes information, that the AIoT device can respond with a one bit indication

Proposal 3: The paging message includes information such a repetition number, that the AIoT device can associate its response with.

Proposal 4: The paging message includes information, on which devices that has either responded or which have not responded.

Proposal 5: During a configured time window, the A-IoT device can ignore responding to subsequent paging transmission for the same service request

# References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

- 3GPP TR 38.769 V2.0.0 (2024-12)]