3GPP TSG RAN WG2 Meeting #129			R2-2500741

Athens, Greece, February 17th – 21st 2025

Agenda Item: 	8.4.4

Source:		Sony

Title:			Considerations on LP-WUS/WUR in RRC Connected mode

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version for the WID for Low Power Wake-up Signal and Receiver was approved including the following objectives [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline Specify necessary RRM requirements-

At RAN2#126 [3], the following agreements were made related to LP-WUS in RRC Connected mode:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1.

For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

At RAN2#127 [4], the following agreements were made;

For option 1-2,

    - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.)

    - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

The options 1-1 and 1-2 has also been discussed in RAN1#118 [6].

At RAN1#118 [5], the following was agreed as a working assumption:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

At RAN2#127bis [6], the following agreements were made:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

At RAN2#128 [7], the following agreements were made:

Agreements on LP-WUS in RRC_CONNECTED

The drx-onDurationTimer is not started with Option 1-2 LP-WUS.

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS.

Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

In this contribution, we discuss our view on some aspects related to LP-WUS in RRC Connected mode.

# Discussions on LP-WUS/WUR in RRC Connected mode

## LP-WUS relation to drx_onDuration

In [2], it was discussed that there could be a benefit for power saving purpose for XR traffic to let the UE monitor LP-WUS instead of PDCCH monitoring while the drx_onDurationTimer is running, i.e., during C-DRX active time. The benefit would also be to handle unpredictable jitter, and hence reduce the uncertainty period of PDCCH monitoring. Though, since the Connected mode DRX periodicity is more statically configured, and that jitter potentially could also imply early or late arrival of packets, even outside PDCCH monitoring period, the potential wake-up signal should preferably not be related to the PDCCH monitoring period.

At RAN2#127 it was agreed that the timer for PDCCH monitoring is started a time offset after triggered by LP-WUS. I.e. option 1-2.

At RAN2#127bis it the difference between option 1-1 and option 1-2 was discussed

As we see it the first option consider a fixed legacy -C-DRX scheme, where the LP-WUS is monitored a time offset before the start of PDCCH monitoring. The other option is where the C-DRX scheme is adjustable, i.e. advanced or delay based on LP-WUS

How to implement option 1-1 was discussed in RAN1#119 and resulting in two approaches. The LP-WUS configuration shall be based on the configured C-DRX. For example, the LP-WUS is relatively with certain time-offset from the start of C-DRX (i.e., ON duration to monitor PDCCH). Hence, there is a dependency of LP-WUS configuration the C-DRX. Based on this observation, we prefer approach #2 (i.e., LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start). Approach #1 may offer some flexibilities as the LPWUS configuration is independent from C-DRX configuration. The independency aspect here may cause or create potential issues. The LP-WUS may be overlapped with ON duration timer or transmitted much earlier so that the power saving gain can be reduced.

LP-WUS option 1-1 with approach 2 is illustrated in Figure 1. The LP-WUS is transmitted or UE monitored the LP-WUS starting from a time-offset to the reference time (the start of C-DRX ON duration). The LP-WUS is transmitted with certain duration that may be across multiple slots (i.e., express as LP-WUS duration). The LP-WUS duration plus the min-gap shall be prior to the intended ON duration. The min-gap is the minimum time gap for the UE to start the ON duration (e.g., the transient period to activate the main receiver). The min-gap can also be seen as UE capability parameter.

Figure 1: LP-WUS Option 1-1 (Approach 2)

Proposal 1: For LP-WUS MOs in connected mode for Option 1-1, LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start (known as Approach 2).

## Adjustment of On-duration

Upon the reception of LP-WUS, the UE is to temporarily adjust the start of DRX ON operation. This is particularly to deal with the time-jitter that occurs in delay-sensitive XR application. Within a LP-WUS occasion, a UE can be configured with multiple multiple monitoring occasions (MOs) in which each occasion is associated with the starting time of DRX ON duration as shown in Figure 2.. The LO can be the same LO as the LO for the option 1-1. For example, in case of option 1-1, the gNB transmits LP-WUS at a given configured time offset within an LO. This triggers the UE to start the DRX ON at the T_reference. T_reference is basically the legacy DRX ON starting time. In case of option 1-2, the gNB shall transmit LP-WUS in one of the MOs. The LP-WUS reception in the early MO indicates the UE to start DRX_ON starting time earlier than the T_reference (i.e., a new ON duration for PDCCH monitoring) as shown in  Figure 2. Here, the starting time is T_offset earlier than the T_reference. The granularity of the T_offset can be in the range of number of slots and configured by gNB.

Figure 2: LP-WUS Option 1-2 (Approach 2)

Proposal 2: For option 1-2, the following are supported (known as Approach 2):

- Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

- LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

## Multiplexing

The LP-WUS resource is allocated prior to the PDCCH resource to be monitored by the UE. In case there are multiple UEs going to monitor the same timing of PDCCH resource, the LP-WUS resource allocation prior to the PDCCH resource can be multiplexed, such as in time and/or frequency. From the UE point of view, the UE is expected to monitor a given LP-WUS resource. The UE is not expected to monitor multiple LP-WUS resources as it will increase the UE power consumption.

Proposal 3: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource (i.e., LP-WUS occasion).

# Summary

In this contribution, we have discussed our views on aspects related to.LP-WUS in RRC Connected mode..

Proposal 1: For LP-WUS MOs in connected mode for Option 1-1, LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start (known as Approach 2).

Proposal 2: For option 1-2, the following are supported (known as Approach 2):

- Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

- LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

Proposal 3: The UE operating in RRC connected mode and configured to receive LP-WUS is expected to monitor a given LP-WUS resource (i.e., LP-WUS occasion).

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869 v18.0.0, Study on low-power wake up signal and receiver for NR

- Minutes RAN2#126

- Minutes RAN2#127

- Minutes RAN1#118

- RAN2#127bis

- RAN2#128