3GPP TSG-RAN WG2 Meeting #129  	                            R2-2500742

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.2

Source: 	Sony

Title:	On-demand SSB Scell operation discussion

Document for:	Discussion

# Introduction

In RAN2#127bis meeting, on-demand SSB Scell operation was discussed, and the agreements are as follows.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we discuss some open issues regarding the OD-SSB and its measurements.

# Discussion

2.1 Cell on/off indication via backhaul

RAN1#116 agreed to support on-demand SSB SCell operation triggered by gNB and cell on/off indication via backhaul is one of the candidate solutions. With the existing CU-DU architecture, it is the CU who is responsible for the cell on/off decision. In gNB-CU configuration update message sent from gNB-CU to gNB-DU, it contains cells to be activated/deactivated list which gNB-DU shall activate/deactivate the corresponding cells as indicated by the cell group and physical cell identity.

With the existing signalling, it is not possible to support dynamic cell on/off use cases where the cell on/off decision is made based on the current traffic and energy consumption status of the cell. It is the gNB-DU who is aware of the up-to-date status of the cells but the cell on/off indication comes from gNB-CU.

Cell on/off indication via backhaul will be a promising solution if it can be enhanced to support:

- Assistance information from gNB-DU to gNB-CU to help gNB-CU making cell on/off decision. We think this information is needed because gNB-DU has more awareness and gNB-DU should be involved in cell on/off decision as explained below.

- Certain level of autonomy is given to gNB-DU to allow it make its own cell on/off decision according to the cell status. The gNB-DU has the up-to-date information e.g., number of active UEs, radio resource utilization ratio, traffic status of the cell. Therefore, if certain level of autonomy is given to gNB-DU, it can make proper cell on/off decision based on the above cell status. But gNB-CU should always control this procedure and being informed of any gNB-DU’s decision.

RAN3 should lead the work and it may have some impacts on the assistance information which can be potentially supported by the UEs in the cell.

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

2.2 OD-SSB measurement framework

In RAN2#127bis meeting, the L3 RRM framework on OD-SSB measurement has been discussed and two options are listed as follows.

Option1: Based on different measurement configuration when OD-SSB is transmitted.

Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted.

Our understanding is that Option 1 is more aligned with the legacy measurement framework while Option 2 may introduce more specification work. Furthermore, the SMTC configurations in Option 1 will need more RAN1 input on the detailed OD-SSB pattern discussion. On the other hand, if the OD-SSB pattern is shared with UE, it will be beneficial to both options, e.g., the OD-SSB pattern will be helpful for UE to choose from the configured multiple SMTC configurations in Option 1 and OD-SSB pattern sharing is essential in Option 2.

Proposal 2: OD-SSB patterns are informed to UE from network.

2.3 L3 measurement in case 2

In case 2, with always-on SSB is periodically transmitted on the cell in addition to OD-SSB, it is still open whether always-on SSB and/or OD-SSB are measured. And we have two options as following:

Option 1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses always-on SSB.

Option 2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to always-on SSB for RRM measurements.

In RAN1#119 meeting, it was agreed that the frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB but still discussing for the case where always-on SSB is CD-SSB. Considering the always-on SSB with SSB adaption will have longer periodicity and the measurement only based on always-on SSB will degrade the measurement performance, it is beneficial to adopt Option 2 to achieve a reasonable measurement performance. But it may only be feasible (due to the specification impact) if RAN1 decides that on-demand SSB and always-on SSB are in the same frequency location.

Proposal 3: Wait RAN1’s decision on whether frequency location of OD-SSB and always-on SSB are the same to decide if Option 2 as following is considered in RAN2.

Option 2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to always-on SSB for RRM measurements.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: Enhancements to support dynamic cell on/off indication via backhaul should be supported and RAN3 should be informed about it.

Proposal 2: OD-SSB patterns are informed to UE from network.

Proposal 3: Wait RAN1’s decision on whether frequency location of OD-SSB and always-on SSB are the same to decide if Option 2 as following is considered in RAN2.

Option 2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to always-on SSB for RRM measurements.