3GPP TSG-RAN WG2 Meeting #129  	                            R2-2500743

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.3

Source: 	Sony

Title:	Remaining details for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN2#128 meeting, the agreements on OD-SIB1 were agreed as follows:

Agreements on OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

In this contribution, we discuss remaining aspects on OD-SIB1.

# Discussion

In RAN1#116 meeting, it was agreed that for study of UL WUS design, consider at least PRACH as a starting point. In Rel-15, on-demand SIB transmission is supported via IDLE/INACITVE UE to trigger a random access procedure. In MSG1 solution, a subset of the PRACH resources is associated with the requested SI, and network will reply a MAC subPDU with RAPID only for acknowledgement.

It is important that network can identify the request to transmit on-demand SIB1 from very beginning, in order to reduce the access delay for UE. And it was agreed in RAN2#125bis that UL WUS configuration includes at least RACH configuration. Therefore, the network should reserve PRACH resources that are associated with on-demand SIB1 transmission request.

Proposal 1: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

In RAN#106 meeting, it was agreed that RAN2 will address the specific case of RLF for RRC_CONNECTED UEs, to use the same procedure as idle/inactive to request OD-SIB1 transmission. The issue comes from during the RRC re-establishment procedure, if UE can’t get the OD-SIB1 from the candidate re-selected cell, it will bar this cell which may delay the RLF recovery procedure. According to the agreements, the same OD-SIB1 request procedure as agreed for IDLE/INACITV UEs should be re-used. Therefore, for a RRC_CONNECTED UE in RRC re-establishment procedure, if it can’t acquire OD-SIB1 from a NES cell, it will not bar the cell, but to initiate the OD-SIB1 request procedure according to the any stored UL WUS configuration if it is still valid. Some companies proposed that this behaviour (whether UE is allowed to send the request or not) should be configured by the network, which in our view is not necessary.

Proposal 2: In RRC connection re-establishment, UE will not bar the NES cell with OD-SIB1 due to the SIB1 is not being broadcasted. UE can send OD-SIB1 request to the selected candidate cell.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

Proposal 2: In RRC connection re-establishment, UE will not bar the NES cell with OD-SIB1 due to the SIB1 is not being broadcasted. UE can send OD-SIB1 request to the selected candidate cell.