3GPP TSG-RAN WG2 Meeting #129  	                            R2-2500744

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.5.4

Source: 	Sony

Title:	RAN2 impacts on SSB and RACH adaptations

Document for:	Discussion

# Introduction

The SSB adaptation and RACH adaptation was not discussed in RAN2 as it depends on the RAN1’s progress. In this contribution, we discuss some RAN2 impact on SSB adaptation and RACH adaptation based on latest RAN1’s progress.

# Discussion

2.1 SSB adaptation

The framework on how to deal with SSB adaptation has not been discussed in RAN2. RAN1 agreed that at least SSB burst periodicity is adapted for SSB adaptation, and the periodicity of on-demand SSB is one of 5ms, 10ms, 20ms, 40ms, 80ms or 160ms. It has been concluded in RAN1#118b meeting that there is no consensus on the support of adaptation of SSB for idle UEs in Rel-19 and further in RAN1#119 meeting, it was agreed that there is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell.

As can be observed that both on-demand SSB and SSB adaptation is for SCell connected UEs in Rel-19 NES. For on-demand SSB, RAN2 has agreed that a new MAC CE will be specified on on-demand SSB transmission activation/deactivation. It is beneficial that at least the same design principle can be re-used for SSB adaptation as well.

Proposal 1: RAN2 to consider a joint design of the MAC CE for both on-demand SSB and SSB adaptation.

Regarding the L3 measurement on the SSB adaptation, it has been agreed in RAN1#119 that SSB burst periodicity is adapted, and the frequency location as well as the spatial relation will be the same before and after SSB adaptation. And we think the same L3 measurement framework can be applied for both on-demand SSB and SSB adaptation, although RAN2 couldn’t make concrete agreement on L3 measurement for on-demand SSB as still waiting for more RAN1’s progress.

Proposal 2: The same L3 measurement framework can be applied for both on-demand SSB and SSB adaptation.

2.2 RACH adaptation

It was agreed in RAN1#116b that the time domain RACH adaptation is based on additional PRACH resources for NES -capable UEs in addition to PRACH resources for legacy UEs. RAN1 has further agreed to use DCI-based adaptation to trigger the additional PRACH resource. Then it is up to RAN2 to decide how the network will configure the additional PRACH resource and it is easy to conclude that the additional PRACH resources should be included in a SIB and further details e.g., to define a new SIB or to be included in SIB1 or any other existing SIB can be decided later.

Proposal 3: The additional PRACH resources for RACH adaptation should be included in a SIB.

Regarding whether the legacy UEs that do not support PRACH adaptation should be barred in the NES cell, we think the legacy UE should be allowed to access to such NES cells as long as these UEs can use the legacy RACH resource. Therefore, we don’t need any new barring mechanisms to bar the legacy UEs to access the NES cells supporting the PRACH adaptation.

Proposal 4: No need to bar the legacy UEs from accessing the NES cells that support PRACH adaptation.

For the NES-capable UEs, RAN2 need to specify the UE behaviour on how to utilise the legacy PRACH resources and additional PRACH resources. The behaviours to be specified included e.g. selection between legacy and additional PRACH for initial RACH request, selection between legacy and additional PRACH for RACH request retransmission, fallback criteria etc.

Proposal 5: UE behaviours on how to utilise the additional PRACH resources should be defined.

# Conclusion

We propose RAN2 to discuss following proposals:

Proposal 1: RAN2 to consider a joint design of the MAC CE for both on-demand SSB and SSB adaptation.

Proposal 2: The same L3 measurement framework can be applied for both on-demand SSB and SSB adaptation.

Proposal 3: The additional PRACH resources for RACH adaptation should be included in a SIB.

Proposal 4: No need to bar the legacy UEs from accessing the NES cells that support PRACH adaptation.

Proposal 5: UE behaviours on how to utilise the additional PRACH resources should be defined.