3GPP TSG RAN WG2 Meeting #129			R2-2500746

Athens, Greece, February 17th – 21st 2025

Agenda Item: 	8.7.3

Source: 		Sony

Title: 			Discussion on enabling TX/RX for XR during RRM measurements

Document for: 	Discussion & Decision

# Introduction

eXtended Reality (XR) for NR phase 3 work item (WI) was revised in RANP#105 [1]. The WI aims for enhancements addressing system capacity, efficient and effective mechanisms to meet QoS requirements and lower device power consumption, in the context of the demanding scenarios and traffic characteristics requirements of XR, including also multi-modal applications. The WI also aims to realize system capacity gains by enabling transmission/reception in gaps/restrictions that are caused by RRM measurements, while keeping impact to mobility performance limited.

The Rel-19 XR ph3 objective that being addressed in this contribution is:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

- Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

At RAN2#127bis [3], the following agreements were made related to RRM measurement gap skipping.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

At RAN2#128 [4], the following agreements were made:

Agreements on RRM measurement gaps impacts - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

This contribution provides our views on enabling transmission/reception in measurement gaps / scheduling restrictions period to support XR application.

# Discussion

Dynamic indication

To support XR application, the UE may require continuing the transmission and skip the RRM measurement. Several proposals / solutions have been proposed in the last few meetings. In RAN1#118, it was agreed as a working assumption on the solution based on the dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. The dynamic indication is provided from gNB to UE via downlink control information (DCI) carried out by PDCCH. This approach is illustrated in Figure 2.

Figure 1: Illustration of Skip / Release Measurement to support data Tx/Rx

The skip measurement here means the UE can continue CG transmission during the affected measurement gap. The other solution is to provide the indication semi-statically. We consider this solution may not be able to support XR characteristics. The gNB may not be known the UE traffic far in advance. Hence, the working assumption to support the dynamic indication made in RAN1#118 [4] as shown below should be confirmed.

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI.

Observation 1: RAN1 has a working assumption to support dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

The dynamic indication by DCI to skip a particular gap/restriction is carried out within DCI formats 0_1/1_1, DCI formats 0_2/1_2, and DCI formats 0_3/1_3. As agreed earlier, one-bit bit-field size in each DCI format is required to support this operation (i.e. enable Tx/Rx during gap/restriction) . We consider the gNB may have a flexibility to enable whether a specific DCI format may carry such information. In this case, it requires a new high layer parameter (e.g., RRC signalling) to enable that one-bit information in each DCI format.

Proposal 1: Support higher layer signalling (i.e., RRC signalling) to enable one-bit information (to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements) in each DCI format.

Partial skipping:

The XR traffic that may collide with RRM measurement may not necessarily be a video transmission which may require a large payload. It can also be the motion control information which require a small payload size. For example, it can be transmitted within one slot. If the UE perform transmission / reception for such short period then in practice, the UE can still continue to perform RRM measurement for the remaining measurement gap. The UE is not necessarily needed to skip the entire measurement gap. The UE can still perform both XR transmission / reception and RRM measurement with reduced measurement gap length. The RRM measurement with reduced measurement gap length is basically the operation of partial skipping of gap(s) / restriction(s).

RAN1 has expressed their view that from RAN1 perspective this feature is not supported in Rel-19 [2]. We think the primary reason is non-technical (e.g., limited time, etc). In our view, RAN2 at least need to discuss as this operation could still beneficial, particularly if the Tx/Rx for data transmission is relatively small. Hence, the UE can still perform RRM measurement.

Proposal 2: RAN2 to discuss supporting partial skipping of gap(s)/restriction(s) to allow RRM measurement (i.e., for the non-colliding data-channel and SSB).

UE assistance:

For the case of uplink transmission, UE may have better information than gNB in terms of the uplink transmission characteristics. The UE has direct interaction with the upper layer (e.g., application layer) so that the UE can predict the upcoming uplink transmission characteristics. The uplink transmission can be in a form of an aperiodic transmission that require spontaneous transmission from the UE. It can also be in a form of semi-static / periodic transmission. Furthermore, the UE may have various types of uplink transmission. Some uplink transmission can be compressed, such that the legacy gap/restriction can still be applied. In this case, the UE can prioritize the measurement over the transmission. Some other uplink transmission (e.g., XR traffic for video, control, data) has higher priority over RRM measurement. In this case, the UE skips RRM measurement and performs UL transmission instead. Based on these reasonings, UE assistance information to serving gNB on indicating an upcoming temporary measurement gap modification (e.g., skipping) shall be supported. The gNB can take that information into account and decide whether to grant the temporary measurement gap modification or not. This mechanism would also ensure that the gNB is still in full control of the UE behaviour. Hence, any potential system performance degradation (e.g., due to the measurement gap skipping) can be avoided.

In RAN1#117bis [5], RAN1 could not reach consensus to support UE assistance information (UAI) related to measurements occasions. The reason is due to lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). Hence, RAN1 sends an LS to RAN4 requesting their views on UAI [6]. Here, we express our observation related to UAI.

Though the use and design of UE assistance information is within RAN2 domain, and there for these aspects should be discussed by RAN2.

Observation 2: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 3: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Proposal 3: Support UE Assistance Information (UAI) indicating the gNB the MG(s) to be skipped by the UE.

At RAN2#127bis, it was agreed to that “RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution”. This could be a solution in addition to the DCI-based solution. The DCI-based solution is currently a working assumption in RAN1 and RAN4 has investigated this solution and confirmed the feasibility of this solution.

Proposal 4: RAN2 to investigate the feasibility of semi-static solution in addition to the DCI-based solution.

RRM measurements quality:

In case of the RRM measurement is affected by the temporary measurement gap modification, for example when one or more of the measurement gaps has been skipped, then the UE can also indicate that the reported RRM measurement has been affected. Skipping one or more measurement gaps is expected to reduce the quality of the reported RRM measurement from the UE, such as reducing the mobility performance. This may require further study in other WG(s), such as RAN2 and RAN4.

Observation 4: Skipping RRM measurement may affect the quality of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Proposal 5: RAN2 to align with RAN4 if any RRC signalling is needed related to UAI, Partial skipping and quality aspects.

# Conclusion

In this contribution, we provide our views on enabling transmission/reception during measurement gap. The observations and proposals are listed below.

Observation 1: RAN1 has a working assumption to support dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements.

Proposal 1: Support higher layer signalling (i.e., RRC signalling) to enable one-bit information (to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements) in each DCI format.

Proposal 2: RAN2 to discuss supporting partial skipping of gap(s)/restriction(s) to allow RRM measurement (i.e., for the non-colliding data-channel and SSB).

Observation 2: UE has better knowledge, particularly for the UL traffic. Hence, UE assistance information could be beneficial in assisting gNB to allow XR traffic when there is a collision between XR traffic and RRM measurement.

Observation 3: UE assistance information indicating the number of gap(s) / restriction(s) that can be skipped during a configured RRM measurement is beneficial in the operation of XR transmission during RRM measurement.

Proposal 3: Support UE Assistance Information (UAI) indicating the gNB the MG(s) to be skipped by the UE.

Proposal 4: RAN2 to investigate the feasibility of semi-static solution in addition to the DCI-based solution.

Observation 4: Skipping RRM measurement may affect the quality of the reported RRM measurement. It would be beneficial for gNB to know whether the RRM measurement has been compromised or not.

Proposal 5: RAN2 to align with RAN4 if any RRC signalling is needed related to UAI, Partial skipping and quality aspects.

# References

- RP-241771, “Revised WID: XR (eXtended Reality) for NR Phase 3”, RANP #105, September 2024

- RAN1 #118bis Chairman’s notes, Hefei, China, October 2024

- RAN2#127bis, Hefei

- RAN2#128, Orlando

- RAN1#117bis

- R1-2405736 “LS on UE assistance information related to measurement occasions”, Nokia