3GPP TSG-RAN WG2 Meeting # 129 	                R2-2500747

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item: 	8.7.5

Source:		Sony

Title:	RLC AM enhancements for timely retransmissions

Document for:	Discussion & Decision

# Introduction

RAN2_125bis agreements for RLC enhancements are:

We focus on RLC AM

RAN2 will analyse solutions to ensure timely RLC retransmission(s) for XR.

RAN2 will analyse how to avoid unnecessary retransmissions (e.g. to avoid reTx of out-dated packets)

RAN2_126 agreements for timely retransmissions of RLC AM enhancements are:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

    - Autonomous retransmission (i.e., without status report) of PDUs based on some triggers (existing or new triggers can be considered)

    - Retransmission based on enhanced status report.

    - Retransmission based on enhanced polling.

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism.

Impact on capacity should be considered.

RAN2 focuses on the enhancements for UL traffic.

RAN2_126 agreements for avoiding unnecessary retransmissions RLC AM enhancements are:

For avoiding unnecessary RLC AM retransmissions, RAN2 to enhance the RLC AM by adopting enhancements from one of the following perspectives:

- Rx initiated approach.

- Tx initiated approach.

RAN2 will discuss details of both approaches, compare them and choose one once the details are clearer.

For Tx initiated approach:

    - The transmitting side of AM RLC entity notifies the receiving RLC side about the obsolete SDUs.

    - Tx side stops retransmit obsolete SDUs.

    - Rx side updates state variables according to the information from Tx side

For Rx initiated approach:

    - For proper advancing of the transmitting window, RLC AM is enhanced with a way for the receiver to indicate abandoned SDUs to the transmitter.

    - Tx side just processes the status report as in legacy.

    - FFS how Rx side determines that an SDU should be abandoned.

RAN2_127 agreements for avoiding unnecessary retransmissions RLC AM enhancements are:

Any solution should ensure that windows at Tx side and Rx side are not out of sync. As a baseline, we assume Rx window advances before Tx window advances. FFS if for Tx approach window sync needs to be achieved in another way, e.g. advancing Tx window first.

In the RX-initiated approach for avoiding unnecessary retransmissions, RLC receiver abandons missing SDUs like already done by PDCP, i.e. based on a timer.

In addition to Tx and Rx approaches, RAN2 will consider a combined Rx and Tx approach, where

    - Tx side stops to retransmit an obsolete SDUs based on the discard indication/a number of retransmissions as for Tx initiated approach.

    - Rx side stops to receive an obsolete SDU based on local timer as for Rx initiated approach.

RAN2_127bis agreements for timely retransmissions of RLC AM enhancements are:

- RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19.

- Exclude enhanced status reporting.

- Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay.

RAN2_127bis agreements for avoiding unnecessary retransmissions RLC AM enhancements are:

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does.

- RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e.

  - TX side stops transmissions of an outdated SDU

  - RX side abandons the SDU based on a local timer

  - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified.

  - FFS if some C-PDU handling is needed to avoid C-PDU discard.

  - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed).

RAN2_128 agreements on unnecessary RLC retransmissions avoidance:

- There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer.

- Unless critical issue is identified, no Tx to Rx indication will be introduced.

- Special handling to avoid PDCP control PDU discard is not needed.

- A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.

- The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

RAN2_128 agreements for timely retransmissions of RLC AM enhancements

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement, and NW can configure either or both of them.

In this contribution, we provide our views on RLC AM enhancements more specifically how to to achieve timely retransmissions with polling enhancement.

# Timely retransmissions based on polling enhancement:

- In RAN2#128 meeting, for achieving timely retransmissions on RLC layer for XR traffic (i.e., the delay critical RLC SDU), RAN2 finally agreed that timely RLC retransmission solution for Rel-19 covers both autonomous retransmission and polling enhancement, and NW can configure either or both of them.

Obviously autonomous ARQ retransmission, where the Tx side determines that the remaining lifetime of an SDU is very short and start retransmitting without waiting feedback from Rx side, is not efficient and impacts the system capacity as retransmissions are done blindly.

On the other hand, polling enhancements where Tx side relies on feedback from Rx side before starting ARQ retransmissions is more efficient if RX side can trigger a status report (SR) more quickly.

Discussion and proposals for enhancements:

In general, for RLC polling enhancements, the delay gap between the initial transmission and the re-transmission of the RLC SDU should be reduced. This means that both the frequency of RLC polling from Tx side and feedback from Rx side must be fast enough, so that Tx side can make selective ARQ retransmissions.

In the current NR Rel-18 specification, the frequency of RLC polling from Tx side depends on the configured parameters: pollPDU with minimum value of 4PDUs, pollByte with minimum value of 1kBytes, and t-PollRetransmit with minimum value of 5ms. In our view, these values are already small values, and there is no need to enhance them.

However, as discussed in the last meeting, one way to increase the polling frequency from Tx side for delay critical SDUs is to specify an additional triggering condition: If the remaining time drops below a configured threshold, the poll is triggered.

Proposal 1: RAN2 should specify an additional triggering condition for polling enhancement: If the remaining time for SDU drops below a configured threshold, polling information is triggered.

Furthermore, at the Rx side, when polling information is received, the speed of the status report (SR) depends on number of parameters: t-Reassembly, t-StatusProhibit timers. While both timers can be set to zero, in our view, it is not desirable to set t-Reassembly to zero because it will cause HARQ retransmissions from the lower layers to be abandoned early, hence resulting degraded radio link performance, and premature NACKs to be reported in the status report.

On one hand, the t-Reassembly timer must be set to an appropriate value by considering the time it takes to complete a number of HARQ retransmissions in the lower layers,. On the other hand, the t-Reassembly timer creates long gap between the first ARQ transmission and retransmissions, more specifically when the remaining lifetime of the SDU is very short.

One solution is to keep the t-Reassembly timer as in the current specification, but when generating the status report, the t-Reassembly timer is bypassed so that the receiver (UE) can immediately report the status of SDU or segments of SDU (i.e., one or more PDUs) that are still stuck in the lower layer HARQ retransmissions whilst t-Reassembly timer is still running (i.e.., the HARQ processing can still be done).

The point is to speed up the status report (SR), in a sense that at the point of reporting, if some SDUs are already confirmed to be ACK or NACK, they are reported as in legacy specification, however, if some SDUs or segments of SDU are still stuck in the HARQ retransmissions (e.g., t-Reassembly timer is still running), they are reported as UNKNOWN status in the report. In this case, after receiving the status report, the Tx side can selectively retransmit (ARQ level) for these SDUs or segments of SDU with UNKNOWN status, hence reducing the gap between the first ARQ transmission and retransmissions.

Note that the t-Reassembly timer is still needed to oversee the HARQ retransmissions for the missing SDU, as it ensures that a sufficient time is given to the HARQ retransmissions to improve the radio link performance, so the result will be either late ACK or firm NACK which can be reported in the next status report.

An example is given on Figure 1 where there are three SDUs in the status report: SDU-5, SDU-6 and SDU-7.

In this example, at the receiver, at the time of reporting, the last SDU-7 is successfully received, SDU-5 has one segment/PDU that has failed (i.e. firm NACK), and SDU-6 has two segments/PDUs that are still under HARQ retransmissions at the time of reporting. This means that after SDU-7 is received successfully, the t-Reassembly timer is started because the middle SDU-6 is missing.

As proposed above, if receiver receives a polling information, the receiver should bypass the t-Reassembly timer, and provide status report including the status of SDU(s) that is stuck in the HARQ retransmissions at the time of reporting. In this case, SDU-6 is reported as UNKNOWN status by indicating/setting the corresponding reserved bit as 1 (R = 1).

In detail as shown on Figure 1, the status report contains D/C = 0, CPT=0, ACK_SN = 8, blue E1 = 1 and R = 0. The following is NACK_SN = 5 to indicate the first lost PDU. In this case, E1 = 1 (to indicate there is another lost PDU following), E2 = 1, and E3 = 0 and R = 0. As E2 was set to 1, the lost PDUs for SDU-5 must be identified as SOstart = 0x0100 and SOend = 0x01FF (the last byte of the lost PDU). The following is UNKNOWN_SN = 6 to indicate that the some PDUs are still under HARQ retransmissions. In this case, E1 = 0, E2 = 1, and E3 = 0 and R = 1. As E2 was set to 1, PDUs that are under HARQ retransmissions for SDU-6 must be identified as SOstart = 0x0000 and SOend = 0x01FF. Note that in this example, during reporting, the t-Reassembly timer has been started due to SDU-7 is received successfully but segments of the middle SDU-6 are missing.

Figure 1. Example of STATUS REPORT (SR) with SDUs where some SDU segments (i.e., one or more PDUs) are still stuck in the HARQ retransmissions and given UNKNOWN status in the report.

Proposal 2: When generating the status report (SR), the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running (i.e., still stuck in the lower layer retransmissions).

Proposal 3: For any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/setting the corresponding reserved bit as 1 (R = 1).

# Conclusion

In this contribution, we have provided our views on RLC AM enhancements more specifically how to to achieve timely retransmissions with polling enhancement and we have the following proposals for RAN2 to agree:

Proposal 1: RAN2 should specify an additional triggering condition for polling enhancement: If the remaining time for SDU drops below a configured threshold, polling information is triggered.

Proposal 2: When generating the status report (SR), the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running (i.e., still stuck in the lower layer retransmissions).

Proposal 3: For any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/setting the corresponding reserved bit as 1 (R = 1).

# References

- RP-243318, “Revised WID on XR (eXtended Reality) for NR Phase 3,” RAN#106.