3GPP TSG-RAN WG2 Meeting #129		R2-2500748

Athens, Greece, 17th – 21st Feb, 2025

Agenda Item: 	8.7.6

Source:		Sony

Title:	Recommended bit rate based XR rate control

Document for:	Discussion & Decision

# Introduction

RAN2_128 agreed for uplink congestion signaling:

- RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful

In this contribution we provide our views on QoS flow vs DRB and wait for SA2,  bit rate values indication based on SA4 reply, and whether to support rate query MAC CE.

# Discussions

RAN aware XR rate control can help the XR application to adjust its codec bit rate according to radio conditions and ensure user experience. RAN2 agreed in the past that it is feasible for RAN to estimate the congestion information per QoS flow and per DRB in both UL and DL direction. For UL, RAN can do it without any UE impacts. If a RAN node is congested, then it can signal the congestion indication to the XR application which in turn will adjust the XR data rate for DL direction. At the same time, RAN should be able to reduce the data rate for XR application on the uplink direction. For rate control, we think the use of Recommended bit rate MAC-CE can be enhanced and used in this case.

For query MAC-CE message, we think it is straightforward to support what is already in the specifications rather than excluding one particular message for one type of traffic.

Proposal 1: Both UL/DL bit rate recommendation and UL/DL bit rate recommendation query messages are supported for XR UL rate control.

On the question of per QoS flow/per DRB, current MAC-CE is per LCH ID. Some companies argued that per DRB indication wont allow QoS enforcement and it is not at the same level as L4S, which is per QoS flow. We think that gNB will receive the indication that rate control needs to be performed for certain QoS flow(s). gNB has the option of indicating both per QoS flow or per DRB to the UE and both options are feasible, as agreed during RAN2-128 meeting. If the indication is per QoS flow and more than one service’s QoS flows are mapped to a single DRB then the UE application layer may need to lower the data rate provided this indication of per QoS flow is passed to the UE application layer. However, if such flexibility does not exist in the UE implementation between modem and application chipset then this indication of per QoS flow is not useful. In the absence of this information, rate control is applied over radio and application layer will eventually learn that the rate control has been applied and may adjust its codec rate. SA2 is still discussing the RAN2 LS and RAN2 should wait for SA2 response before any discussion on QoS flow vs DRB.

Proposal 2: Wait for SA2 response before any further discussions on QoS flow vs DRB.

SA4 reply [3]

The SA4 reply is as follows: The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

Based on SA4 reply and finer granularity of 10%, 64 code points should be sufficient. Current MAC-CE also has 6 bits for UL/DL data rate and 64 values permitted. We think no change is required to the format of MAC-CE but a new table with new values can be introduced.

Proposal 3: Use existing MAC-CE format and introduce a new table with existing code points.

# Summary

In this contribution, we have the following proposals for RAN2 to agree:

Proposal 1: Both UL/DL bit rate recommendation and UL/DL bit rate recommendation query messages are supported for XR UL rate control.

Proposal 2: Wait for SA2 response before any further discussions on QoS flow vs DRB.

Proposal 3: Use existing MAC-CE format and introduce a new table with existing code points.

# References

- RP-241771, “Revised WID on XR (eXtended Reality) for NR Phase 3,” RAN#105.

- TR 23.700-60, SA2, Study on XR (Extended Reality) and media services

- R2-2500056: Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications

Annex: code points for 100k to 40mbps with 10% granularity

| 1 | 100 |
| --- | --- |
| 2 | 110 |
| 3 | 121 |
| 4 | 133.1 |
| 5 | 146.41 |
| 6 | 161.051 |
| 7 | 177.1561 |
| 8 | 194.8717 |
| 9 | 214.3589 |
| 10 | 235.7948 |
| 11 | 259.3742 |
| 12 | 285.3117 |
| 13 | 313.8428 |
| 14 | 345.2271 |
| 15 | 379.7498 |
| 16 | 417.7248 |
| 17 | 459.4973 |
| 18 | 505.447 |
| 19 | 555.9917 |
| 20 | 611.5909 |
| 21 | 672.75 |
| 22 | 740.025 |
| 23 | 814.0275 |
| 24 | 895.4302 |
| 25 | 984.9733 |
| 26 | 1083.471 |
| 27 | 1191.818 |
| 28 | 1310.999 |
| 29 | 1442.099 |
| 30 | 1586.309 |
| 31 | 1744.94 |
| 32 | 1919.434 |
| 33 | 2111.378 |
| 34 | 2322.515 |
| 35 | 2554.767 |
| 36 | 2810.244 |
| 37 | 3091.268 |
| 38 | 3400.395 |
| 39 | 3740.434 |
| 40 | 4114.478 |
| 41 | 4525.926 |
| 42 | 4978.518 |
| 43 | 5476.37 |
| 44 | 6024.007 |
| 45 | 6626.408 |
| 46 | 7289.048 |
| 47 | 8017.953 |
| 48 | 8819.749 |
| 49 | 9701.723 |
| 50 | 10671.9 |
| 51 | 11739.09 |
| 52 | 12912.99 |
| 53 | 14204.29 |
| 54 | 15624.72 |
| 55 | 17187.19 |
| 56 | 18905.91 |
| 57 | 20796.51 |
| 58 | 22876.16 |
| 59 | 25163.77 |
| 60 | 27680.15 |
| 61 | 30448.16 |
| 62 | 33492.98 |
| 63 | 36842.28 |
| 64 | 40526.51 |