3GPP TSG-RAN WG2 Meeting #129  	                            R2-2500753

Athens, Greece, Feb. 17th – 21st, 2025							          (Resubmission of R2-2410392)

Agenda Item:	8.13.2

Source: 	Sony

Title:	Multi-hop relay selection/re-selection

Document for:	Discussion

# Introduction

In RAN2#128 meeting, multi-hop relay discovery/selection/re-selection was discussed with agreements as following:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

In this contribution, we discuss the issues of sidelink relay selection/re-selection in multi-hop relay.

# Discussion

In Rel-17 sidelink L2 U2N relay, if there are multiple suitable U2N Relay UEs, it is up to U2N Remote UE implementation to choose one of the U2N Relay UEs among them. With the introduction of multi-hop L2 U2N relay, both L2 single hop U2N relay and L2 multi-hop U2N relays could be deployed in the network for remote UE to choose from. On the other hand, from reliability and latency point of view, the reliability will reduce as the number of hops will increase and the latency will increase as the number of hops increases. If it is fully up to remote UE implementation to select/re-select the relay among the relay candidates and if remote UE is not aware of the number of hops to the network, the remote UE will tend to only consider the PC5 radio link quality (and/or any existing criteria) on the first hop e.g., the radio link quality between remote UE and first relay UE candidate. But the QoS performance e.g., reliability, latency will fluctuate among the choice of different relay paths with different number of hops on the path, the QoS requirement may not be met for certain relay paths if only the first hop PC5 radio link quality is considered.

Observation: The QoS requirement may not be guaranteed for certain relay paths with different number of hops on the path, if only existing one hop L2 U2N relay criteria e.g., PC5 radio link quality of the first hop is considered for multi-hop relay selection/re-selection.

So, the criteria on how to select the relay among different types e.g., number of hops of relay candidates should be specified, in addition to existing one hop L2 U2N relay criteria, e,g., the radio link quality of the first hop, if the above problem is recognised.

Proposal 1: RAN2 to discuss the criteria to select/re-select L2 U2N relay, in addition to the existing one hop L2 U2N relay criteria, with the introduction of multi-hop relay.

In Rel-17 U2N relay, the remote UE triggers U2N relay selection when the direct Uu signal strength of current serving cell of the remote UE is below a configured signal strength threshold. Similarly, the same principle should be applied when remote UE performs the selection of U2N multi-hop relay. Network may configure a Uu link quality threshold e.g., RSRP threshold, indicating that only when the Uu link quality is below this threshold, the remote UE will be allowed to select multi-hop L2 U2N relay. This threshold can be included in the broadcast signalling or pre-configured.

It is FFS whether the same Uu RSRP threshold as single hop U2N relay will be configured for multi-hop U2N relay or not. It is preferrable that a separate Uu RSRP threshold is configured for multi-hop U2N relay if network would like to restrict the condition for remote UE to perform multi-hop U2N relay selection.

Proposal 2: Network configures a Uu signal strength threshold for remote UE to perform the multi-hop U2N relay selection.

# Conclusion

We have observation as follows.

Observation: The QoS requirement may not be guaranteed for certain relay paths with different number of hops on the path, if only existing one hop L2 U2N relay criteria e.g., PC5 radio link quality of the first hop is considered for multi-hop relay selection/re-selection.

We propose RAN2 to discuss following proposals:

Proposal 1: RAN2 to discuss the criteria to select/re-select L2 U2N relay, in addition to the existing one hop L2 U2N relay criteria, with the introduction of multi-hop relay.

Proposal 2: Network configures a Uu signal strength threshold for remote UE to perform the multi-hop U2N relay selection.