3GPP TSG-RAN-WG2 Meeting #129	                                        R2-2500755

Athens, Greece, Feb 17th - 21st, 2025

Agenda Item:	8.1.2.2

Source: 	Sharp

Title:  	On LCM for UE-sided Model for Beam Management Use Case

Document for: 	Discussion

# Introduction

In RAN2#127, following agreements were made [1]:

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can be considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In RAN2#127bis, following agreements were made [2]:

Agreements for BM - UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality. We should aim to align the design on how the applicable functionality are signaled. FFS on the applicability reporting content. - FFS if inference configuration can be signaled in step3. - UE can report to the network when an applicable AI functionality becomes non-applicable. FFS how this is signaled (e.g. explicitly/implicitly). Consider different scenarios, whether it is regarding an active functionality) - Data collection initiation and configuration for data collection is under network control. FFS how the NW determines whether data collection should be initiated (e.g. via UE requests (UE directly or UE server) - For the purpose of discussion of AI/ML BM LCM operations, existing procedures and terminologies from the CSI Framework should be used, including those defined for aperiodic, semipersistent on PUCCH, semipersistent on PUSCH, and periodic reporting configurations (as/if defined in RAN1 pending response LS from RAN1). - For now, RAN2 will not define terminology specific to the activation or deactivation for AI/ML models. Can come back to this discussion later.

In RAN2#128, following agreements were made [3]:

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In this document, we discuss signaling methods to configure the UE with conditions for determining and reporting applicable functionalities. We also discuss the scenario where network or the UE may indicate their preferred or default configurations associated with a functionality. Additionally, the document discusses issues associated with the reporting of target gNB applicable functionality after the handover.

# Discussion on Applicable Functionality Reporting

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “functionality” of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

RAN2 further has made following agreements and signalling procedure (as shown in the figure below) on applicable functionality reporting for beam management UE-sided model:

Figure 1: Signaling flow for applicable functionality reporting

We propose to introduce the following aspects for the agreed signalling flow:

Step 3 as shown in Fig. 1, only outlines the configuration elements i.e. applicability report is configured, without addressing how the UE determines applicability of a functionality, and when or under what conditions the functionalities should be reported by the UE to the network. These aspects—timing, triggering conditions, and the mechanism for functionality reporting—are critical for ensuring that the network can receive and act on reports effectively, especially in scenarios where AI/ML functionalities may dynamically change.

Observation 1: Step 3 only outlines the configuration elements, without addressing how the applicability of a functionality is decided or under what conditions the applicable functionalities should be reported by the UE.

In legacy, non-AI/ML-based networks, feature’s configuration and activation are typically performed simultaneously. If the UE is unable to comply with the provided configuration, it encounters radio link issues that may affect its connectivity or performance.

However, AI/ML-based functionalities introduce a new dynamic: the network may need to frequently update or provide new configurations (e.g. in CSI-ReportConfig) as applicable functionalities change. These changes can be triggered by modifications in network-side additional conditions or associated IDs, UE-side additional conditions, or when a specific AI/ML model becomes (un)available or due to the change in behavior and performance of functionality(s) itself.

In [4], RAN 1 made the following agreements on Step 3, 4 and 5 of Fig. 1 related to the Questions from RAN 2 LS:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

The RAN 1 agreement [4] related to step 3 fig. 1 indicates that functionality applicability report is based on A) and/or B) options.

The applicability report (Functionality applicability) is based on two options (A and B).

- Option A implies: The CSI-ReportConfig must be configured to support inference configuration.

- Option B implies: The inference-related parameters must be provided and appropriately selected.

Based on the two options A and B as configured in step 3., the UE may report inference reports to the network. Thus, it needs to be determined whether the inference report is integrated within the legacy CSI-Report or if it is managed through a distinct report or a message.

Proposal 1: RAN2 to clarify if the inference is reported as in the legacy CSI report.

In step 3, as per RAN1 agreement [4], the network provides configuration for applicability reporting to the UE. Given the dynamic nature of the applicable functionalities and UE environment, the network should proactively/pre-configure configure the UE to report any changes in the applicability of the functionalities. Therefore, the network should configure the UE with conditions for determining and reporting applicability and non-applicability of functionality(s) respectively.

For instance, the network may configure specific conditions or triggers for the User Equipment (UE) to help it assess the relevance or applicability of a given functionality. These triggers may include:

- Event-based triggers: These may involve thresholds related to model or functionality performance (e.g., KPIs such as prediction accuracy), channel conditions (e.g., Doppler shift), or resource constraints.

- Periodic triggers: These could involve timers that provide recurring updates on the applicability of the functionality.

Proposal 2: RAN2 to discuss signaling procedure to allow the network to configure the conditions which the UE may use to determine and report (non) applicable functionalities (e.g., applicable CSI-ReportConfig), for example, in step 3, Fig. 1.

Figure 2: Signaling flow for applicable functionality reporting

Figure 2 illustrates an example of signaling that may be used to configure the conditions under which the UE determines and reports the applicability of a functionality. This signaling process allows the network to set specific parameters or triggers, helping the UE determine whether a functionality is applicable. It also enables the UE to report the applicability of relevant functionalities based on dynamic changes or predefined criteria. The configuration may include various conditions such as network-side or UE-side parameters, model availability, or other context-specific factors that impact the UE's process to determine and report applicable functionalities.

Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3a, 3b” (can also be combined): Following configurations are provided from NW to UE:

  - The necessary configurations for the UE to report applicable functionalities via UAI OtherConfig reporting.

  - Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional.

  - Events/Conditions (e.g., model availability, thresholds for signal quality or latency, Functionality related KPI thresholds) that trigger a given functionality to be deemed applicable/inapplicable.

- Step 3c:  The UE evaluates the applicability of the AI/ML functionality based on several factors, which may include the associated ID and inference configurations provided by the network, as well as UE-specific internal parameters and the availability of the relevant model. This process ensures that the AI/ML functionality is applied appropriately, considering both network-provided inputs and the UE's internal conditions, to determine whether the functionality is ready to be activated or requires adaptation. In case any further adaptation to the functionality is needed, the network may provide an updated configuration in step 5, Fig. 2 respectively.

- Step 4: UE reports applicable functionality (in applicability report) in the following scenarios:

    - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI, for e.g. based on the events or conditions as configured in step 3 or step 4.

    - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

In RAN2, there is an FFS and ongoing discussions regarding whether the UE can consider functionality configurations, such as inference configuration, in its decision-making process to determine applicable functionality(s). Additionally, it remains an open issue how the UE determines applicable functionality in cases where the network-side additional conditions or associated IDs are not provided in Step 3.

Proposal 3: The network may provide specific conditions or events to the UE, for example, in CSI-ReportConfig. in step 3, Fig. 1, to help determine the applicability of a functionality.

The network configured conditions would enable the UE to notify the network when functionality changes occur, allowing network to provide timely updates or new inference configurations.

The network-configured conditions help the UE determine the applicability of a functionality. This occurs when NW-side additional conditions or associated IDs are provided to the UE, or when they are not provided in Step 3. By addressing these changes proactively, the network can reduce the risk of failures and enhance system performance in dynamic environments.

For example, for the Beam Management use-case, CSI configuration could determine when a specific functionality/inference configuration should be applied. When a significant variation in channel quality (e.g., CQI threshold or any pre-defined/configured KPI metric threshold violation) is detected, the network could (de) activate or update a specific functionality/inference configuration that adapts to these new conditions.

Observation 2: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

Based on observation 2, there is a possibility for a scenario where functionalities are frequently activated and deactivated, leading to a ping-pong effect. This occurs for example, when functionalities transition between applicable and inapplicable states due to varying conditions. Such frequent state changes can introduce inefficiencies and degrade the performance of the UE.

This may further result in increased signaling between the UE and the network, further degrading system performance. This problem may further be amplified if the UE reports to the network whenever an applicable AI functionality becomes non-applicable (as per the RAN2#127bis agreement [2]) or if Multiple functionalities may be (de) activated simultaneously.

Proposal 4: RAN2 to discuss mechanisms to prevent the frequent activation and deactivation of applicable functionalities (e.g., when functionalities transition between applicable and inapplicable states) by considering UE-side internal conditions, environmental factors, and related factors.

# Discussion on Functionality Activation

The final applicability of a functionality may be determined either by the UE or the network, depending on whether the UE has sufficient information to evaluate the functionality's applicability. If the UE has adequate information, such as inference configurations, associated ID(s) or additional conditions, it may autonomously determine the applicability. Otherwise, the network may take responsibility for making this determination.

While the framework for (de) activating applicable functionalities has been outlined, key aspects concerning the initial activation states and their reliance on the inference configurations provided in Steps 3 and 5 in Fig. 1, still necessitate further study and discussion within RAN2. Additionally, the potential for multiple inference configurations to be assigned to a single functionality, as well as the possibility of simultaneously activating multiple functionalities, are critical factors that must be incorporated to enhance flexibility and ensure optimal performance. In a beam management use case, the network can leverage AI/ML inference configurations to optimize beam prediction, selection and signal quality. These inference configurations or parameters can be communicated to the UE through AI/ML CSI-ReportConfigs.

As per the RAN 1 agreements [4], for beam management, multiple CSI-ReportConfigs for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. This implies that each UE may be assigned multiple CSI-ReportConfig instances, tailored for different reporting types, such as periodic or aperiodic reporting.

In Step 4, the UE reports the applicability of one or more CSI-ReportConfigs and/or sets of inference-related parameters. It is still to be determined whether any additional information, apart from the applicability, is needed. One or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) is configured in Step 3, the applicable aperiodic and semi-persistent CSI reports can be activated or triggered by the network after receiving the applicability report from the UE. For applicable periodic CSI reports, they are considered activated only if the applicability of the corresponding CSI-ReportConfig is reported in the RRCReconfigurationComplete message.

In Step 5, the network can optionally configure the CSI-ReportConfig for inference configuration through the RRCReconfiguration message, where the associated ID may be configured in the CSI framework as a working assumption. Note that Step 5 may be optional if the UE has already been configured with the CSI-ReportConfig in Step 3.

This allows flexibility in measurement and reporting of applicable functionality based on specific conditions or requirements. However, with multiple configurations present, both the network and UE may need to decide on a primary report or configuration to use at any given time.

To facilitate this selection process, the network can designate a default CSI-ReportConfig during configuration. This default setting helps ensure that the UE has a clear starting point for reporting in scenarios where multiple options exist, simplifying the decision-making process.

Proposal 5: The network may provide a default or preferred configuration in functionality configuration for e.g., a default or preferred CSI-ReportConfig either in step 3 or 5 respectively.

As per RAN 1 agreements [4], In step 4, the UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters. It is FFS on whether/what other information along with the applicability is needed. Thus, in step 4 the UE may also indicate to the network its preferred configuration. The network may take into account UE’s preference when deciding which configurations/functionality to activate or deactivate.

Proposal 6: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig) in step 4.

# Discussion on Neighbour cell Applicable Functionality reporting

RAN2 #128 has the following agreements on Neighbour cell applicable functionality reporting:

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

Based on the RAN2 #128 agreements, the target gNB transmits network-side additional conditions and/or inference configurations within the Handover Command. The UE then responds with an applicability report (in either RRCReconfigurationComplete or UAI) after handover completion, thus determining the target gNB's applicable functionalities. This raises two critical scenarios requiring RAN2 discussion:

Scenario 1: Functionality Mismatch Between Source and Target gNBs

A UE may have an active, applicable functionality in the source gNB that is not supported by the target gNB.

Proposal 7: RAN2 to discuss and clarify UE’s action when a functionality is applicable in source gNB but not applicable in the target gNB.

Scenario 2: Functionality Availability in a Neighbouring Cell different than the target gNB

A functionality currently active in the source gNB may not be applicable in the target gNB after handover. However, it is possible than the functionality maybe applicable in a different neighbouring gNB (s) other than the target gNB. This neighbouring gNB(s) (other than the target gNBs) may not have the best RSRP but they may have acceptable RSRP levels. This type of scenarios may present additional complexity and challenges.

Proposal 8: RAN2 to discuss and clarify UE’s action when a functionality is applicable in source gNB and a different neighbor gNB other than the target gNB.

# Conclusions

Based on the discussion above, we have the following observations and proposals:

Observation 1: Step 3 only outlines the configuration elements, without addressing how the applicability of a functionality is decided or under what conditions the applicable functionalities should be reported by the UE.

Observation 2: Applicable functionalities are dynamic in nature and may fluctuate due to variations in the UE's environment, network-side additional conditions, UE-side additional conditions, or the availability of the AI/ML model.

Proposal 1: RAN2 to clarify if the inference is reported as in the legacy CSI report.

Proposal 2: RAN2 to discuss signaling procedure to allow the network to configure the conditions which the UE may use to determine and report (non) applicable functionalities (e.g., applicable CSI-ReportConfig), for example, in step 3, Fig. 1.

Proposal 3: The network may provide specific conditions or events to the UE, for example, in CSI-ReportConfig. in step 3, Fig. 1, to help determine the applicability of a functionality.

Proposal 4: RAN2 to discuss mechanisms to prevent the frequent activation and deactivation of applicable functionalities (e.g., when functionalities transition between applicable and inapplicable states) by considering UE-side internal conditions, environmental factors, and related factors.

Proposal 5: The network may provide a default or preferred configuration in functionality configuration for e.g., a default or preferred CSI-ReportConfig either in step 3 or 5 respectively.

Proposal 6: The UE may indicate preferred configuration (s) to the network (e.g., preferred CSI-ReportConfig) in step 4.

Proposal 7: RAN2 to discuss and clarify UE’s action when a functionality is applicable in source gNB but not applicable in the target gNB.

Proposal 8: RAN2 to discuss and clarify UE’s action when a functionality is applicable in source gNB and a different neighbor gNB other than the target gNB.

# References

[1] Chair notes, RAN2 127 3GPP TSG-RAN WG2 Meeting #127	Maastricht, Netherlands, 2024

[2] Chair notes, RAN2 127bis 3GPP TSG-RAN WG2 Meeting #127bis Hefei, China, 2024

[3] Chair notes, RAN2 128 128 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, 2024

[4] R2-2406381 Report of [POST126][032] [AI/ML PHY] LCM (Intel/Samsung)Phase 2.

[5] R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model