3GPP TSG-RAN WG2 Meeting #129	R2-2500789
Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.5.3

Source:	InterDigital

Title:	On-demand SIB1 request and reception

Document for:	Decision

# Introduction

RAN approved a new work item on network energy saving (NES) [1], which was revised as [2]. The revised WID includes the following objective where study item objective was converted to the normative phase, as in the following:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

RAN2 made the following agreements in RAN2 #128 meeting [3]

Agreements on OD-SIB1

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

This contribution discusses various aspects to support on-demand SIB1 for UEs in idle/inactive mode.

# UL-WUS configuration for On-demand SIB1

Triggering for on-demand SIB1 is based on use of UL WUS signal of an existing signal/channel. One important step to enable on-demand SIB1 transmissions is the provisioning of UL WUS configuration for the UEs which are currently in idle/inactive mode.

When Cell A is providing the UL-WUS configuration for one or more NES cells, e.g., to support the scenario 1a, the Cell A may transmit UL-WUS configuration as part of the system information.

RAN2 #127Bis [4] agreed to provide UL-WUS configuration through a new SIB.

UL-WUS configuration will allow the UE to identify the NES cell, as agreed in RAN2#126 [6] through inclusion of PCI and frequency as part of the UL-WUS configuration. Based on the received UL-WUS configuration, the UE can transmit the UL WUS to the NES cell. Thus, at minimum, the UL-WUS configuration provides the RACH configuration for the NES cell. The RACH configuration should provide the suitable RO and preamble for requesting the OD-SIB1 based on the SSBs being transmitted by the NES cell, as agreed in RAN2#125Bis [7].

In order to transmit Msg1, the UE needs to know the mapping of SSB to RO. In legacy, SSB to RO mapping is part of the RACH configuration. Thus, following the legacy behavior, the RACH configuration of the NES cell, provided through UL-WUS configuration, should include the SSB to RO mapping to request on-demand SIB1. RAN1 #128 agreed to have legacy mapping rule for RA preamble index and SSB for on-demand SIB1 configuration.

RAN1 #128 agreed that the UE will receive RAR in response to the UL-WUS transmission in CORESET 0 of NES cell. As the network will transmit RAR through CORESET 0 in response to a UE’s UL-WUS transmission, the UE may acquire the CORESET 0 configuration from the SSB (e.g., same as legacy UEs get CORESET 0 configuration through SSB/MIB). Nevertheless, the UE still needs the quantity K_SSB to locate the CORESET 0 to decode RAR and the network may facilitate UE by providing CORESET 0 configuration for the NES cell as part of UL-WUS configuration.

Proposal 1: 	RAN2 to discuss whether the UE acquires fully or partially CORESET 0 configuration (i) through SSB of the NES cell, or (ii) as part of UL-WUS configuration for the NES cell.

One aspect related to the UL-WUS RACH configuration is whether the RACH configuration is as per legacy or if there is a separate RACH configuration to trigger SIB1. Although RACH configuration triggering SIB1 transmissions shared with legacy RACH functionality can work, a separate RACH configuration as trigger can be more advantageous, separate from legacy UEs, and can be used for all UEs in the cell. It is not likely to need too many preambles for requesting OD-SIB1, as in legacy request for other SI. Legacy UEs are also not likely to be camped on such SIB1-less cell.

Proposal 2:	UL-WUS configuration provides a dedicated RACH configuration for requesting OD-SIB1.

RAN2 agreed to provide UL WUS configuration through a new SIB. RAN2 #128 discussed the topic of providing UL WUS configuration as part of RRC release message, and following working assumption was made: UL-WUS configuration in RRC release is not supported.

In our understanding, if the network has the mechanism to provide UL WUS configuration through SIB, it does not need additional handling of providing the same configuration as part of RRC release message. This may require additional handling of timing and validity for the configurations provided through release message as well. Based upon this, we have the following proposal:

Proposal 3:	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

# RA procedure for WUS transmission

RAN2 has agreed that Msg3 is not used for OD-SIB1 request. It was further agreed that “RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline”. In a legacy RA procedure initiated for SI request, the procedure is considered successful upon reception of a RAR with the selected RAPID. A similar condition for success can be used for determining the success of an RA initiated for OD-SIB1 request.

Proposal 4:	Similar to legacy RA procedure initiated for SI request, an RA initiated for OD-SIB1 request is initiated with 4 step RA and is considered successful upon reception of RAR.

OD-SIB1 monitoring however is controlled by a time window configured part of the UL-WUS configuration, as agreed by R1. It’s FFS whether this time window starts along with the RAR window or after the expiry of the RAR window. To avoid delaying OD-SIB1 reception, it is desirable to start the OD-SIB1 reception window immediately after preamble transmission (e.g. the start slot of the RAR window).

Proposal 5:	UE starts the OD-SIB1 reception window after preamble transmission (e.g. the start slot of the RAR window).

In a legacy RA procedure initiated for SI request, the UE can be configured with ra-AssociationPeriodIndex and si-RequestPeriod. si-RequestPeriod specifies the periodicity of the SI-Request configuration in number of association periods, i.e. the periodicity at which a UE can request SI. RAN2 can thus discuss whether such parameters need to be provided in the UL WUS configuration for OD-SIB1 request, when multiple SSBs are present.

Proposal 6:	RAN2 to discuss whether to configure si-RequestPeriod in the UL WUS configuration for OD-SIB1.

# SIB1 Validity

One signaling aspect for UL WUS transmission is related to the target cell recipient of the UL WUS signal. If the UL WUS configuration is available, the NES cell should be the target receiver for the UL WUS signal. If the UL WUS is transmitted directly to the NES cell, the NES cell may transmit SIB1 after detecting the PRACH signal. For the situations when the anchor cell is not good enough for the UEs, and these UEs are located much closer to the NES cell than an anchor cell, having the possibility of transmitting the UL WUS to the NES cell can be more beneficial.

In RAN2 #117, it was discussed to have an OD-SIB1 configuration validity criterion, such that if a UE has a valid version of SIB1 stored, UE does not initiate the on-demand SIB1 request. When the UE camps on a NES cell, the network may not know when the UE acquired SIB1. For the cells not transmitting SIB1, it may be important to maintain a valid version of SIB1 at the UEs. Following the validity functionality for other SIBs, time-based validity for SIB1 can be introduced.

Proposal 7:	Time based validity can be introduced for OD-SIB1 maintenance.

In a scenario where the UE has a valid UL WUS configuration and is camped on the NES cell, if the UE needs to acquire SIB1 from the NES cell, e.g., after a certain validity period, the UE may transmit UL WUS to the NES cell.

Proposal 8:	Upon expiry of a validity time associated with OD-SIB1, if the UE is camped on the NES cell and a valid UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

Handling of RRC connected UEs

In RAN2 #128, it was discussed whether RRC connected UEs can request OD-SIB1, if SIB1 expires. Some companies pointed out that handling of RRC connected UEs is not under the WID scope. This issue was discussed in RP #106 and according to the way forward on NES, RP-243297, the WID scope stays as it is but RAN2 will address the case for RRC connected UEs. As per the indication in WID, the RRC connected UEs can use the same procedure to request on-demand SIB1 transmission. Thus, RRC connected UEs can transmit UL towards the NES cell once their SIB1 expires.

Proposal 9:	Upon expiry of SIB1 for an OD-SIB1 NES cell, an RRC connected UE can transmit UL WUS towards the NES cell, if it has a valid UL WUS configuration for the NES cell.

# UL-WUS configuration validity

One design aspect with OD-SIB1 through the transmission of UL-WUS is to maintain the validity of the UL-WUS configurations. As UL-WUS configuration will be provided to the UE through SIB (e.g., new or existing SIBs), it would make sense to reuse the legacy validity behavior. Following the validity functionality for other SIBs, time-based validity for UL-WUS can be introduced.

Proposal 10:	Time based validity is introduced for UL WUS configuration maintenance.

When a UE performs cell (re-)selection to a NES cell, it may transmit UL-WUS based only upon a valid UL-WUS configuration.

In one scenario, UL WUS configuration may expire with time. In another scenario, the network may change/update the UL WUS configuration for the NES cell that the UE is camped on. To acquire the NES cell configuration from a different anchor cell may be challenging and even not feasible in some situations and deployments. RAN2 #127Bis [3] agreed that the new SIB includes the cell’s own WUS configuration. Thus, the UE should be able to acquire the UL-WUS configuration from the NES cell directly when it is already camped on the NES cell. This can be achieved by the legacy SI modification procedure. If the network modifies the UL-WUS configuration for the NES cell, the NES cell can indicate this modification through system information update message.

Proposal 11:	The network can indicate the modification of UL-WUS configuration for a NES cell through the NES cell itself by SI modification indication.

In a scenario where a UE does not have a valid UL-WUS configuration and is camped on the NES cell, the UE may attempt to re-acquire a valid UL-WUS configuration for the NES cell. If the UE has neither valid SIB1 nor valid UL-WUS configuration for the NES cell, the UE will perform cell reselection and considers the NES cell as barred.

Proposal 12:	If a UE cannot acquire a valid UL-WUS configuration for its camped NES cell and the UE doesn’t have/cannot acquire a valid SIB1, the UE considers the NES cell as barred and performs cell reselection.

# Conclusions

The following proposals are made in this document for on demand SIB1 acquisition in IDLE/Inactive states:

Proposal 1: 	RAN2 to discuss whether the UE acquires fully or partially CORESET 0 configuration (i) through SSB of the NES cell, or (ii) as part of UL-WUS configuration for the NES cell.

Proposal 2:	UL-WUS configuration provides a dedicated RACH configuration for requesting OD-SIB1.

Proposal 3:	Confirm the working assumption that “UL-WUS configuration in RRC release is not supported”.

Proposal 4:	Similar to legacy RA procedure initiated for SI request, an RA initiated for OD-SIB1 request is initiated with 4 step RA and is considered successful upon reception of RAR.

Proposal 5:	UE starts the OD-SIB1 reception window after preamble transmission (e.g. the start slot of the RAR window).

Proposal 6:	RAN2 to discuss whether to configure si-RequestPeriod in the UL WUS configuration for OD-SIB1.

Proposal 7:	Time based validity can be introduced for OD-SIB1 maintenance.

Proposal 8:	Upon expiry of a validity time associated with OD-SIB1, if the UE is camped on the NES cell and a valid UL WUS configuration is available, the UE transmits UL WUS towards the NES cell once triggered.

Proposal 9:	Upon expiry of SIB1 for an OD-SIB1 NES cell, an RRC connected UE can transmit UL WUS towards the NES cell, if it has a valid UL WUS configuration for the NES cell.

Proposal 10:	Time based validity is introduced for UL WUS configuration maintenance.

Proposal 11:	The network can indicate the modification of UL-WUS configuration for a NES cell through the NES cell itself by SI modification indication.

Proposal 12:	If a UE cannot acquire a valid UL-WUS configuration for its camped NES cell and the UE doesn’t have/cannot acquire a valid SIB1, the UE considers the NES cell as barred and performs cell reselection.

# References

RP-240170, Revised WID: Enhancements of network energy savings for NR, Mar 2024

RP-242353, Revised WID: Enhancements of network energy savings for NR, September 2024

RAN2 Chairman’s note, RAN2 #128, November 2024.

RAN2 Chairman’s note, RAN2 #127Bis, October 2024.

RAN2 Chairman’s note, RAN2 #127, August 2024.

RAN2 Chairman’s note, RAN2 #126, May 2024.

RAN2 Chairman’s note, RAN2 #125Bis, April 2024.

3GPP TR 38.864, Study on network energy savings for NR (Release 18), V18.1.0, Mar 2023